NOTE:

- The above rates are governed by the definition of carriage given under the clause 34 of PS to A.P.S.S.

- The above rates are inclusive of Loading, Unloading and Stacking Charges (single operation). As the rates for the various items under materials include stacking charges also, the same should be deducted from the conveyance rates obtained from the table where not applicable.

- Where sandy track and cart track are met with, the equivalent length of these tracks may be arrived by multiplying with factors 1.40 and 1.10 for the length of sandy or cart track respectively and lead for the equivalent length so arrived at may be worked out for the cost of conveyance.

- When the material like Surkhi, Flyash and Bitumen are conveyed on emergency basis to avoid Wharfage, Demurrage from Railway station/Factories to Departmental Godown which are available within a lead of  8 km., 40% extra rate shall be allowed over above rates (Subject to the condition that the Departmental Godown is not available within 8 Kms).

5. The conveyance rates of Surkhi and Flyash are allowed for conveyance of food grains under food for work program.

- Lead with fraction should not be rounded off twice.  Illustratively, if actual lead on sandy track is 10.20 Km. say, then the lead for purpose of data should be only 10.20 x 1.40 = 14.280 or 15 km and not 10.20 rounded to 11.00 multiplied by 1.40 and again rounded to 16 Kms.