GOVERNMENT OF ANDHRA PRADESH

BUILDING – SSR: 2008-2009

COMMITTEE OF CHIEF ENGINEERS

STANDARD SCHEDULE OF RATES

FOR

BUILDING WORKS

FOR THE YEAR 2008-2009                                                                                                          (Effective from 1st June, 2008)

ALL ENGINEERING DEPARTMENTS

Office of the Chief Engineer (R&B)

Buildings, R&B Department,

Errummanzil, Hyderabad.

Procs.No. CEB/TA8/AEE3/SSR/2008-2009/		 Dt:18-06-2008.

Sub:-	Printing of Building-SSRs for all Engineering Departments specific to the functioning of Building works – Building-SSR:2008-09 – Communication of Building-SSRs – Reg.,

Ref:- 	1. G.O.Rt.No.471, T,R&B (R.1) Dept., dt:20-04-2007.

2. Minutes of Committee of Chief Engineers meeting for Building-SSRs held on Dt: 12-6-2008.

*  *  *  *  *

The Government of Andhra Pradesh have constituted a Committee of Chief Engineers of Engineering departments dealing with Building works for finalizing and communicating Building-SSR (Civil & Electrical) vide G.O. 1st cited. In accordance therewith the Committee of Chief Engineers has finalized and approved the Building-SSR: 2007-08 in the Minutes of Meeting held on 12-6-2008.

The rates for the Building items (Civil and Water Supply & Sanitary & Electrical items) are to be followed henceforth by all the concerned departments for building works. The rates of Labour Rates, Common Material & Work items, Public Health items, Conveyance Rates of Materials & Hire charges of Common SSR-2008-09 remain unchanged. No other items other than the items communicated in this Building-SSR shall be introduced.

The Heads of Departments mentioned below are authorized to print the Building-SSR: 2008-09. The unauthorized printing & selling of the Building-SSR is strictly prohibited.

The revised rates will come into force with immediate effect.

The receipt of the proceedings may please be acknowledged.

Sd/-T.Baburaj (18.6.2008)

Chief Engineer (R&B) Buildings

Convener-Member,

Committee of Chief Engineers- Building-SSR

To

- Engineer-in-Chief (Admn.)., Roads & Buildings Department.

- Engineer-in-Chief (I&CAD)., Irrigation Department.

- Engineer-in-Chief (P.H.), Public Health Department.

- Engineer-in-Chief (P.R.), Panchayat Raj Department.

- Chief Engineer (Roads), R&B Department.

- Chief Engineer (NH), R&B Department.

- Chief Engineer(Panchayat Raj)

- Chief Engineer(AP Housing Board)

- Chief Engineer, Greater Hyderabad (MCH)

- Chief Engineer(AP Housing Corporation)

- Chief Engineer(APSRTC)

- Chief Engineer(APHMHIDC)

- Superintending Engineer (R&B), Head Quarters Circle, Hyderabad

- Superintending Engineer (R&B), Electrical Circle, Hyderabad

INDEX

| Sl. No: | DESCRIPTION | Page No: |  |
| --- | --- | --- | --- |
|  |  | From | To |
| BUILDING – SSR:2008-09 |  |  |  |
| 1 | Preamble of Building-SSR-2008-09 | 5 | 7 |
| 2 | Building Items - Building-SSR-2008-09 | 8 | 66 |
| 3 | Water Supply & Sanitary Items - Building-SSR-2008-09 | 67 | 99 |
| 4 | Electrical Items – Building-SSR-2008-09 | 100 | 139 |
| 5 | Conveyance Rates of Building Materials –Building – SSR -2008-09 | 140 | 142 |
| 6 | Extract of G.O. Ms. No: 471 T,R&B (R1) Dept. dt. 24.04.2007 on Constitution of Committee of Chief Engineers for Building:SSR | 143 | 143 |

PREAMBLE

- INTRODUCTION

The Government have issued orders vide G.O. Rt. No: 471 TR&B (R-1) Dept., Dt. 20.4.2007 constituting a Committee comprising of Chief Engineers of departments in-charge of execution of building works for finalization of SSR for both Electrical Civil items specific to the functioning of building works. The 1st - Meeting of the above “SSR Committee for Buildings (Civil & Electrical)” was convened on 7.7.2007 with an objective to finalize and release of Building-SSR: 2008-09.

- APPLICABILITY OF COMMON SSR

The rates of Labour Rates, Common Material & Work items, Public Health items, Conveyance Rates of Materials & Hire charges of Common SSR-2008-09 remain applicable. The specifications, stipulations & all other conditions of Common-SSR other than specified in this context remain applicable.

- ISSUES

It is observed during the recent period that the building tenders are not receiving any response due to various reasons. Steep hike in the basic material costs such as steel & cement being one of the primary reasons, but the apathy continued although the Government issued orders facilitating price adjustments in the contracts for steel & cement items on par with the market rates vide G.O. Rt. No: 94 T, R&B (R.1) Dept. The variation in the prices of POL and other materials were continued to be adjusted in the same manner as was practiced before the issue of the Government orders. Contractors are showing indifference to the building works throughout the state.

The building construction has become very complicated and weird due to the advent of new varieties of materials every day. The regular building items are becoming obsolete every now and then. There is a substantial increase in the market prices. It is also observed from the Whole Sale Price Indices that the variation in the building commodities is very much significant. More so, the contractors are finding the infrastructure development works taken up in other sectors more attractive and lucrative. In this scenario, finalization of Building SSR is more complicated and demanding. The SSR now finalized may not remain perpetual till another year.

- BUILDING ITEMS

There is a substantial increase in the prices of the clay ordinary kiln burnt clay Bricks, also there is an increase in the Fly-ash cement lime solid blocks. Aerocon thermal building blocks and panels also have increased when compared to previous year.

It is proposed to finalize the ex-quarry rates for cuddapah, bethamcherla and shahabad natural stones both polished and unpolished for all thicknesses. The rates now proposed are ex-quarry prices and suitable lead/ conveyances have to be added in the respective estimates by the estimate preparing authorities. However, prices of the polished granite, marble stones, ceramic tiles, vitrified tiles and such other flooring materials have been finalized as before and shall continue to be the landing prices.

The prices for Galvanized iron sheets, nails, wire meshes and all other related materials have been proposed based on the current market prices where the increase is sizeable. It is found that the market prices for Aluminium door fixtures and extrusions, sections pipes have increased considerably, so also the Brass items.

As regards the Ready Mix Concrete, it is observed that there is confusion with regard to the minimum cement contents mentioned alongside the grades of concrete. It is also observed that in some cases, the estimates are being prepared and sanctioned with the minimum cement contents mentioned therewith without actually designing the mixes. In such cases the cement contents may vary from the actual design values. There seems to be problems despite the clause specifying the rate adjustments with regard to the variation of cement contents owing to design results. Further it is observed that the Ready Mix Concrete suppliers are not adhering to the specifications of aggregates, cement, admixtures etc while supplying the Ready Mix Concrete resulting in cracks and making the buyers liable for the failures.

It is felt by this Committee of Chief Engineers, that it is more sensible to advise the fraternity to use the Ready Mix Concrete supplied by the suppliers only for the structures where Ordinary Concretes as per IS: 456-2000 are desired and for the structures where the Standard and High Strength Concretes vide IS: 456-2000 are required mix design may be carried out and rates may be arrived as per the design mixes. In view of the above it is decided to delete the rates for the following Grades of Ready Mix Concrete – M25, M30, M35 and M40.

- WATER SUPLY & SANITARY ITEMS

The increase in the supply of SWG Pipes, bends, fittings and other related items as proposed by the Engineer-in-Chief (PH), has been finalized in the Building SSR-2008-09. The rates for the Manholes and inspection chambers are finalized based on the detailed rate analysis.  The rates of CI Pipes and fittings have been fixed based on the price lists obtained from the dealers & manufacturers. Similarly the Vitreous china ware, porcelain, taps & faucets, G.I Pipes, valves & fittings and PVC Pipes & fittings have been finalized.

There is no increase in the prices of the Jaquar fittings as per the recommendations of the dealers. CPVC Pipes and fittings have been introduced in the draft Building SSR 2008-09 as per the demand and need to use green building products in the construction sectors.

BUILDING ITEMS

| BUILDING ITEMS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No | Details |  |  |  |  |  | Unit | SS RATE FOR 2008-09 |
| 1 | 2 |  |  |  |  |  | 3 | 4 |
|  | IV . RATES OF MATERIALS |  |  |  |  |  |  |  |
|  | (A) BRICKS & TILE PRODUCTS |  |  |  |  |  |  |  |
| 1 | Bricks second class or ground moulded (Non-Modular or traditional size) 23 x 11 x 7 cm (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 1000 Nos | 3100.00 |
| 2 | - do- modular size 19 x 9 x 9 cm (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 1000 Nos | 4185.00 |
|  | Flyash cement/lime solid blocks with compressive strength of 75 kg/sq.cm - IS:12894-1990 - CLASS - 7.5 (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  |  |  |
| 3 | 290 x 225 x 140 mm |  |  |  |  |  | Each | 15.00 |
| 4 | 290 x 200 x 140 mm |  |  |  |  |  | Each | 13.00 |
| 5 | 290 x 150 x 140 mm |  |  |  |  |  | Each | 9.00 |
| 6 | 290 x 112/100 x 140 mm |  |  |  |  |  | Each | 8.00 |
| 7 | 225 x 100 x 60 mm |  |  |  |  |  | Each | 4.00 |
| 8 | 145 x 225 x 140 mm |  |  |  |  |  | Each | 9.00 |
| 9 | 145 x 200 x 140 mm |  |  |  |  |  | Each | 7.00 |
|  | Flyash cement/ lime solid blocks with compressive strength of 50 kg/sq.cm - IS:12894-1990 - CLASS - 5 (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  |  |  |
| 10 | 290 x 225 x 140 mm |  |  |  |  |  | Each | 13.00 |
| 11 | 290 x 200 x 140 mm |  |  |  |  |  | Each | 10.00 |
| 12 | 290 x 150 x 140 mm |  |  |  |  |  | Each | 7.00 |
| 13 | 290 x 112/100 x 140 mm |  |  |  |  |  | Each | 6.00 |
| 14 | 225 x 100 x 60 mm |  |  |  |  |  | Each | 3.00 |
| 15 | 145 x 225 x 140 mm |  |  |  |  |  | Each | 7.00 |
| 16 | 145 x 200 x 140 mm |  |  |  |  |  | Each | 6.00 |
| 17 | Cinder |  |  |  |  |  | 1 Cum | 495.00 |
|  | Supply of ‘AEROCON’ HQ building Blocks & ‘COMFORTILE-Thermal Roof Tile’ excluding transportation charges |  |  |  |  |  |  |  |
|  | A. AEROCON – HQ BUILDING BLOCKS |  |  |  |  |  |  |  |
| 18 | 600 x 200 x 75 mm |  |  |  |  |  | Each | 49.00 |
| 19 | 600 x 200 x 100 mm |  |  |  |  |  | Each | 65.00 |
| 20 | 600 x 200 x 125 mm |  |  |  |  |  | Each | 76.00 |
| 21 | 600 x 200 x 150 mm |  |  |  |  |  | Each | 91.00 |
| 22 | 600 x 200 x 200 mm |  |  |  |  |  | Each | 122.00 |
| 23 | 600 x 200 x 230 mm |  |  |  |  |  | Each | 140.00 |
|  | B. AEROCOOL THERMAL BLOCKS excluding transportation charges |  |  |  |  |  |  |  |
| 24 | 300 x 200 x 50 mm |  |  |  |  |  | Each | 19.00 |
|  | Supply & fixing of AEROCON PANELS excluding transportation charges |  |  |  |  |  |  |  |
| 25 | Supply & fixing of 50mm thick Aerocon sandwich Panels having Tongue and Groove joint for partitions and walls using G.I. Flooring & Ceiling channels, hardware, Labour, transportation, wastages, etc., as a complete item of work excluding finishing, fittings & fixtures. |  |  |  |  |  | Sqm. | 1224.00 |
| 26 | Supply & fixing of 75mm thick Aerocon sandwich Panels having Tongue & groove joint for partitions And walls using G.I. Flooring & Ceiling channels, hardware, Labour, transportation, wastages, etc., As a complete item of work Excluding finishing, fittings & fixtures |  |  |  |  |  | Sqm. | 1523.00 |
|  | AEROCON PANELS |  |  |  |  |  |  |  |
| 27 | 50mm thick x 600mm standard width x 2400mm, 2700mm & 3000mm lengths |  |  |  |  |  | 1 Sqm. | 804.00 |
| 28 | 75mm thick x 600mm standard width x 2400mm, 2700mm & 3000mm lengths |  |  |  |  |  | 1 Sqm. | 1087.00 |
|  | NATURAL FLOORING STONES - CUDDAPAH/ SHAHABAD/ TANDUR STONES (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  |  |  |
| 29 | Kadapa slab 40 mm thick and size not less than 0.457 M x 0.457 M (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 1 Sqm | 75.00 |
| 30 | Kadapa slab 50 mm thick and size not less than 0.457 M x 0.457 M (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 1 Sqm | 86.00 |
| 31 | Shahabad /Tandur Rough Stone slabs of minimum of 25 mm thick (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 750.00 |
| 32 | Shahabad / Tandur Rough Stone slabs of minimum 40 mm thick (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 968.00 |
| 33 | Polished Shahabad/ Tandur stone slabs of 15 to 18 mm thick (0.457x0.304) or any other size as specified (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 1291.00 |
| 34 | Polished black Kadapa slabs minimum of 15 mm thick (0.457x0.457M) (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 1076.00 |
| 35 | Polished Bethamcherla pure (marble type) white stone minimum 25 mm thick (0.254Mx0.254M) (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 3012.80 |
| 36 | Polished Bethamcherla coloured stone minimum 25 mm thick (0.254M x0.254M) (ex-quarry price - excluding cost of conveyance, loading & unloading) |  |  |  |  |  | 10 Sqm | 2152.00 |
| 37 | High Polished Granite 16 to 18 mm thick up to 8'-00 (2.43 M) other than black/ premium colours |  |  |  |  |  | 1Sqm | 2209.00 |
| 38 | High Polished Granite 16 to 18 mm thick up to 8'-00 (2.43 M) other than black and regular colours |  |  |  |  |  | 1Sqm | 1940.00 |
| 39 | High Polished Granite 16 to 18 mm thick up to 8'-00 (2.43 M) black |  |  |  |  |  | 1Sqm | 1639.00 |
| 40 | Rounding the edges of Kadapa / Shahabad stone slabs of any thickness including polishing the same |  |  |  |  |  | 1 RM | 38.00 |
| 41 | Half Rounding the edges of Marble/ Granite stone slabs of all thicknesses including polishing the same. |  |  |  |  |  | 1 RM | 115.00 |
| 42 | Full Rounding the edges of Marble/ Granite stone slabs of all thicknesses including polishing the same. |  |  |  |  |  | 1 RM | 150.00 |
| 43 | Cutting Marble/ Granite slabs up to 50 mm thick by mechanical device. |  |  |  |  |  | 1 RM | 7.00 |
| 44 | Flat nosing to Kadapa/ Shahabad stone slabs of any thick |  |  |  |  |  | 1 RM | 7.00 |
|  | FLOORING SLABS/ TILES |  |  |  |  |  |  |  |
| 45 | Polished Marble slabs of any variety (Such as Dongari/ Adanga/ Marodh/ Rajganpur etc) 16 mm to 20 mm (average) thick (size 0.457 M x 0.457 M) / (0.6M x 0.6M) |  |  |  |  |  | 1 Sqm | 523.00 |
| 46 | Polished Marble slabs of any variety (Such as Dongari/ Adanga/ Marodh/ Rajganpur etc) polished 16 mm to 20 mm (average) thick 0.305 M x 0.305 M |  |  |  |  |  | 1 Sqm | 425.00 |
| 47 | Polished Marble slabs of any variety and gangsaw cut (Such as Dongari/ Adanga/ Marodh/ Rajganpur etc) 16 mm to 20 mm (average) thick ( 0.610 M width of any length) |  |  |  |  |  | 1 Sqm | 718.00 |
| 48 | Polished Marble Tiles 8 mm thick of all sizes and varieties |  |  |  |  |  | 1 Sqm | 371.00 |
| 49 | Granite stone tiles 8 mm thick (mirror polished of all shades) |  |  |  |  |  | 1 Sqm | 732.00 |
| 50 | Ceramic tiles 7.3mm thick 1st quality of all shades (as per manufacture specification) |  |  |  |  |  | 1 Sqm | 374.00 |
| 51 | Edge Cut - Rectified Ceramic tiles 8 mm thick 1st quality of all shades |  |  |  |  |  | 1 Sqm | 450.00 |
| 52 | Chequered Terrazzo tiles of 22 mm thick (Medium shade) 0.254 x 0.406 M |  |  |  |  |  | 1 Sqm | 229.00 |
| 53 | Chequered Terrazzo tiles of 22 mm thick (Dark shade) 0.254 M x 0.406 M |  |  |  |  |  | 1 Sqm | 209.00 |
| 54 | Chequered Terrazzo tiles of 30 mm thick ( Light shade) 0.305 M x 0.305 M |  |  |  |  |  | 1 Sqm | 209.00 |
| 55 | Chequered Terrazzo tiles of 30 mm thick (Dark shade) 0.305 M x 0.305 M |  |  |  |  |  | 1 Sqm | 222.00 |
| 56 | Chequered Terrazzo tiles of 30 mm thick ( medium shade) 0.305 x 0.305 M |  |  |  |  |  | 1 Sqm | 196.00 |
| 57 | Decorated white background glazed tiles 200 mm x 152 mm |  |  |  |  |  | 1 Sqm | 459.00 |
| 58 | Decorated coloured background glazed tiles 200mm x 300 mm |  |  |  |  |  | 1 Sqm | 512.00 |
| 59 | Fantasy Glazed Tiles size 200mm x152 mm |  |  |  |  |  | 1 Sqm | 485.00 |
| 60 | White glazed Tiles of any size Ist quality |  |  |  |  |  | 1 Sqm | 280.00 |
| 61 | Glazed coloured Tiles of any size Ist Quality |  |  |  |  |  | 1 Sqm | 332.00 |
| 62 | Precast Terrazzo Tiles 20 mm thick ( Medium shade) |  |  |  |  |  | 1 Sqm | 238.00 |
| 63 | Precast Terrazzo Tiles 20 mm thick (dark shade) |  |  |  |  |  | 1 Sqm | 231.00 |
| 64 | Supply & Fixing precast Terrazzo Tiles Marbleite in white & colour vains 20 mm thick |  |  |  |  |  | 1 Sqm | 250.00 |
| 65 | a) Fixing charges only |  |  |  |  |  | 1 Sqm | 24.00 |
| 66 | b) Polishing charges only |  |  |  |  |  | 1 Sqm | 31.00 |
| 67 | c) Wax polishing charges only. |  |  |  |  |  | 1 Sqm | 19.00 |
| 68 | Supply & Fixing precast Terrazzo Tiles Marbleite in white tiles 20 mm thick |  |  |  |  |  | 1 Sqm | 194.00 |
| 69 | a) Fixing charges only |  |  |  |  |  | 1 Sqm | 24.00 |
| 70 | b) Polishing charges only |  |  |  |  |  | 1 Sqm | 31.00 |
| 71 | c) Wax polishing charges only. |  |  |  |  |  | 1 Sqm | 19.00 |
| 72 | Supply & fixing Terrazzo tiles 20 mm thick grey tiles |  |  |  |  |  | 1 Sqm | 112.00 |
| 73 | a) Fixing charges only |  |  |  |  |  | 1 Sqm | 24.00 |
| 74 | b) Polishing charges only |  |  |  |  |  | 1 Sqm | 31.00 |
| 75 | c) Wax polishing charges only. |  |  |  |  |  | 1 Sqm | 19.00 |
|  | Supply & fixing of Crazy Marble flooring |  |  |  |  |  |  |  |
| 76 | Marble powder |  |  |  |  |  | 1 Kg | 2.00 |
| 77 | Marble chips |  |  |  |  |  | 1 Sqm | 446.00 |
| 78 | Mosaic chips |  |  |  |  |  | 1 Sqm | 2521.00 |
| 79 | Grinding, Polishing, electricity charges |  |  |  |  |  | 1 Sqm | 100.00 |
| 80 | Vitrified polished floor tiles 900x900 of 10 to 12 mm thickness premium type 1st quality |  |  |  |  |  | 1 Sqm | 1290.00 |
| 81 | Vitrified polished floor tiles 800x800 of 8 to10 mm thickness premium type 1st quality |  |  |  |  |  | 1 Sqm | 953.00 |
| 82 | Vitrified polished floor tiles 600x600 of 8 to 10 mm thickness premium colour 1st quality. |  |  |  |  |  | 1 Sqm | 826.00 |
| 83 | Vitrified polished floor tiles 600x600 of 8 to 10 mm thickness normal colour 1st quality. |  |  |  |  |  | 1 Sqm | 763.00 |
| 84 | Vitrified polished floor tiles 400x400x7mm thickness normal colour 1st quality. |  |  |  |  |  | 1 Sqm | 572.00 |
| 85 | Glass Mosaic Tiles Group 1 series or equivalent quality 20x20x4 mm thick |  |  |  |  |  | 1 Sqm | 1525.00 |
|  | Providing of Vitrified/ Ceramic Tile flooring for required areas using different sizes, approved colour and make fixing with LATICRETE ADHESIVE as Specified thickness without joint including all taxes excluding cost of tiles |  |  |  |  |  |  |  |
| 86 | 600 x 600 mm tiles. |  |  |  |  |  | 1 Sqm | 113.00 |
| 87 | 400 x 400 mm tiles or 300 x 300 mm tiles or 300 x 300 mm tiles or 300 x 200 mm tiles |  |  |  |  |  | 1 Sqm | 90.00 |
|  | Providing of vitrified /Ceramic Tile flooring for required areas using different sizes, approved colour and make fixing with LATICRETE ADHESIVE as Specified thickness with spacers including joints shall be pointed with specified colour cement based polymer grout of LATICRETE mixed with Laticrete 1776 grout admix plus or EQUIVELANT including all taxes excluding cost of tiles |  |  |  |  |  |  |  |
| 88 | 600 x 600 x 8mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 128.00 |
| 89 | 600 x 600 x 8mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 134.00 |
| 90 | 400 x 400 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 129.00 |
| 91 | 400 x 400 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 137.00 |
| 92 | 300 x 300 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 134.00 |
| 93 | 300 x 300 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 146.00 |
| 94 | 300 x 200 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 141.00 |
| 95 | 300 x 200 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 154.00 |
|  | Providing vitrified / Ceramic Tile flooring for required areas using different sizes, approved colour and make fixing with LATICRETE ADHESIVE as Specified thickness with spacers including joints shall be pointed with specific colour Latapoxy grout of LATICRETE SP-100 or EQUIVELANT excluding cost of tiles |  |  |  |  |  |  |  |
| 96 | 600 x 600 x 8mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 223.00 |
| 97 | 600 x 600 x 8mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 260.00 |
| 98 | 400 x 400 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 214.00 |
| 99 | 400 x 400 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 255.00 |
| 100 | 300 x 200 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 255.00 |
| 101 | 300 x 200 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 310.00 |
| 102 | 300 x 200 x 6mm tiles with 3 mm joints width |  |  |  |  |  | 1 Sqm | 262.00 |
| 103 | 300 x 200 x 6mm tiles with 4 mm joints width |  |  |  |  |  | 1 Sqm | 319.00 |
| 104 | Providing Glass Mosaic tiles using specified pattern fixing with LATICRETE 252 Ag Silver white of Equivalent and Approved colour of joint filling with LATICRETE Cement grouting with admix of LATICRETE 1776. Including all taxes excluding cost of tiles |  |  |  |  |  | 1 Sqm | 215.00 |
| 105 | Providing Glass Mosaic tiles using specified pattern fixing with LATICRETE 252 Ag Silver white of Equivalent and Approved colour of joint filling with LATICRETE LATAPOXY SP-100 or EQUIVALENT including all taxes excluding cost of tiles |  |  |  |  |  | 1 Sqm | 958.00 |
|  | ULTRA DESIGNER TILES |  |  |  |  |  |  |  |
| 106 | Supply of prepolished ready to lay floor tiles of ULTRA or equivalent brand of any colour, design and any size with 22 mm thickness including all taxes with compressive strength not less than 450 Kg/cm² conforming to IS 516 |  |  |  |  |  | 1 Sqm | 485.00 |
| 107 | Supply of wall tiles of Category - I of ULTRA or equivalent brand of any colour, design like Camroon, Sphinx, Opus with 10 mm thickness including all taxes |  |  |  |  |  | 1 Sqm | 495.00 |
| 108 | Supply of wall tiles of Category - II of ULTRA or equivalent brand of any colour, design like Arrow, Bedrock, Eden, Gladstone, Mastro, Monarch, Plato, Santo, Ultra Brique, Rhine Stone with 10 mm thickness including all taxes |  |  |  |  |  | 1 Sqm | 565.00 |
| 109 | Supply of wall tiles of Category - III of ULTRA or equivalent brand of any colour, design like Cleopatra, Simbel, Sixten, Spectra, Elmo, Elit with 10 mm thickness including all taxes |  |  |  |  |  | 1 Sqm | 630.00 |
| 110 | Supply of Step tiles of Category - I of ULTRA or equivalent brand of any colour, design like emperor, Dynasty with 25 mm thickness including all taxes |  |  |  |  |  | 1 Sqm | 555.00 |
| 111 | Supply of Step tiles of Category - II of ULTRA or equivalent brand of any colour, design like Stefano, Brigadia, Ventura with 25 mm thickness including all taxes |  |  |  |  |  | 1 Sqm | 619.00 |
|  | SUPPLY OF SPHINX OR EQUIVALENT TILES FOR FLOOR, WALL / EXTERIOR : |  |  |  |  |  |  |  |
|  | Regular Series: Floor |  |  |  |  |  |  |  |
| 112 | VEECUMCY EMERALD, RAINBOW, PLAIN TILE Thickness: 20 mm (Various sizes) |  |  |  |  |  | 1 Sqm | 360.00 |
| 113 | Pavers: KING’S PATH, ZIG ZAG, DUMB BELLS, I-BLOCK - Thickness: 60 mm (Various sizes) |  |  |  |  |  | 1 Sqm | 400.00 |
| 114 | Pavers: KING’S PATH, ZIG ZAG, DUMB BELLS, I-BLOCK - Thickness: 80 mm (Various sizes) |  |  |  |  |  | 1 Sqm | 440.00 |
| 115 | Pavers: KING’S PATH, ZIG ZAG, DUMB BELLS, I-BLOCK - Thickness: 100 mm (Various sizes) |  |  |  |  |  | 1 Sqm | 480.00 |
| 116 | Special Economy Series: Floor & Wall |  |  |  |  |  | 1 Sqm | 460.00 |
|  | Floor / Thickness:15mm / (Various sizes): VEECUMCY EMERALD, RAINBOW, PLAIN TILE, ONE QUARTER |  |  |  |  |  |  |  |
|  | Wall / Thickness: 9mm-15mm / (Various sizes): PANTHER, CASTLE BRICK |  |  |  |  |  |  |  |
|  | Elegance Series: Floor & Wall |  |  |  |  |  |  |  |
| 117 | Floor / Thickness:15mm / (Various sizes): VEECUMCY EMERALD, PEBBLES, FANTASY, STUD MAZE, SAUSAGE, ONE QUARTER, KNG’S PATH, RAINBOW, WAVE |  |  |  |  |  | 1 Sqm | 517.00 |
|  | Wall / Thickness: 9mm-15mm / (Various sizes):WINDSOR CASTLE, PANTHER, WAVE, PEBBLES, BANIAN CREASE, FANTASY, FAUNA FLAUNT, FOCUS, STUD MAZE, SAUSAGE, ONE QUARTER, KING’S PATH, CASTLE BRICK |  |  |  |  |  |  |  |
| 118 | Floor / Thickness:15mm / (Various sizes): TEXTURED, FAUNA FLAUNT, ROCK N ROLL, ETHNIC, TWISTER, FOCUS |  |  |  |  |  | 1 Sqm | 555.00 |
|  | Wall / Thickness: 9mm-15mm / (Various sizes):WAFER , ROYAL EMPEROR, TEXTURED, ROCK N ROLL, ETHNIC, TWISTER |  |  |  |  |  |  |  |
|  | Premium Series: |  |  |  |  |  |  |  |
| 119 | Floor / Thickness:15mm / (Various sizes): VEECUMCY EMERALD, PEBBLES, FANTASY, STUD MAZE, SAUSAGE |  |  |  |  |  | 1 Sqm | 747.00 |
|  | Wall / Thickness: 9mm-15mm / (Various sizes):WINDSOR CASTLE, PEBBLES, BANIAN CREASE, FANTASY, FAUNA FLAUNT, FOCUS, STUD MAZE, SAUSAGE, ONE QUARTER, KING’S PATH, PANTHER, WAVE, CASTLE BRICK |  |  |  |  |  |  |  |
| 120 | Floor / Thickness:15mm / (Various sizes): TEXTURED, ETHNIC, ROCK N ROLL |  |  |  |  |  | 1 Sqm | 777.00 |
|  | Wall / Thickness: 9mm-15mm / (Various sizes): TEXTURED, ROCK N ROLL, ETHNIC, WAFER, TWISTER, ROYAL EMPEROR |  |  |  |  |  |  |  |
|  | Vice Roy Series: |  |  |  |  |  |  |  |
| 121 | Floor / Thickness:15mm / (Various sizes): VEECUMCY EMERALD |  |  |  |  |  | 1 Sqm | 975.00 |
| 122 | Floor / Thickness:15mm / (Various sizes): TEXTURED, ROCK N ROLL |  |  |  |  |  | 1 Sqm | 999.00 |
|  | Wall / Thickness: 9mm-15mm / (Various sizes): TWISTER, ROYAL EMPEROR, TEXTURED, ROCK N ROLL |  |  |  |  |  |  |  |
|  | (C ) TIMBER AND ROOFING MATERIALS |  |  |  |  |  |  |  |
|  | TEAK WOOD |  |  |  |  |  |  |  |
|  | Note: Before proposing the Teak Wood Items, the estimating authorities shall obtain prior approval from the competent authority |  |  |  |  |  |  |  |
| 123 | Teak wood scantlings below 2 meters First class |  |  |  |  |  | 1 cum | 93096.00 |
| 124 | Teak wood scantlings below 2 meters Second class |  |  |  |  |  | 1 cum | 75194.00 |
| 125 | Teak wood scantlings above 2 meters First class |  |  |  |  |  | 1 cum | 96677.00 |
| 126 | Teak wood scantlings above 2 meters Second class |  |  |  |  |  | 1 cum | 78774.00 |
| 127 | Teak wood planks of all sizes First Class |  |  |  |  |  | 1 cum | 84916.00 |
| 128 | Teak wood Planks of all sizes second class. |  |  |  |  |  | 1 cum | 70018.00 |
| 129 | Burma Teak wood scantlings below 2 meters |  |  |  |  |  | 1 cum | 108274.00 |
| 130 | Burma Teak wood scantlings above 2 meters |  |  |  |  |  | 1 cum | 112239.00 |
| 131 | Burma Teak wood planks 0.15m to 1 m |  |  |  |  |  | 1 cum | 115823.00 |
| 132 | Assam Teak Wood frames |  |  |  |  |  | 1 cum | 28628.00 |
| 133 | Assam Teak Wood Planks |  |  |  |  |  | 1 cum | 34927.00 |
| 134 | Sal Wood Frames |  |  |  |  |  | 1 cum | 33495.00 |
| 135 | Sal Wood Planks |  |  |  |  |  | 1 cum | 38071.00 |
|  | GALVANIZED IRON SHEETS |  |  |  |  |  |  |  |
| 136 | Plain or Corrugated Galvanized iron sheets as per IS 277 (0.1 mm to 0.8 mm thickness) |  |  |  |  |  | 1 kg | 60.00 |
| 137 | Country Nails |  |  |  |  |  | 1 Kg | 55.00 |
| 138 | Wire Nails |  |  |  |  |  | 1 Kg | 55.00 |
| 139 | Mild Steel Tubes & pipes of all diameters as per IS-1161 |  |  |  |  |  | 1 MT | 50000.00 |
|  | Labour charges for Iron Fabrication works |  |  |  |  |  |  |  |
| 140 | a. Labour charges for fabricating steel works like Window Grills, Compound Wall Grills, Iron Doors, Windows including cost of welding rods, power charges, excluding cost of fixing in position |  |  |  |  |  | 1 Kg | 12.00 |
| 141 | b. Labour charges for fixing Iron Doors, Iron Windows and Window Grills in position |  |  |  |  |  | 1 Kg | 3.00 |
| 142 | c. Labour charges for fabricating all heavy steel works like Trusses, Stanchions, Heavy Beams and Girders including cost of welding rods, power charges, etc excluding cost of fixing in position |  |  |  |  |  | 1 Kg | 10.00 |
| 143 | d. Labour charges for fixing heavy iron grills, Trusses, Stanchions, Heavy Beams and Girders erecting in position and fixing by using chain pulley blocks, Derek pole arrangements and cranes etc., complete in position |  |  |  |  |  | 1 Kg | 9.00 |
| 144 | Stainless Steel Tubes, sections, plates, sheets & pipes of all sizes & diameters - 204 Grade |  |  |  |  |  | 1 Kg | 160.00 |
| 145 | Stainless Steel Tubes, sections, plates, sheets & pipes of all sizes & diameters for - 304 Grade |  |  |  |  |  | 1 Kg | 340.00 |
| 146 | Labour charges for Fabrication of stainless steel railing works using stainless steel welding rods including buffing, polishing, lacquer finishing, to present seamless finish |  |  |  |  |  | 1 Kg | 75.00 |
| 147 | labour charges for fabrication of reinforcement including bending placing at site in position including cost of binding wire |  |  |  |  |  | 1 Kg | 9.00 |
| 148 | Lift Charges for Steel reinforcement per each additional floor other than Ground Floor |  |  |  |  |  | 1 Kg | 0.25 |
|  | Supplying Galvanized Steel Barbed wire conforming to IS 278 - 1978 |  |  |  |  |  |  |  |
| 149 | Type A - IOWA Type Designation as per standard |  |  |  |  |  | 1 Kg | 59.00 |
| 150 | Type B - Glidden Type Designation as per standard |  |  |  |  |  | 1 Kg | 59.00 |
| 151 | Supply & fixing of Rolling shutter made of 80 x 1.25 mm machine rolled CRCA laths, interlocked together through their entire length and jointed together at the ends by end-locks, mounted on specially designed pipe shaft of 50mm dia nominal bore MS B class pipe with brackets, plates, guide channels, stoppers, bottom locking plates and arrangements for inside & outside locking with push-pull operations including cost of hood cover and springs complete, painted with one coat of approved steel primer, locks, ball bearings, all accessories etc complete for finished item of work as per special spn: 1108 |  |  |  |  |  | 1 Sqm | 2000.00 |
| 152 | Supplying & fixing collapsible steel shutters with vertical, double channel of 20 x10x2 mm of 100 mm centre ,Bracers with flat iron 40x40x6 mm with 38 mm dia steel pulleys, the top, bottom and side vertical frames of the collapsible gate with 65x65mmx8mm MS Angle and middle guide rail at site height with 65mmx8mm MS flat for the pulleys to guide and fixed with necessary hold fasts, bolts, nuts, rivets, locking arrangements, stoppers, handles, all accessories all fixtures and painted with one coat of approved steel primer etc., complete for finished item of work as per special spn 1105 |  |  |  |  |  | 1 Sqm | 1750.00 |
|  | (D) DOOR FITTINGS |  |  |  |  |  |  |  |
|  | 1) TOWER BOLTS AS PER IS 204 |  |  |  |  |  |  |  |
|  | BRASS (10 mm BOLT dia) |  |  |  |  |  |  |  |
| 153 | 100 mm Long |  |  |  |  |  | Each | 76.00 |
| 154 | 150 mm Long |  |  |  |  |  | Each | 112.00 |
| 155 | 200 mm Long |  |  |  |  |  | Each | 150.00 |
| 156 | 250 mm Long |  |  |  |  |  | Each | 186.00 |
| 157 | 300 mm Long |  |  |  |  |  | Each | 230.00 |
|  | ALUMINIUM (10 mm BOLT dia) |  |  |  |  |  |  |  |
| 158 | 75 mm Long |  |  |  |  |  | Each | 47.00 |
| 159 | 100 mm Long |  |  |  |  |  | Each | 57.00 |
| 160 | 150 mm Long |  |  |  |  |  | Each | 69.00 |
| 161 | 200 mm Long |  |  |  |  |  | Each | 86.00 |
| 162 | 250 mm Long |  |  |  |  |  | Each | 103.00 |
| 163 | 300 mm Long |  |  |  |  |  | Each | 120.00 |
|  | MILDSTEEL (POWDER COATED) (10 mm BOLT dia) |  |  |  |  |  |  |  |
| 164 | 100 mm Long |  |  |  |  |  | Each | 18.00 |
| 165 | 150 mm Long |  |  |  |  |  | Each | 27.00 |
| 166 | 200 mm Long |  |  |  |  |  | Each | 38.00 |
| 167 | 250 mm Long |  |  |  |  |  | Each | 51.00 |
| 168 | 300 mm Long |  |  |  |  |  | Each | 63.00 |
|  | 2) BUTT HINGES AS PER IS : 205 |  |  |  |  |  |  |  |
|  | BRASS |  |  |  |  |  |  |  |
| 169 | 75 mm Long |  |  |  |  |  | Each | 32.00 |
| 170 | 100 mm Long |  |  |  |  |  | Each | 104.00 |
| 171 | 125 mm Long |  |  |  |  |  | Each | 126.00 |
| 172 | 150 mm Long |  |  |  |  |  | Each | 166.00 |
|  | ALUMINIUM |  |  |  |  |  |  |  |
| 173 | 75 mm Long |  |  |  |  |  | Each | 68.00 |
| 174 | 100 mm Long |  |  |  |  |  | Each | 81.00 |
| 175 | 125 mm Long |  |  |  |  |  | Each | 96.00 |
| 176 | 150 mm Long |  |  |  |  |  | Each | 96.00 |
|  | MILD STEEL (POWDER COATED) |  |  |  |  |  |  |  |
| 177 | 75 mm Long |  |  |  |  |  | Each | 8.00 |
| 178 | 100 mm Long |  |  |  |  |  | Each | 16.00 |
| 179 | 125 mm Long |  |  |  |  |  | Each | 23.00 |
| 180 | 150 mm Long |  |  |  |  |  | Each | 31.00 |
|  | 3) DOOR HANDLES AS PER IS:208 |  |  |  |  |  |  |  |
|  | ALUMINIUM |  |  |  |  |  |  |  |
| 181 | 75 mm Long |  |  |  |  |  | Each | 45.00 |
| 182 | 100 mm Long |  |  |  |  |  | Each | 51.00 |
| 183 | 125 mm Long |  |  |  |  |  | Each | 76.00 |
| 184 | 150 mm Long |  |  |  |  |  | Each | 85.00 |
|  | MILD STEEL (POWDER COATED) |  |  |  |  |  |  |  |
| 185 | 125 mm Long |  |  |  |  |  | Each | 23.00 |
| 186 | 150 mm Long |  |  |  |  |  | Each | 34.00 |
|  | ALDROPS AS PER IS:2681 |  |  |  |  |  |  |  |
|  | BRASS |  |  |  |  |  |  |  |
| 187 | 300 mm long |  |  |  |  |  | Each | 675.00 |
| 188 | Aldrop heavy duty 350 mm |  |  |  |  |  | Each | 1299.00 |
| 189 | Aldrop heavy duty 450 mm |  |  |  |  |  | Each | 1850.00 |
|  | ALUMINIUM |  |  |  |  |  |  |  |
| 190 | 200 mm long |  |  |  |  |  | Each | 200.00 |
| 191 | 250 mm long |  |  |  |  |  | Each | 238.00 |
| 192 | 300 mm long |  |  |  |  |  | Each | 258.00 |
|  | MILD STEEL (POWDER COATED) |  |  |  |  |  |  |  |
| 193 | 200 mm long |  |  |  |  |  | Each | 80.00 |
| 194 | 250 mm long |  |  |  |  |  | Each | 104.00 |
| 195 | 300 mm long |  |  |  |  |  | Each | 124.00 |
|  | 5)FLAT LATCHES |  |  |  |  |  |  |  |
|  | BRASS |  |  |  |  |  |  |  |
| 196 | Door Stoppers |  |  |  |  |  | Each | 135.00 |
| 197 | Windows Stay |  |  |  |  |  | Each | 154.00 |
|  | ALUMINIUM |  |  |  |  |  |  |  |
| 198 | 200 mm long |  |  |  |  |  | Each | 96.00 |
| 199 | 250 mm long |  |  |  |  |  | Each | 109.00 |
| 200 | 300 mm long |  |  |  |  |  | Each | 115.00 |
| 201 | 350 mm ISI marked IS 204 |  |  |  |  |  | Each | 324.00 |
| 202 | 450 mm ISI marked IS 204 |  |  |  |  |  | Each | 410.00 |
|  | MILD STEEL (POWDER COATED) |  |  |  |  |  |  |  |
| 203 | 250 mm long |  |  |  |  |  | Each | 39.00 |
| 204 | 300 mm long |  |  |  |  |  | Each | 50.00 |
| 205 | Door Stoppers |  |  |  |  |  | Each | 34.00 |
| 206 | Supply & fixing drapery rods (25.4 mm) 1" x 21 G Powder coated with PVC rings |  |  |  |  |  | Rft | 27.00 |
| 207 | Supply & fixing drapery bracket - brass powder coated |  |  |  |  |  | pair | 309.00 |
| 208 | Deluxe window stay Aluminium |  |  |  |  |  | Each | 88.00 |
| 209 | Heavy duty Aluminium Door stopper |  |  |  |  |  | Each | 38.00 |
| 210 | Brass fancy handles 150 mm |  |  |  |  |  | Each | 230.00 |
| 211 | Brass fancy handles 200 mm |  |  |  |  |  | Each | 312.00 |
| 212 | Brass fancy handles 250 mm |  |  |  |  |  | Each | 562.00 |
| 213 | Brass fancy handles 300 mm |  |  |  |  |  | Each | 718.00 |
| 214 | Brass fancy handles 450 mm |  |  |  |  |  | Each | 1122.00 |
| 215 | Door closer automatic hyper brand IS 3564 |  |  |  |  |  | Each | 766.00 |
| 216 | Double action brass plate floor spring Hyper brand IS 6315 |  |  |  |  |  | Each | 969.00 |
| 217 | Providing and fixing Floor spring 1st Quality for Aluminium door |  |  |  |  |  | Each | 2422.00 |
| 218 | Heavy duty mortise lock 6/7 levers with PC or CP handles complete set |  |  |  |  |  | Each | 474.00 |
| 219 | Heavy duty mortise lock 6/7 levers with brass heavy handles |  |  |  |  |  | Each | 1651.00 |
| 220 | Brass latch heavy 300 mm |  |  |  |  |  | Each | 394.00 |
| 221 | Brass latch heavy 350 mm |  |  |  |  |  | Each | 453.00 |
| 222 | Brass latch heavy 450 mm |  |  |  |  |  | Each | 598.00 |
| 223 | Friction stay hinges for windows |  |  |  |  |  | Each | 74.00 |
| 224 | Keyless lockset for toilet with CP or PC handles (complete set) |  |  |  |  |  | Each | 213.00 |
| 225 | Keyless lock set for toilet with brass handles |  |  |  |  |  | Each | 565.00 |
|  | (E) MISCELLANEOUS ITEMS |  |  |  |  |  |  |  |
| 226 | Aluminium sheet 24 Gauge |  |  |  |  |  | 1 Sqm | 262.00 |
| 227 | 7.5mm thick Aluminium Grill (as approved by the department) 3.58 Kg/Sqm. |  |  |  |  |  | 1 Sqm | 965.00 |
| 228 | Mastic pad 2'0 " x 4'0" (0.60M x 1.2M) of 1" (25.4 mm ) thick |  |  |  |  |  | 1 Sqm | 805.00 |
| 229 | Mastic pad 2'0 " x 4'0" (0.60M x 1.2M) of 1/2" (12.7 mm ) thick |  |  |  |  |  | 1 Sqm | 461.00 |
|  | Expanded metal sheets as per IS-412-1962 |  |  |  |  |  |  |  |
|  | a. Light Weight |  |  |  |  |  |  |  |
| 230 | 25 mm(SWM)x75 mm (LWM)3.25 mm Wide and 1.25 mm thick |  |  |  |  |  | 1 Sqm | 104.00 |
| 231 | 20 mm(SWM)x60 mm(LWM) 2.50 mm Wide and 1.25 mm thick |  |  |  |  |  | 1 Sqm | 104.00 |
| 232 | 20 mm(SWM)x50 mm(LWM) 2.50 mm Wide and 1.25 mm thick |  |  |  |  |  | 1 Sqm | 104.00 |
|  | b. Medium Weight |  |  |  |  |  |  |  |
| 233 | 25 mm(SWM)x75 mm(LWM) 3.25 mm Wide and 1.60 mm thick |  |  |  |  |  | 1 Sqm | 143.00 |
| 234 | 20 mm(SWM)x60 mm(LWM) 2.50 mm Wide and 1.60 mm thick |  |  |  |  |  | 1 Sqm | 143.00 |
| 235 | 20 mm(SWM)x50 mm(LWM) 2.50 mm Wide and 1.60 mm thick |  |  |  |  |  | 1 Sqm | 143.00 |
|  | c. Heavy Weight |  |  |  |  |  |  |  |
| 236 | 25 mm(SWM)x75 mm(LWM) 3.25 mm Wide and 2.24 mm thick |  |  |  |  |  | 1 Sqm | 163.00 |
| 237 | 20 mm(SWM)x60 mm(LWM) 2.50 mm Wide and 2.24 mm thick |  |  |  |  |  | 1 Sqm | 163.00 |
| 238 | 20 mm(SWM)x50 mm(LWM) 2.50 mm Wide and 2.24 mm thick |  |  |  |  |  | 1 Sqm | 163.00 |
| 239 | GI (M.S) fly proof wire mesh 16 x 20 gauge |  |  |  |  |  | 1 Sqm | 155.00 |
| 240 | SS fly proof mesh |  |  |  |  |  | 1 Sqm | 376.00 |
|  | Hot dipped Galvanized Iron Chain link mesh 8 gauge - (4.0 mm) as per IS: 2721-1979 |  |  |  |  |  |  |  |
|  | GI |  |  |  |  |  |  |  |
| 241 | 2"x2" (50 mm x 50 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 340.00 |
| 242 | 2½"x2½" (63 mm x 63 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 270.00 |
| 243 | 3"x3" (76 mm x 76 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 230.00 |
| 244 | 4"x4" (102 mm x 102 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 170.00 |
|  | PVC |  |  |  |  |  |  |  |
| 245 | 2"x2" (50 mm x 50 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 410.00 |
| 246 | 2½"x2½" (63 mm x 63 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 340.00 |
| 247 | 3"x3" (76 mm x 76 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 280.00 |
| 248 | 4"x4" (102 mm x 102 mm) - 8 Gauge |  |  |  |  |  | 1 Sqm | 220.00 |
| 249 | Rabbit wire mesh (chicken mesh) |  |  |  |  |  | 1 Sqm | 12.00 |
| 250 | Providing Tar felting roof repairs duly applying one coat of appropriate grade of bitumen in hot condition @ 1.2 Kg/1Sqm of roof area, fixing tar felt (Manufactures to IS 1322/1972 specification), applying second coat of appropriate grade of hot bitumen at 1.20 Kg/1Sqm of roof area and spreading course aggregate (sand) at 0.063 cum per 1 Sqm of roof area finished item |  |  |  |  |  | 1 Sqm | 172.00 |
| 251 | Providing waterproofing treatment by supplying and applying ROOFSHIELD-1500 OR EQUIVALENT (1.50 mm thickness, 2.25Kg per Sq. Mt and 20 X1 Mtr. roll with a centre core of 90 micron thickness HMHDPE film) Water Proofing Membrane.A) apply one coat of Bituminous Emulsion, first coat of bituminous grade 85/25, fix "ROOFSHIELD-1500 OR EQUIVALENT" water proofing membrane, apply second coat of Bitumen grade 90/15 after sealing all the joints and abutments with a Gas torch and apply one coat of BITUMINOUS ALUMINIUM PAINT for heat resistance for finished item |  |  |  |  |  | 1 Sqm. | 254.00 |
| 252 | Supply, laying & fixing of 60 mm thick cement Mosaic reflective concrete pavers from "Super decorative floorings" make of grade M-40 and above including labour charges, loading & unloading, laying to desired lines and levels complete. However cement, sand, water, electricity etc shall be provided by you free of cost at site. |  |  |  |  |  | 1 Sqm. | 610.00 |
| 253 | Supply, laying & fixing of 30 mm thick cement Mosaic reflective concrete pavers from "Super decorative floorings" make of grade M-40 and above including labour charges, loading & unloading, laying to desired lines and levels complete. However cement, sand, water, electricity etc shall be provided by you free of cost at site. |  |  |  |  |  | 1 Sqm. | 549.00 |
|  | PHOENIX or Equivalent Designer Tiles and Pavers |  |  |  |  |  |  |  |
| 254 | Supply of high strength Cement Concrete Designer Tiles of 30 mm thickness of M- 40 Grade of which top 12 mm thick shall be for colour pigmentation with imported colours and white cement to get required reflective glazing inclusive of all taxes |  |  |  |  |  | 1 Sqm | Rs.484.00 |
| 255 | Supply of high strength Cement Concrete interlocking Designer Pavers of 60 mm thickness of M- 40 Grade of which top 12 mm thick shall be for colour pigmentation with imported colours and white cement to get required reflective glazing inclusive of all taxes |  |  |  |  |  | 1 Sqm | Rs.533.00 |
| 256 | Supply of high strength Cement Concrete interlocking Designer Pavers of 80 mm thickness of M- 40 Grade of which top 12 mm thick shall be for colour pigmentation with imported colours and white cement to get required reflective glazing inclusive of all taxes |  |  |  |  |  | 1 Sqm | Rs.606.00 |
| 257 | Supply of high strength Cement Concrete interlocking Regular Pavers of 60 mm thickness of M- 40 Grade of which top 12 mm thick shall be for colour pigmentation with imported colours and white cement to get required reflective glazing inclusive of all taxes |  |  |  |  |  | 1 Sqm | Rs.291.00 |
| 258 | Supply of high strength Cement Concrete interlocking Regular Pavers of 80 mm thickness of M- 40 Grade of which top 12 mm thick shall be for colour pigmentation with imported colours and white cement to get required reflective glazing inclusive of all taxes |  |  |  |  |  | 1 Sqm | Rs.315.00 |
|  | WATER PROOFING & CONSTRUCTION CHEMICALS AND EPOXIES Any company make As per I.SI. Standards, First Quality Rate |  |  |  |  |  |  |  |
|  | A) Concrete Admixtures and Plasticizers |  |  |  |  |  |  |  |
|  | Conflow SNP1 or Equivalent |  |  |  |  |  |  |  |
| 259 | Pack size 1 liter |  |  |  |  |  | 1 liter | 46.00 |
|  | Conflow SNP20 or Equivalent |  |  |  |  |  |  |  |
| 260 | Pack size 1 liter |  |  |  |  |  | 1 liter | 58.00 |
|  | Conflow SNPA or Equivalent |  |  |  |  |  |  |  |
| 261 | Pack size 1 liter |  |  |  |  |  | 1 liter | 76.00 |
|  | B) Super Plasticizers |  |  |  |  |  |  |  |
|  | Conflow SNS I OR EQUIVALENT. |  |  |  |  |  |  |  |
| 262 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 58.00 |
|  | Conflow SNS 2 OR EQUIVALENT. |  |  |  |  |  |  |  |
| 263 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 58.00 |
|  | Accelerating Plasticizers |  |  |  |  |  |  |  |
|  | Conflow SNA OR EQUIVALENT. |  |  |  |  |  |  |  |
| 264 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 73.00 |
|  | Retarding Plasticizers |  |  |  |  |  |  |  |
|  | Conflow SNR OR EQUIVALENT. |  |  |  |  |  |  |  |
| 265 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 46.00 |
|  | Air Entraining Agents |  |  |  |  |  |  |  |
|  | Conflow SNE OR EQUIVALENT. |  |  |  |  |  |  |  |
| 266 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 46.00 |
|  | Retarding super Plasticizers |  |  |  |  |  |  |  |
|  | Conflow SNS3 SPL OR EQUIVALENT |  |  |  |  |  |  |  |
| 267 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 58.00 |
|  | Conflow SNS3 OR EQUIVALENT |  |  |  |  |  |  |  |
| 268 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 76.00 |
|  | Mortar Plasticizers |  |  |  |  |  |  |  |
|  | Morflow SN1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 269 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 22.00 |
|  | Integral Water Proofing compounds |  |  |  |  |  |  |  |
|  | Conflow SNW1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 270 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 67.00 |
|  | Conflow SNW2 OR EQUIVALENT |  |  |  |  |  |  |  |
| 271 | Pack size 25 Kg |  |  |  |  |  | Kg | 10.00 |
| 272 | Pack size 750 Gms |  |  |  |  |  | Pack | 12.00 |
|  | Block Admixtures |  |  |  |  |  |  |  |
|  | Conflow SNB OR EQUIVALENT |  |  |  |  |  |  |  |
| 273 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 23.00 |
|  | Sprayed Concrete Accelerators |  |  |  |  |  |  |  |
|  | Shotset SN1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 274 | pack size 250 Kg. |  |  |  |  |  | Kg | 23.00 |
|  | B) Concrete surface Treatments curing compounds |  |  |  |  |  |  |  |
|  | CURAID SN1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 275 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 58.00 |
|  | CURAID SN2 OR EQUIVALENT |  |  |  |  |  |  |  |
| 276 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 61.00 |
|  | Formwork & Mould Treatment |  |  |  |  |  |  |  |
|  | Shutterol SNU OR EQUIVALENT |  |  |  |  |  |  |  |
| 277 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 58.00 |
|  | Cleaning Agents |  |  |  |  |  |  |  |
|  | Uniklen SN1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 278 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 24.00 |
|  | Uniklen SN2 OR EQUIVALENT |  |  |  |  |  |  |  |
| 279 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 54.00 |
|  | Uniklen SN3 OR EQUIVALENT |  |  |  |  |  |  |  |
| 280 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 129.00 |
|  | C)Free flow - Non shrink Grouts |  |  |  |  |  |  |  |
|  | Pre – Packed Cementitious Grouts |  |  |  |  |  |  |  |
|  | Expacrete SN 50 OR EQUIVALENT |  |  |  |  |  |  |  |
| 281 | Pack size 25 Kg |  |  |  |  |  | Kg | 10.00 |
|  | Expacrete SN 70 OR EQUIVALENT |  |  |  |  |  |  |  |
| 282 | Pack size 25 Kg |  |  |  |  |  | Kg | 12.00 |
|  | Expacrete SN 90 OR EQUIVALENT |  |  |  |  |  |  |  |
| 283 | Pack size 25 Kg |  |  |  |  |  | Kg | 19.00 |
|  | Expacem SN 2 OR EQUIVALENT |  |  |  |  |  |  |  |
| 284 | Pack size 250 gm |  |  |  |  |  | pack | 29.00 |
|  | Epoxy Resin Grouts |  |  |  |  |  |  |  |
|  | Expacrete SNE 1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 285 | Pack size 1 Kg |  |  |  |  |  | Kg | 612.00 |
|  | Expacrete SNE 3 OR EQUIVALENT |  |  |  |  |  |  |  |
| 286 | Pack size 15 Kg |  |  |  |  |  | pack | 2333.00 |
|  | Polyester Resin Grouts |  |  |  |  |  |  |  |
|  | Anchrset SN1 OR EQUIVALENT |  |  |  |  |  |  |  |
| 287 | Pack size 1 Kg |  |  |  |  |  | pack | 146.00 |
|  | Anchrset SN2 OR EQUIVALENT |  |  |  |  |  |  |  |
| 288 | Pack size 10 Kg |  |  |  |  |  | pack | 1371.00 |
|  | D) Industrial Flooring systems |  |  |  |  |  |  |  |
|  | Floor Hardeners |  |  |  |  |  |  |  |
|  | Flortop SNC OR EQUIVALENT |  |  |  |  |  |  |  |
| 289 | pack size 25 Kg |  |  |  |  |  | Kg | 23.00 |
|  | Floor top SNS OR EQUIVALENT |  |  |  |  |  |  |  |
| 290 | Pack size 25 Kg. |  |  |  |  |  | Kg | 14.00 |
|  | Epoxy Floor coating & protective coatings |  |  |  |  |  |  |  |
|  | Procoat SNF or Equivalent |  |  |  |  |  |  |  |
| 291 | Pack size 1 Kg |  |  |  |  |  | Pack | 484.00 |
|  | Procoat SNS or Equivalent |  |  |  |  |  |  |  |
| 292 | Pack size 1 Kg |  |  |  |  |  | Pack | 402.00 |
|  | Essenputty SNE or Equivalent |  |  |  |  |  |  |  |
| 293 | pack size 1 Kg |  |  |  |  |  | Pack | 245.00 |
|  | Procoat SNZ or Equivalent |  |  |  |  |  |  |  |
| 294 | Pack size 1 Ltr |  |  |  |  |  | Pack | 583.00 |
|  | Procoat SNL or Equivalent |  |  |  |  |  |  |  |
| 295 | Pack size 2.5 Kg. |  |  |  |  |  | Pack | 846.00 |
|  | Epoxy Self Leveling (Flooring system) |  |  |  |  |  |  |  |
|  | Flortop SNSL or Equivalent |  |  |  |  |  |  |  |
| 296 | Pack size 14 Kg |  |  |  |  |  | Pack | 3383.00 |
|  | Flortop SNSL (SPL) or Equivalent |  |  |  |  |  |  |  |
| 297 | Pack size 14 Kg |  |  |  |  |  | Pack | 2578.00 |
|  | Heavy Duty Epoxy Base (Flooring system) |  |  |  |  |  |  |  |
|  | Flortop SNM or Equivalent |  |  |  |  |  |  |  |
| 298 | Pack size 16 Kg |  |  |  |  |  | Pack | 1371.00 |
|  | Flortop SNFM or Equivalent |  |  |  |  |  |  |  |
| 299 | Pack size 16 Kg |  |  |  |  |  | Pack | 1050.00 |
|  | Epoxy resin primer |  |  |  |  |  |  |  |
|  | Florcoat SNP or Equivalent |  |  |  |  |  |  |  |
| 300 | Pack size 1 Kg |  |  |  |  |  | Pack | 583.00 |
|  | Procoat SNC or Equivalent |  |  |  |  |  |  |  |
| 301 | pack size 3 Kg |  |  |  |  |  | Pack | 968.00 |
|  | Procoat SNP or Equivalent |  |  |  |  |  |  |  |
| 302 | pack size 1 Kg |  |  |  |  |  | Pack | 565.00 |
|  | Procoat SNP2 or Equivalent |  |  |  |  |  |  |  |
| 303 | pack size 3 Kg |  |  |  |  |  | Pack | 1015.00 |
|  | E) Repair Products |  |  |  |  |  |  |  |
|  | Repcrete SNC or Equivalent |  |  |  |  |  |  |  |
| 304 | Pack size 25 Kg |  |  |  |  |  | Kg | 14.00 |
|  | Repcrete SNE1 or Equivalent |  |  |  |  |  |  |  |
| 305 | Pack size 16 kg |  |  |  |  |  | Pack | 1610.00 |
|  | Repcrete SNW Primer or Equivalent |  |  |  |  |  |  |  |
| 306 | Pack size 1 Kg |  |  |  |  |  | Pack | 805.00 |
|  | Repcrete SNW Mortar or Equivalent |  |  |  |  |  |  |  |
| 307 | Pack size 1 Kg |  |  |  |  |  | Pack | 227.00 |
|  | Uniplug SN or Equivalent |  |  |  |  |  |  |  |
| 308 | pack size 5 Kg |  |  |  |  |  | Kg | 31.00 |
|  | F) Water Proofing Systems |  |  |  |  |  |  |  |
|  | Aqua coat SNF or Equivalent |  |  |  |  |  |  |  |
| 309 | Pack size 15+5 Kg |  |  |  |  |  | Pack | 1773.00 |
|  | Water Repellents |  |  |  |  |  |  |  |
|  | Aquarepel SN1 or Equivalent |  |  |  |  |  |  |  |
| 310 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 55.00 |
|  | G) Adhesive Tile Adhesives |  |  |  |  |  |  |  |
|  | Tilegrip SNU or Equivalent |  |  |  |  |  |  |  |
| 311 | Pack size 15 Kg |  |  |  |  |  | Kg | 19.00 |
|  | Bonding Adhesives |  |  |  |  |  |  |  |
|  | Conbond SNA or Equivalent |  |  |  |  |  |  |  |
| 312 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 245.00 |
|  | Conbond SNE or Equivalent |  |  |  |  |  |  |  |
| 313 | Pack size 1 Kg |  |  |  |  |  | Pack | 671.00 |
| 314 | Supply of concare anti-corrosion admixture inhibitor liquid as per Central Electro Chemical Research Institute developed specification |  |  |  |  |  | Ltr | 140.00 |
|  | WATER PROOFING & CONSTRUCTION CHEMICALS AND EPOXIES of "HI-BOND" (ISO 9001:2000 Regn.no.756979438) Any company make As per I.SI. Standards, First Quality Rate |  |  |  |  |  |  |  |
|  | A) Concrete Admixtures and Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST 01 or Equivalent |  |  |  |  |  |  |  |
| 315 | Pack size 1 liter |  |  |  |  |  | Ltr | 45.00 |
|  | HI-PLAST 02 or Equivalent |  |  |  |  |  |  |  |
| 316 | Pack size 1 liter |  |  |  |  |  | Ltr | 57.00 |
|  | HI-PLAST PA or Equivalent |  |  |  |  |  |  |  |
| 317 | Pack size 1 liter |  |  |  |  |  | Ltr | 75.00 |
|  | B) Super Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST SP or Equivalent |  |  |  |  |  |  |  |
| 318 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 57.00 |
|  | HI-PLAST SP AR or Equivalent |  |  |  |  |  |  |  |
|  | Pack size 1 Ltr |  |  |  |  |  | Ltr | 57.00 |
| 319 | Accelerating Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST AR or Equivalent |  |  |  |  |  |  |  |
|  | Pack size 1 Ltr |  |  |  |  |  | Ltr | 72.00 |
| 320 | Retarding Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST RT or Equivalent |  |  |  |  |  |  |  |
|  | Pack size 1 Ltr |  |  |  |  |  | Ltr | 45.00 |
| 321 | Air Entraining Agents |  |  |  |  |  |  |  |
|  | HI-PLAST AE or Equivalent |  |  |  |  |  |  |  |
|  | Pack size 1 Ltr |  |  |  |  |  | Ltr | 45.00 |
| 322 | Retarding Super Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST SP RT or Equivalent |  |  |  |  |  |  |  |
| 323 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 57.00 |
|  | HI-PLAST RSP or Equivalent |  |  |  |  |  |  |  |
| 324 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 75.00 |
|  | Mortar Plasticizers |  |  |  |  |  |  |  |
|  | HI-PLAST MP or Equivalent |  |  |  |  |  |  |  |
| 325 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 21.00 |
|  | Integral Water Proofing compounds |  |  |  |  |  |  |  |
|  | HI-PROOF (L) or Equivalent |  |  |  |  |  |  |  |
| 326 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 65.00 |
|  | HI-PROOF (P) or Equivalent |  |  |  |  |  |  |  |
| 327 | Pack size 25 Kg |  |  |  |  |  | Kg | 9.00 |
|  | Pack size 750 Gms |  |  |  |  |  | Pack | 11.00 |
| 328 | Block Admixtures |  |  |  |  |  |  |  |
|  | HI-ADMIX or Equivalent |  |  |  |  |  |  |  |
| 329 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 22.00 |
|  | Sprayed Concrete Accelerators |  |  |  |  |  |  |  |
|  | HI-ACCEL or Equivalent |  |  |  |  |  |  |  |
| 330 | Pack size 250 Kg |  |  |  |  |  | Kg | 22.00 |
|  | B) Concrete surface Treatments curing compounds |  |  |  |  |  |  |  |
|  | HI-CURE 01 or Equivalent |  |  |  |  |  |  |  |
| 331 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 57.00 |
|  | HI-CURE 02 or Equivalent |  |  |  |  |  |  |  |
| 332 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 60.00 |
|  | E) Repair Products |  |  |  |  |  |  |  |
|  | HI-CRM or Equivalent |  |  |  |  |  |  |  |
| 333 | Pack size 25 Kg |  |  |  |  |  | Kg | 13.00 |
|  | HI-CRM EX or Equivalent |  |  |  |  |  |  |  |
| 334 | Pack size 16 Kg |  |  |  |  |  | Pack | 1609.00 |
|  | HI-CRM EXW or Equivalent |  |  |  |  |  |  |  |
| 335 | Pack size 1 Kg |  |  |  |  |  | Pack | 804.00 |
|  | HI-CRM EX M or Equivalent |  |  |  |  |  |  |  |
| 336 | Pack size 1 Kg |  |  |  |  |  | Pack | 226.00 |
|  | F) Water Proofing Systems |  |  |  |  |  |  |  |
|  | HI-SHIELD or Equivalent |  |  |  |  |  |  |  |
| 337 | Pack size 15+5 Kg |  |  |  |  |  | Pack | 1772.00 |
|  | Water Repellents |  |  |  |  |  |  |  |
|  | HI-REPEL or Equivalent |  |  |  |  |  |  |  |
| 338 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 54.00 |
|  | G) Adhesive Tile Adhesives |  |  |  |  |  |  |  |
|  | HI-ADHESIVE or Equivalent |  |  |  |  |  |  |  |
| 339 | Pack size 15 Kg |  |  |  |  |  | Kg | 18.00 |
|  | Bonding Adhesives |  |  |  |  |  |  |  |
|  | HI-FIX HFA or Equivalent |  |  |  |  |  |  |  |
| 340 | Pack size 1 Ltr |  |  |  |  |  | Ltr | 244.00 |
|  | HI-FIX HFE or Equivalent |  |  |  |  |  |  |  |
| 341 | Pack size 1 Kg |  |  |  |  |  | Pack | 670.00 |
| 342 | HI-BOND Wall Putty Normal or Equivalent |  |  |  |  |  | 25 Kg | 375.00 |
| 343 | HI-BOND Wall Putty Fine or Equivalent |  |  |  |  |  | 25 Kg | 310.00 |
| 344 | HI-BOND Wall Putty Super Fine or Equivalent |  |  |  |  |  | 25 Kg | 336.00 |
| 345 | HI-BOND Wall Putty Super Fine (Water Resistant) or Equivalent |  |  |  |  |  | 25 Kg | 971.00 |
|  | Note: Literature for usage / application of above construction /water proofing materials, epoxies etc, will be obtained from manufacturers. |  |  |  |  |  |  |  |
|  | BITUMINOUS HOT SEALING COMPOUND |  |  |  |  |  |  |  |
| 346 | Grade 'A' I.S.I excluding ST |  |  |  |  |  | 1 Kg | 26.00 |
| 347 | Bitumen solution primer of approved quality |  |  |  |  |  | 1 Kg | 45.00 |
| 348 | Tar Felt Type - 2 Grade - 1 |  |  |  |  |  | 1 Sqm | 74.00 |
|  | GLASS - SAINT GOBIAN / MODIGUARD OR EQUIVALENT |  |  |  |  |  |  |  |
| 349 | Plain Float Glass 4 mm thick |  |  |  |  |  | 1 Sqm | 334.00 |
| 350 | Plain Float Glass 5 mm thick |  |  |  |  |  | 1 Sqm | 427.00 |
| 351 | Plain Float Glass 6 mm thick |  |  |  |  |  | 1 Sqm | 460.00 |
| 352 | Plain Float Glass 8 mm thick |  |  |  |  |  | 1 Sqm | 713.00 |
| 353 | Plain Float Glass 10 mm thick |  |  |  |  |  | 1 Sqm | 840.00 |
| 354 | Plain Float Glass 12 mm thick |  |  |  |  |  | 1 Sqm | 1047.00 |
| 355 | Tinted - Bronze/ Green Glass 4 mm thick |  |  |  |  |  | 1 Sqm | 453.00 |
| 356 | Tinted - Bronze/ Green Glass 5 mm thick |  |  |  |  |  | 1 Sqm | 570.00 |
| 357 | Tinted - Bronze/ Green Glass 6 mm thick |  |  |  |  |  | 1 Sqm | 638.00 |
| 358 | Tinted - Bronze/ Green Glass 8 mm thick |  |  |  |  |  | 1 Sqm | 1021.00 |
| 359 | Tinted - Bronze/ Green Glass 10 mm thick |  |  |  |  |  | 1 Sqm | 1304.00 |
| 360 | Tinted - Bronze/ Green Glass 12 mm thick |  |  |  |  |  | 1 Sqm | 1562.00 |
| 361 | Pin headed Glass 4 mm thick |  |  |  |  |  | 1 Sqm | 220.00 |
| 362 | Pin headed Glass 5 mm thick |  |  |  |  |  | 1 Sqm | 376.00 |
| 363 | Frosted / Ground Glass 4 mm thick |  |  |  |  |  | 1 Sqm | 485.00 |
| 364 | Frosted / Ground Glass 5 mm thick |  |  |  |  |  | 1 Sqm | 609.00 |
| 365 | Toughened Plain Glass 5 mm (including Glass) |  |  |  |  |  | 1 Sqm | 962.00 |
| 366 | Toughened Plain Glass 6 mm (including Glass) |  |  |  |  |  | 1 Sqm | 1107.00 |
| 367 | Toughened Plain Glass 8 mm (including Glass) |  |  |  |  |  | 1 Sqm | 1606.00 |
| 368 | Toughened Plain Glass 10 mm (including Glass) |  |  |  |  |  | 1 Sqm | 1892.00 |
| 369 | Toughened Plain Glass 12 mm (including Glass) |  |  |  |  |  | 1 Sqm | 2182.00 |
|  | Labour charges for Glass designing works on any thickness of Glass including fixing at site |  |  |  |  |  |  |  |
| 370 | Etching works |  |  |  |  |  | 1 Sqm | 436.00 |
| 371 | Reverse Etching work |  |  |  |  |  | 1 Sqm | 538.00 |
| 372 | Acid wash works |  |  |  |  |  | 1 Sqm | 673.00 |
|  | CEMENT JALI |  |  |  |  |  |  |  |
| 373 | 25 mm thick |  |  |  |  |  | 1 Sqm | 189.00 |
| 374 | 40 mm thick |  |  |  |  |  | 1 Sqm | 245.00 |
| 375 | 50 mm thick |  |  |  |  |  | 1 Sqm | 335.00 |
|  | BRUSHES |  |  |  |  |  |  |  |
| 376 | Wire Brush |  |  |  |  |  | Each | 29.00 |
| 377 | Soft Brush |  |  |  |  |  | Each | 130.00 |
|  | DISTEMPERS |  |  |  |  |  |  |  |
| 378 | Dry Powder Distemper |  |  |  |  |  | 1 Kg | 30.00 |
| 379 | Oil Bound Washable Distemper Acrylic Based |  |  |  |  |  | 1 Kg | 77.00 |
|  | POLISH |  |  |  |  |  |  |  |
| 380 | French Spirit Polish |  |  |  |  |  | 1 Ltr | 128.00 |
| 381 | Wax Polish (Ready made) |  |  |  |  |  | 1 Kg | 255.00 |
| 382 | Putty for steel work |  |  |  |  |  | 1 Kg | 18.00 |
| 383 | Putty for wood work |  |  |  |  |  | 1 Kg | 36.00 |
| 384 | Supply & applying Melamine Polish Glossy/ Matt finish to the wood works duly cleaning the surface and applying emery paper, Sand the wood with 180 No., emery paper and then with 320 No., emery paper, clean & wipe off loose dust, applying suitable knifing paste filler / wood filler by putty knife / muslin pad, air dry for 2 - 3 hrs, sand with 180 and 320 No., emery paper, apply two component wood sealer, air dry for 24 hrs, Sand with 320 No emery paper, applying one coat of approved spraying thinner (for spraying)/ applying one coat of approved brushing thinner or general purpose thinner (for brushing) and apply (either with spray or brush) two coats of approved brand melamine including cost & labour charges, emery papers, cost of thinner & melamine polish of approved brands such as Jenson & Nicholson, Asian Paints, Berger Paints or equivalent etc., complete for finished item of work |  |  |  |  |  | 1Sqm | 356.00 |
| 385 | Supply & applying Poly-Urethane Water Proof Polish Glossy/ Matt finish to the wood works duly cleaning the surface and applying emery paper, Sand the wood with 180 No., emery paper and then with 320 No., emery paper, clean & wipe off loose dust, applying suitable knifing paste filler / wood filler by putty knife / muslin pad, air dry for 2 - 3 hrs, sand with 180 and 320 No., emery paper, applying two component wood sealer, after the surface preparation applying one coat of approved spraying PU thinner (for spraying) / applying one coat of approved brushing PU thinner or general purpose thinner (for brushing) and apply one coat of PU by brush or spray, air-dry overnight, Sand again with 180 No. emery paper and removing dust, applying second coat of PU, air drying for 4 - 6 Hrs, Sand with 320 No emery paper, and applying (either with spray or brush) two coats of approved brand PU including cost & labour charges, emery papers, cost of thinner & PU of approved brands such as Jenson & Nicholson, Asian Paints, Berger Paints or equivalent etc., complete for finished item |  |  |  |  |  | 1Sqm | 651.00 |
|  | PAINTS (CONFORMING TO I.S.I) |  |  |  |  |  |  |  |
| 386 | Aluminium paint 1st Grade |  |  |  |  |  | 1 Ltr | 255.00 |
| 387 | Anti Corrosive bitumen paint (Black ) grade1 |  |  |  |  |  | 1 Ltr | 389.00 |
| 388 | Red Oxide Primer Paint Grade -1 |  |  |  |  |  | 1 Ltr | 88.00 |
| 389 | Red Oxide Primer Paint Grade – II |  |  |  |  |  | 1 Ltr | 67.00 |
| 390 | Synthetic Enamel paints in all shades Grade - I |  |  |  |  |  | 1 Ltr | 175.00 |
| 391 | Synthetic Enamel paints in all shades Grade - II |  |  |  |  |  | 1 Ltr | 130.00 |
| 392 | Plastic Emulsion Paint Grade – I |  |  |  |  |  | 1 Ltr | 189.00 |
| 393 | Water proof Cement paint of Superior Quality |  |  |  |  |  | 1 Kg | 35.00 |
| 394 | White Lead |  |  |  |  |  | 1 Kg | 74.00 |
| 395 | Marble Powder |  |  |  |  |  | 1 Kg | 19.00 |
| 396 | Cement primer Grade –I |  |  |  |  |  | 1 Kg | 84.00 |
| 397 | Cement primer Grade –II |  |  |  |  |  | 1 Kg | 65.00 |
| 398 | Impervious Water Proofing compound |  |  |  |  |  | 1 Kg | 24.00 |
| 399 | Fevicol/ Wood adhesive compound |  |  |  |  |  | 1 Kg | 156.00 |
| 400 | Wood Primer |  |  |  |  |  | 1 Ltr | 121.00 |
| 401 | Oil bound Distemper Primer. |  |  |  |  |  | 1 Ltr | 69.00 |
| 402 | Supplying and finishing (Altek or equivalent brand) to interior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying primary coat etc, complete for finished item of work. |  |  |  |  |  | 1 Sqm | 81.00 |
| 403 | Altek paint |  |  |  |  |  | 25 Kg | 360.00 |
| 404 | Trump |  |  |  |  |  | 1 Ltr | 165.00 |
| 405 | Snowcem or equivalent quality |  |  |  |  |  | 25 Kg | 950.00 |
| 406 | Surya Cem or equivalent quality |  |  |  |  |  | 25 Kg | 178.00 |
| 407 | a)Unitek Fine Grade or equivalent brand |  |  |  |  |  | 25kg | 285.00 |
| 408 | b)Supply and finishing of Unitek fine Grade or equivalent brand t with superfine grade to interior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work |  |  |  |  |  | 1 Sqm | 109.00 |
| 409 | Unitek Superfine Grade or equivalent brand |  |  |  |  |  | 25kg | 309.00 |
| 410 | Supply and finishing of Unitek Superfine Grade or equivalent brand to interior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work |  |  |  |  |  | 1sqm. | 77.00 |
| 411 | Unitek Granito Grade or equivalent brand |  |  |  |  |  | 1kg | 1426.00 |
| 412 | Unitek Granito Grade or equivalent brand |  |  |  |  |  | 5kg | 6950.00 |
| 413 | Unitek Granito Grade or equivalent brand |  |  |  |  |  | 20kg | 27621.00 |
| 414 | Supply and finishing of Unitek Granito Grade or equivalent brand to interior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work |  |  |  |  |  | 1sqm. | 1402.00 |
| 415 | a).Unitek Superfine (Water resistant) Grade or equivalent brand |  |  |  |  |  | 25kg | 891.00 |
| 416 | b).Supply and finishing of Unitek Superfine(Water resistant Grade or equivalent brand to interior and exterior faces of new walls and ceiling of approved colour as per manufacturers specifications including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work. |  |  |  |  |  | 1 Sqm. | 149.00 |
| 417 | Unitek Orient Grade or equivalent brand |  |  |  |  |  | 25kg | 1164.00 |
| 418 | Supply and finishing of Unitek Orient Grade or equivalent brand to interior and exterior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work |  |  |  |  |  | 1sqm. | 214.00 |
| 419 | Unitek Antico Grade or equivalent brand |  |  |  |  |  | 25kg | 1236.00 |
| 420 | Supply and finishing of Unitek Antico Grade or equivalent brand to interior and exterior faces of new walls and ceiling of approved colour as per manufacturers specification including cost and conveyance of materials to site and labour charges such as preparing the wall, applying the primary coat etc., complete for finished item of work |  |  |  |  |  | 1Sqm. | 196.00 |
| 421 | Supply & Application of Two Coats of Altek Super Fine, One Coat of Altek Water Based Cement Primer & Two Coats of Altek Flora (Plastic Emulsion Paint) |  |  |  |  |  | 1 Sqm. | 112.00 |
| 422 | Supply & Application of Two Coats of Altek Super Fine, One Coat of Altek Water Based Cement Primer & Two Coats of Delight (Acrylic Washable Distemper) |  |  |  |  |  | 1 Sqm. | 93.00 |
| 423 | Supply & Application One Coat of Altek Water Based Cement Primer & Two Coats of Altek Harmony (Acrylic Emulsion Paint Extra. Grade). |  |  |  |  |  | 1 Sqm. | 99.00 |
| 424 | Supply & Application of One Coat of Altek Water Based Cement Primer & Two Coats of Altek Alltimate (100% Pure Acrylic Emulsion Paint) |  |  |  |  |  | 1 Sqm. | 136.00 |
| 425 | Alltek 1127 Cement Based Puttty |  |  |  |  |  | 20 Kg | 600.00 |
| 426 | Alltek Top Coat High UV resistant Exterior Paint |  |  |  |  |  | 25 Kg | 1403.00 |
| 427 | Alltek Deco Orient Base Exterior Texture |  |  |  |  |  | 25 Kg | 540.00 |
| 428 | Alltek Goldline Exterior Texture |  |  |  |  |  | 25 Kg | 1312.00 |
| 429 | NCL Skimcoat 1127 is white cement based, polymer modified dry mix putty, specially made to work on leveled, cement sponge finished interior and exterior surfaces. Supply and application of Two Coats of Cement based putty 1127, inclusive of all taxes for finished item of work |  |  |  |  |  | 1 Sqm | 65.00 |
| 430 | Supply & application of One coat of Alltek Water base Cement Primer & two coats of top coat high UV resistant & water resistant exterior paint, inclusive of all taxes for finished item of work |  |  |  |  |  | 1 Sqm | 60.00 |
| 431 | Supply & application of One Coat Water based Cement Primer, One coat of Deco Orient Base Exterior Texture & two coats of Alltek Harmony (Acrylic Emulsion paint exterior grade) inclusive of all taxes for finished item of work |  |  |  |  |  | 1 Sqm | 215.00 |
| 432 | Supply & application of One coat of Water based Cement Primer, One coat of Goldline Exterior Roller Finish, inclusive of all taxes for finished item of work |  |  |  |  |  | 1 Sqm | 225.00 |
|  | ANTI TERMITE TREATMENT TO OLD & EXISTING BUILDINGS |  |  |  |  |  |  |  |
| 433 | Providing post construction anti termite treatment to drilling 1/2 inch dia., holes in 12mm depth at the edges of wall floor junction every foot apart and pouring anti termite chemical (Chloripyriphos 20% E.C / Lindane - 20 % E.C.) with a hand operator pressure pump until refusal or to a maximum of one litter per hole. Holes will sealed with cement motor after treatment and providing spraying and injection termiticides for treating and applying doors, windows, frames, removing, crusting treating with preventive chemicals (the plinth area of the building each floor shall be taken for purpose of payment) finished item including all costs, conveyances |  |  |  |  |  | 1 Sqm. | 130.00 |
| 434 | Providing pre construction anti termite treatment for new buildings: Providing, applying and Injecting for pre construction anti termite treatment to wall trenches bottom of excavation, foundations, vertical walls, top surface of plinth filling wall and floor junctions, joints, conduits and external perimeter of the building (Chloripyriphos 20% E.C / Lindane - 20 % E.C. ) with a hand operator pressure pump at applicable areas as per site condition in various stages as per IS: 6313-2003 (Par-II), the plinth area of the building each floor shall be taken for purpose of payment and finished item including all costs, conveyances |  |  |  |  |  | 1 Sqm. | 146.00 |
| 435 | Vinycide, Matt finish plastic emulsion Artilin Anti Insect paint OR EQUIVALENT for interior walls and ceilings, which will be applied on cement plastered surface, brick, old matt finish paints adhering to the substrate, wood etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 226.00 |
| 436 | Kremasoie INS, Semi gloss finish Artilin Anti Insect paint OR EQUIVALENT for interior and exterior surfaces, which will be applied on cement plastered surface, brick, old matt finish paints adhering to the substrate, wood, wood products, plastic products, tiles, glass textile covering etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 262.00 |
| 437 | Remarry, Matt finish Artilin Anti Insect paint OR EQUIVALENT for interior and exterior, which will be applied on cement plastered surface, brick, old matt finish paints adhering to the substrate, wood etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 287.00 |
| 438 | Vernis 107, Semi gloss finish transparent Artilin Anti Insect Varnish OR EQUIVALENT for wood work, works against wood borers and termites also for interiors only, which will be applied on wooden surfaces, steel surfaces and tiles, washable finished item . |  |  |  |  |  | 1 Sqm. | 262.00 |
| 439 | Kermasoie Fongicide, Semi gloss finish Artilin Anti Fungus paint OR EQUIVALENT for interiors and exteriors which will be applied on cement plastered surface, brick, old Matt finish paints adhering to the substrate, wood and wood products, plaster products, tiles and glass textile covering etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 237.00 |
| 440 | Kermasoie INS& Fong, Semi gloss finish acrylic Artilin Anti Insect and Anti Fungus paint OR EQUIVALENT for interiors and exteriors which will be applied on cement plastered surface, brick, old Matt finish paints adhering to the substrate, wood and wood products, plaster products, tiles and glass textile covering etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 292.00 |
| 441 | Bactesoie, Semi gloss finish Artilin bactericidal and fungicidal paint OR EQUIVALENT for interiors only which will be applied on cement plastered surface, brick, old Matt finish paints adhering to the substrate, wood and wood products, plaster products, tiles and glass textile covering etc., washable finished item . |  |  |  |  |  | 1 Sqm. | 320.00 |
|  | Particle Board (Exterior Grade) (Bhutan or Equivalent) |  |  |  |  |  |  |  |
|  | A. Plain Particle Board |  |  |  |  |  |  |  |
| 442 | 8 mm thick |  |  |  |  |  | 1 Sqm | 229.00 |
| 443 | 12 mm thick |  |  |  |  |  | 1 Sqm | 255.00 |
| 444 | 18 mm thick |  |  |  |  |  | 1 Sqm | 334.00 |
| 445 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 392.00 |
| 446 | 25 mm thick |  |  |  |  |  | 1 Sqm | 444.00 |
| 447 | 35 mm thick |  |  |  |  |  | 1 Sqm | 726.00 |
|  | Melamine Faced prevaricated |  |  |  |  |  |  |  |
|  | B. Particle Board (Single Deco) |  |  |  |  |  |  |  |
| 448 | 8 mm thick |  |  |  |  |  | 1 Sqm | 475.00 |
| 449 | 12 mm thick |  |  |  |  |  | 1 Sqm | 538.00 |
| 450 | 18 mm thick |  |  |  |  |  | 1 Sqm | 624.00 |
| 451 | 25 mm thick |  |  |  |  |  | 1 Sqm | 707.00 |
| 452 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1176.00 |
|  | Melamine Faced prelaminated |  |  |  |  |  |  |  |
|  | C. Particle Board (Twin Deco) |  |  |  |  |  |  |  |
| 453 | 8 mm thick |  |  |  |  |  | 1 Sqm | 546.00 |
| 454 | 12 mm thick |  |  |  |  |  | 1 Sqm | 624.00 |
| 455 | 18 mm thick |  |  |  |  |  | 1 Sqm | 722.00 |
| 456 | 25 mm thick |  |  |  |  |  | 1 Sqm | 799.00 |
| 457 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1503.00 |
|  | Particle Board (Interior Grade)(Bhutan or Equivalent) |  |  |  |  |  |  |  |
|  | A. Plain Particle Board |  |  |  |  |  |  |  |
| 458 | 6 mm thick |  |  |  |  |  | 1 Sqm | 183.00 |
| 459 | 8 mm thick |  |  |  |  |  | 1 Sqm | 196.00 |
| 460 | 10 mm thick |  |  |  |  |  | 1 Sqm | 216.00 |
| 461 | 12 mm thick |  |  |  |  |  | 1 Sqm | 229.00 |
| 462 | 18 mm thick |  |  |  |  |  | 1 Sqm | 282.00 |
| 463 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 334.00 |
| 464 | 25 mm thick |  |  |  |  |  | 1 Sqm | 353.00 |
| 465 | 35 mm thick |  |  |  |  |  | 1 Sqm | 614.00 |
|  | Melamine Faced Prelaminated |  |  |  |  |  |  |  |
|  | B. Particle Board (Single Deco) |  |  |  |  |  |  |  |
| 466 | 8 mm thick |  |  |  |  |  | 1 Sqm | 418.00 |
| 467 | 10 mm thick |  |  |  |  |  | 1 Sqm | 443.00 |
| 468 | 12 mm thick |  |  |  |  |  | 1 Sqm | 468.00 |
| 469 | 18 mm thick |  |  |  |  |  | 1 Sqm | 538.00 |
| 470 | 25 mm thick |  |  |  |  |  | 1 Sqm | 608.00 |
|  | Melamine Faced prelaminated |  |  |  |  |  |  |  |
|  | C. Particle Board (Twin Deco) |  |  |  |  |  |  |  |
| 471 | 8 mm thick |  |  |  |  |  | 1 Sqm | 478.00 |
| 472 | 10 mm thick |  |  |  |  |  | 1 Sqm | 523.00 |
| 473 | 12 mm thick |  |  |  |  |  | 1 Sqm | 542.00 |
| 474 | 18 mm thick |  |  |  |  |  | 1 Sqm | 620.00 |
| 475 | 25 mm thick |  |  |  |  |  | 1 Sqm | 682.00 |
|  | NATURAL COLOUR INSULATING BOARD |  |  |  |  |  |  |  |
| 476 | 12 mm thick |  |  |  |  |  | 1 Sqm | 177.00 |
| 477 | 18 mm thick |  |  |  |  |  | 1 Sqm | 261.00 |
| 478 | 25 mm thick |  |  |  |  |  | 1 Sqm | 333.00 |
|  | WHITE FACE INSULATING BOARD |  |  |  |  |  |  |  |
| 479 | 12 mm thick |  |  |  |  |  | 1 Sqm | 216.00 |
| 480 | 18 mm thick |  |  |  |  |  | 1 Sqm | 308.00 |
| 481 | 25 mm thick |  |  |  |  |  | 1 Sqm | 274.00 |
| 482 | Extra for veneered particle Board with commercial veneering on both sides. |  |  |  |  |  | 1 Sqm | 105.00 |
| 483 | Veneering on both sides with one side teak veneered and other side commercial veneered |  |  |  |  |  | 1 Sqm | 222.00 |
| 484 | Teak veneering on both sides |  |  |  |  |  | 1Sqm | 347.00 |
| 485 | Particle Board 12 mm thick (Three Layer) |  |  |  |  |  | 1 Sqm | 274.00 |
|  | PANELS |  |  |  |  |  |  |  |
|  | Plain particle Board (PPB) (Interior Grade) |  |  |  |  |  |  |  |
| 486 | 9 mm thick |  |  |  |  |  | 1 Sqm | 226.00 |
| 487 | 12 mm thick |  |  |  |  |  | 1Sqm | 251.00 |
| 488 | 15 mm thick |  |  |  |  |  | 1 Sqm | 284.00 |
| 489 | 18 mm thick |  |  |  |  |  | 1Sqm | 314.00 |
| 490 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 356.00 |
| 491 | 25 mm thick |  |  |  |  |  | 1Sqm | 378.00 |
|  | Plain particle Board (PPB) (Exterior Grade) |  |  |  |  |  |  |  |
| 492 | 9 mm thick |  |  |  |  |  | 1 Sqm | 301.00 |
| 493 | 12 mm thick |  |  |  |  |  | 1Sqm | 325.00 |
| 494 | 15 mm thick |  |  |  |  |  | 1 Sqm | 389.00 |
| 495 | 18 mm thick |  |  |  |  |  | 1Sqm | 424.00 |
| 496 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 491.00 |
| 497 | 25 mm thick |  |  |  |  |  | 1Sqm | 531.00 |
|  | Prelaminated particle Board (OSL) (Interior Grade) |  |  |  |  |  |  |  |
| 498 | 9 mm thick |  |  |  |  |  | 1 Sqm | 461.00 |
| 499 | 12 mm thick |  |  |  |  |  | 1Sqm | 497.00 |
| 500 | 15 mm thick |  |  |  |  |  | 1 Sqm | 548.00 |
| 501 | 18 mm thick |  |  |  |  |  | 1Sqm | 582.00 |
| 502 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 625.00 |
| 503 | 25 mm thick |  |  |  |  |  | 1Sqm | 653.00 |
|  | Prelaminated particle Board (OSL) (Exterior Grade) |  |  |  |  |  |  |  |
| 504 | 9 mm thick |  |  |  |  |  | 1 Sqm | 518.00 |
| 505 | 12 mm thick |  |  |  |  |  | 1Sqm | 582.00 |
| 506 | 15 mm thick |  |  |  |  |  | 1 Sqm | 622.00 |
| 507 | 18 mm thick |  |  |  |  |  | 1Sqm | 661.00 |
| 508 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 727.00 |
| 509 | 25 mm thick |  |  |  |  |  | 1Sqm | 769.00 |
|  | Prelaminated particle Board (BSL) (Interior Grade ) |  |  |  |  |  |  |  |
| 510 | 9 mm thick |  |  |  |  |  | 1 Sqm | 523.00 |
| 511 | 12 mm thick |  |  |  |  |  | 1Sqm | 568.00 |
| 512 | 15 mm thick |  |  |  |  |  | 1 Sqm | 603.00 |
| 513 | 18 mm thick |  |  |  |  |  | 1Sqm | 646.00 |
| 514 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 727.00 |
| 515 | 25 mm thick |  |  |  |  |  | 1Sqm | 811.00 |
|  | Prelaminated particle Board (BSL) (Exterior Grade) |  |  |  |  |  |  |  |
| 516 | 9 mm thick |  |  |  |  |  | 1 Sqm | 580.00 |
| 517 | 12 mm thick |  |  |  |  |  | 1Sqm | 650.00 |
| 518 | 15 mm thick |  |  |  |  |  | 1 Sqm | 708.00 |
| 519 | 18 mm thick |  |  |  |  |  | 1Sqm | 781.00 |
| 520 | 21.4 mm thick |  |  |  |  |  | 1 Sqm | 815.00 |
| 521 | 25 mm thick |  |  |  |  |  | 1Sqm | 848.00 |
|  | LAMINATED SHEETS (EQUIVALENT TO DECOLAM) |  |  |  |  |  |  |  |
|  | Glossy Finish |  |  |  |  |  |  |  |
| 522 | 1.00 mm thick |  |  |  |  |  | 1 Sqm | 282.00 |
| 523 | 1.50 mm thick |  |  |  |  |  | 1Sqm | 405.00 |
| 524 | 1.80 mm thick |  |  |  |  |  | 1Sqm | 444.00 |
|  | Mat Finish |  |  |  |  |  |  |  |
| 525 | 1.00 mm thick |  |  |  |  |  | 1 Sqm | 301.00 |
| 526 | 1.50 mm thick |  |  |  |  |  | 1 Sqm | 418.00 |
| 527 | 1.80 mm thick |  |  |  |  |  | 1Sqm | 451.00 |
|  | MEDIUM FIBRE BOARD INTERIOR GRADE (EQUIVALENT TO NUWUD) |  |  |  |  |  |  |  |
| 528 | 6 mm thick |  |  |  |  |  | 1 Sqm | 288.00 |
| 529 | 8 mm thick |  |  |  |  |  | 1Sqm | 327.00 |
| 530 | 12 mm thick |  |  |  |  |  | 1 Sqm | 438.00 |
| 531 | 15 mm thick |  |  |  |  |  | 1Sqm | 556.00 |
| 532 | 18 mm thick |  |  |  |  |  | 1 Sqm | 621.00 |
| 533 | 25 mm thick |  |  |  |  |  | 1Sqm | 883.00 |
| 534 | 30 mm thick |  |  |  |  |  | 1 Sqm | 1066.00 |
| 535 | 35 mm thick |  |  |  |  |  | 1Sqm | 1163.00 |
|  | MEDIUM FIBRE BOARD EXTRIOR GRADE (EQUIVALENT TO NUWUD) |  |  |  |  |  |  |  |
| 536 | 6 MM Thick |  |  |  |  |  | 1 Sqm. | 393.00 |
| 537 | 8 MM Thick |  |  |  |  |  | 1 Sqm. | 457.00 |
| 538 | 12 MM Thick |  |  |  |  |  | 1 Sqm. | 605.00 |
| 539 | 15 MM Thick |  |  |  |  |  | 1 Sqm. | 760.00 |
| 540 | 18 MM Thick |  |  |  |  |  | 1 Sqm. | 851.00 |
| 541 | 25 MM Thick |  |  |  |  |  | 1 Sqm. | 1135.00 |
| 542 | 30 MM Thick |  |  |  |  |  | 1 Sqm. | 1413.00 |
| 543 | 35 MM Thick |  |  |  |  |  | 1 Sqm. | 1669.00 |
|  | MEDIUM DENSITY FIBRE ECO BOARD or Equivalent |  |  |  |  |  |  |  |
|  | PLAIN PARTICLE BOARD (INTERIOR GRADE) |  |  |  |  |  |  |  |
| 544 | 9 mm thick |  |  |  |  |  | 1 Sqm | 264.00 |
| 545 | 12 mm thick |  |  |  |  |  | 1 Sqm | 301.00 |
| 546 | 18 mm thick |  |  |  |  |  | 1 Sqm | 372.00 |
| 547 | 25 mm thick |  |  |  |  |  | 1 Sqm | 448.00 |
| 548 | 30 mm thick |  |  |  |  |  | 1 Sqm | 681.00 |
| 549 | 35 mm thick |  |  |  |  |  | 1 Sqm | 805.00 |
|  | PLAIN PARTICLE BOARD (EXTERIOR GRADE) |  |  |  |  |  |  |  |
| 550 | 9 mm thick |  |  |  |  |  | 1 Sqm | 346.00 |
| 551 | 12 mm thick |  |  |  |  |  | 1 Sqm | 372.00 |
| 552 | 18 mm thick |  |  |  |  |  | 1 Sqm | 489.00 |
| 553 | 25 mm thick |  |  |  |  |  | 1 Sqm | 613.00 |
| 554 | 30 mm thick |  |  |  |  |  | 1 Sqm | 805.00 |
| 555 | 35 mm thick |  |  |  |  |  | 1 Sqm | 929.00 |
|  | PRE-LAMINATED PARTICLE BOARD |  |  |  |  |  |  |  |
|  | ONE SIDE LAMINATED INTERIOR GRADE |  |  |  |  |  |  |  |
| 556 | 9 mm thick |  |  |  |  |  | 1 Sqm | 415.00 |
| 557 | 12 mm thick |  |  |  |  |  | 1 Sqm | 469.00 |
| 558 | 18 mm thick |  |  |  |  |  | 1 Sqm | 545.00 |
| 559 | 25 mm thick |  |  |  |  |  | 1 Sqm | 613.00 |
| 560 | 30 mm thick |  |  |  |  |  | 1 Sqm | 886.00 |
| 561 | 35 mm thick |  |  |  |  |  | 1 Sqm | 945.00 |
|  | PRE-LAMINATED PARTICLE BOARD |  |  |  |  |  |  |  |
|  | ONE SIDE LAMINATED EXTERIOR GRADE |  |  |  |  |  |  |  |
| 562 | 9 mm thick |  |  |  |  |  | 1 Sqm | 477.00 |
| 563 | 12 mm thick |  |  |  |  |  | 1 Sqm | 534.00 |
| 564 | 18 mm thick |  |  |  |  |  | 1 Sqm | 607.00 |
| 565 | 25 mm thick |  |  |  |  |  | 1 Sqm | 709.00 |
| 566 | 30 mm thick |  |  |  |  |  | 1 Sqm | 945.00 |
| 567 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1004.00 |
|  | PRE-LAMINATED PARTICLE BOARD |  |  |  |  |  |  |  |
|  | BOTH SIDE LAMINATED INTERIOR GRADE |  |  |  |  |  |  |  |
| 568 | 9 mm thick |  |  |  |  |  | 1 Sqm | 474.00 |
| 569 | 12 mm thick |  |  |  |  |  | 1 Sqm | 534.00 |
| 570 | 18 mm thick |  |  |  |  |  | 1 Sqm | 613.00 |
| 571 | 25 mm thick |  |  |  |  |  | 1 Sqm | 769.00 |
| 572 | 30 mm thick |  |  |  |  |  | 1 Sqm | 945.00 |
| 573 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1004.00 |
|  | PRE-LAMINATED PARTICLE BOARD |  |  |  |  |  |  |  |
|  | BOTH SIDE LAMINATED EXTERIOR GRADE |  |  |  |  |  |  |  |
| 574 | 9 mm thick |  |  |  |  |  | 1 Sqm | 534.00 |
| 575 | 12 mm thick |  |  |  |  |  | 1 Sqm | 606.00 |
| 576 | 18 mm thick |  |  |  |  |  | 1 Sqm | 716.00 |
| 577 | 25 mm thick |  |  |  |  |  | 1 Sqm | 784.00 |
| 578 | 30 mm thick |  |  |  |  |  | 1 Sqm | 992.00 |
| 579 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1063.00 |
| 580 | DECOLAM SHEET GLOSSY FINISH-1mm thick |  |  |  |  |  | 1 Sam | 317.00 |
| 581 | DECOLAM SHEET MATT FINISH – 1mm thick |  |  |  |  |  | 1 Sqm | 342.00 |
| 582 | Teak Veneer Lamination - 4 mm thick - Pencil Grains |  |  |  |  |  | 1 Sqm | 316.00 |
| 583 | Teak Veneer Lamination - 4 mm thick - Natural Grains |  |  |  |  |  | 1 Sqm | 582.00 |
|  | PARTICLE BOARD CEILINGS |  |  |  |  |  |  |  |
| 584 | Gypsum Board 0.595 M x 0.595 M of 12.5 mm thick as per IS 2095 -1992 |  |  |  |  |  | 1 Sqm | 220.00 |
| 585 | False ceiling with 12.5 mm Gypsum Board IS 2095 -1982 using Gyp steel section including installation and complete finish excluding painting. |  |  |  |  |  | 1 Sqm | 675.00 |
| 586 | Supply & fixing Gyp board 0.595 x 0.595M 12.5mm thick tile using fine line grid system including installation and complete finished excluding painting |  |  |  |  |  | 1 Sqm | 740.00 |
|  | Gypboard wall lining : |  |  |  |  |  |  |  |
| 587 | Supply & fixing of Gypboard wall lining using 9.5/12.5 mm thick Gyp - board (conforming to IS 2095-1982), fixing to Gypsteel GI channels of perimeter channel (20mm x30mm x27mm x0.55mm thick) and ceiling channels of (51.5 mm x 26mm x26mmx10.5mmx0.55mm thick) and fixing and finishing using joint compound and paper tape complete finish as per specification without painting. |  |  |  |  |  | 1 Sqm | 662.00 |
|  | Gypboard plain false ceiling: |  |  |  |  |  |  |  |
| 588 | Supply and fixing of Gypboard plain false ceiling using 12.5 mm thick Gypboard conforming to IS 2095 -1982, fixing to Gypsteel GI channels frame work using perimeter channel (20mm x 30 mm x 27mm x 0.55mm) Intermediate channels (45mm x 15mm x 15 mm x 0.9mm) and ceiling channels (51.5 mm x26mm x10.5mm x 0.55 mm thick) jointing and finishing using joint compound and paper tape complete finish as per specification without painting |  |  |  |  |  | 1 Sqm | 740.00 |
|  | Gypboard (fine line) Grid suspended ceiling Supply and fixing of Gypboard (conforming to IS 2095-1982 ) FALSE CEILING WITH FINE LINE EXPOSED Grid suspended ceiling system using pre-coated galvanized steel main "T" section (24x38x0.35 mm thick x 3600 mm) Cross "T" (24x38x0.35mm x 600mm)and wall angle of (24x24x0.8 mm thick x 3050 mm) to form Grid size 600x 600 mm using MS Rods, "J" Bolts and anchor fasteners etc as per specification & as directed by dept, officers. |  |  |  |  |  |  |  |
| 589 | Using Plain Gypsum Board 9.5 mm laid on to the Grid |  |  |  |  |  | 1 Sqm | 600.00 |
| 590 | Using Plain Gypsum board 12.5 mm laid on to the Grid. |  |  |  |  |  | 1 Sqm | 640.00 |
|  | Gypboard metal stud partition single layer:- |  |  |  |  |  |  |  |
| 591 | Supply &fixing 75mm thick metal stud partition using 12.5 mm thick Gypboard conforming to IS 2095-1982 screw fixed to either side to 48 mm studs of 0.55 mm thick with the 50 mm floor and ceiling channel 0.55 mm and the joints duly finished with joint compound, paper tape and two coats Dry wall top coat as per specification. |  |  |  |  |  | 1 Sqm | 1072.00 |
| 592 | Providing and fixing in true horizontal level 600 mm x 600 mm 15 mm thick Arm strong false ceiling system manufactured by M/s Arm strong world Industries using hot dipped Galvanized Steel section exposed surface with pre-coated capping, main Tee of size 24 x 32 mm at every 1200 mm c/c maximum and rotary stitched cross tee of size 24 x 27 mm at every 600 mm c/c and sub-cross tee of size 24 mm x 25 mm at 1200 mm c/c and wall angle of size 19 x 19 mm fixed to periphery of the wall and the above grid is suspended at every 1200 mm c/c in both directions using 2.0 mm thick pre-straightened GI Wire laying fine fissured butt edge ceiling tiles of 15 mm thick mineral fiber Board manufactured by M/s Arm Strong World Industries Ltd., having RH 99% and for finished of size 600 x 600 mm including Cost &conveyance of all materials and labour charges such as cutting , fixing of standing of frame work exposing roof making complete for finished item of work |  |  |  |  |  | 1 Sqm | 720.00 |
| 593 | Providing wooden paneling 12 mm thick Novapan one side laminated sheet fixed on wooden frame of BT wood size 50.8 mm x 25.4 mm (2"x1") at distance of 609.6mm (2'-0") intervals of both ways providing 3/4" x 1/2" (19.05 mm x 12.7 mm) size BT wood ornamental cover beading and necessary screws, fevicol glue complete for finished item of work. |  |  |  |  |  | 1 Sqm | 1587.00 |
| 594 | Supply , manufacturing , installing of library Rack of 19 mm thick ISI marked commercial ply All round boxing verticals, Horizontal for depth of 609.6mm (2'-0) and shelves with 3.5 mm thick teak veneer covered with on exposed surface externally and internally and each panels with 6mm thick ISI commercial ply with 3.5 mm thick teak veneer for glazed shutters teak wood size 55 mm x 28mm with 5 mm thick glass fixed with ornamental beading and other ornamental beading is covered as per drawing including brass hinges, ornamental handles, ball catchers, buffers, locking system etc, and all the commercial ply exposed edges are to be covered with 6 mm thick teak wood lipping. All the teak veneer and teak wood and ornamental beading is coated with polishing and a coat of melamine finish etc, complete for finished item of work. |  |  |  |  |  | 1 Sqm | 6796.00 |
| 595 | Supply of Thermocole 12 mm thick sheets |  |  |  |  |  | 1 Sqm | 23.00 |
| 596 | Supply of Thermocole 19 mm thick sheets |  |  |  |  |  | 1 Sqm | 40.00 |
|  | Supply and fixing of Thermocole False ceiling in 2'-0"x2'-0" Grid (609.6x609.6 mm) fixing to anodized Aluminium 'L' angles of size 24x24x1mm for perimeter, Cross "T" of size 24x38x1 mm including fixing complete as per specification for finished item of work |  |  |  |  |  |  |  |
| 597 | 25 mm thick |  |  |  |  |  | 1 Sqm | 399.00 |
| 598 | 19 mm thick |  |  |  |  |  | 1 Sqm | 373.00 |
|  | MULTIPURPOSE RAMCO HILUX BOARDS (CALCIUM SILICATE BOARD) or EQUIVALENT |  |  |  |  |  |  |  |
| 599 | 6mm thick |  |  |  |  |  | 1sqm | 202.00 |
| 600 | 8mm thick |  |  |  |  |  | 1sqm | 270.00 |
| 601 | 10mm thick |  |  |  |  |  | 1sqm | 337.00 |
| 602 | 12mm thick |  |  |  |  |  | 1sqm | 404.00 |
|  | RAMCO HILUX BOARD EXPOSED GRID FALSE |  |  |  |  |  |  |  |
|  | CEILING OR EQUIVALENT |  |  |  |  |  |  |  |
| 603 | a) Supply and fixing of Ramco Hilux false ceiling with 6mm thick 595mm x 595mm Ramco Hilux board laid on to the grid using main "T" 24x38x3600mm, Cross "T" of size 24x38x1200mm and steel angle of size 24x24mm. |  |  |  |  |  | 1sqm | 547.00 |
| 604 | b) Using Ramco Hilux Board 0.595M x 0.595M 8mm thick |  |  |  |  |  | 1sqm | 614.00 |
| 605 | c) Using Ramco Hilux Board 0.595M x 0.595M 10mm thick |  |  |  |  |  | 1sqm | 681.00 |
| 606 | d) Using Ramco Hilux Board 0.595M x 0.595M 12mm thick |  |  |  |  |  | 1sqm | 749.00 |
|  | RAMCO HILUX BOARD PLAIN FALSE CEILING |  |  |  |  |  |  |  |
|  | OR EQUIVALENT |  |  |  |  |  |  |  |
|  | a) Supply and fixing of Ramco Hilux false ceiling fixing to Hilux G.I channels framework using perimeter channel (20mmx 30mmx27mmx0.55mm) Intermediate channel (45mmx15mmx15mmx0.9mm) and ceiling channels (51.5mmx26mmx10.5mmx0.55mm thick jointing and finishing using joint compound and glass fiber tape complete finished item of work as per specification without painting |  |  |  |  |  |  |  |
| 607 | a) 8mm thick Ramco Hilux board |  |  |  |  |  | 1sqm | 731.00 |
| 608 | b) Using 10mm thick Ramco Hilux board |  |  |  |  |  | 1sqm | 798.00 |
| 609 | c) Using 12mm thick Ramco Hilux board |  |  |  |  |  | 1sqm | 865.00 |
|  | RAMCO HILUX METAL STUD PARTITION SINGLE |  |  |  |  |  |  |  |
|  | LAYER OR EQUIVALENT |  |  |  |  |  |  |  |
|  | Supply and Fixing 66mm thick metal stud partition screw fixed to either side of 48mm studs of 0.55mm thick with 50mm floor and ceiling channel 0.55mm and the joints duly finished with joining compound, glass fiber tape and two coats dry wall top coat as per specification finished Item of work |  |  |  |  |  |  |  |
| 610 | a) using 8mm thick Ramco Hilux boards |  |  |  |  |  | 1sqm | 1131.00 |
| 611 | b) Using 10mm thick Ramco Hilux boards |  |  |  |  |  | 1sqm | 1266.00 |
| 612 | c) Using 12mm thick Ramco Hilux boards |  |  |  |  |  | 1sqm | 1400.00 |
|  | RAMCO HILUX WALL LINING or EQUIVALENT |  |  |  |  |  |  |  |
|  | Supply and fixing of Ramco Hilux wall lining fixing to GI channels of perimeter channel 30mm x 27mm x 0.55 thick and ceiling channels of 51.1mm x 26mm x 10.5mm x 0.55mm thick and fixing and finishing using joint compound and glass fiber tape complete finish as per specification without painting for finished item of work |  |  |  |  |  |  |  |
| 613 | a) using 8mmthick Ramco Hilux board |  |  |  |  |  | 1sqm | 671.00 |
| 614 | b) Using 10mm thick Ramco Hilux boards |  |  |  |  |  | 1sqm | 739.00 |
| 615 | c) Using 12mm thick Ramco Hilux boards |  |  |  |  |  | 1sqm | 806.00 |
|  | RAMCO HILUX FIRE RATED PARTITION-Two Layers OR EQUIVALENT |  |  |  |  |  |  |  |
| 616 | Supply and Fixing 66mm thick metal stud partition using 2 layers of 12mm thick Ramco Hilux boards screw fixed to either side to 48mm studs of 0.55mm thick with 50mm floor and ceiling channel 0.55mm.The boards joints are to be staggered to avoid through passage. Ceramic wool of 50mm thick, 64kg/m3 density is to be placed in cavity of the partition. The joints duly finished with joining compound, glass fiber tape and two coats dry wall top coat as per specification without painting for finished item of work |  |  |  |  |  | 1sqm | 3759.00 |
| 617 | Supply of Plaster of paris Boards 12 mm thick |  |  |  |  |  | 1 Sqm | 392.00 |
| 618 | Supply of Plaster of Paris Boards 10 mm thick |  |  |  |  |  | 1 Sqm | 327.00 |
| 619 | Labour charges for fixing plaster of paris false ceiling with for GI channels of perimeter channel 30mm x 27mm x 0.55 thick and ceiling channels of 51.1mm x 26mm x 10.5mm x 0.55mm thick and fixing and finishing using joint compound and luppum/ putty finish as per specification without painting including cost of all materials but excluding cost of Plaster of paris sheets/ boards for finished item of work |  |  |  |  |  | 1 Sqm | 426.00 |
|  | PLYWOOD ITEMS |  |  |  |  |  |  |  |
|  | PLYWOOD - IS 303 (Both Sides Commercial) |  |  |  |  |  |  |  |
|  | MR Grade as per IS 848 (UF Bonded) |  |  |  |  |  |  |  |
| 620 | 3 Ply - 4 mm thick Plywood |  |  |  |  |  | 1 Sqm | 278.00 |
| 621 | 5 Ply - 6 mm thick Plywood |  |  |  |  |  | 1 Sqm | 306.00 |
| 622 | 5 Ply - 8 mm thick Plywood |  |  |  |  |  | 1 Sqm | 360.00 |
| 623 | 5 Ply - 9 mm thick Plywood |  |  |  |  |  | 1 Sqm | 360.00 |
| 624 | 9 Ply - 12 mm thick Plywood |  |  |  |  |  | 1 Sqm | 453.00 |
| 625 | 9 Ply - 15 mm thick Plywood |  |  |  |  |  | 1 Sqm | 667.00 |
| 626 | 9 Ply - 18 mm thick Plywood |  |  |  |  |  | 1 Sqm | 653.00 |
|  | BWR/ BWP Grades - IS 848 (PF Bonded) |  |  |  |  |  |  |  |
| 627 | 3 Ply - 4 mm thick Plywood |  |  |  |  |  | 1 Sqm | 318.00 |
| 628 | 5 Ply - 6 mm thick Plywood |  |  |  |  |  | 1 Sqm | 373.00 |
| 629 | 5 Ply - 8 mm thick Plywood |  |  |  |  |  | 1 Sqm | 465.00 |
| 630 | 5 Ply - 9 mm thick Plywood |  |  |  |  |  | 1 Sqm | 439.00 |
| 631 | 9 Ply - 12 mm thick Plywood |  |  |  |  |  | 1 Sqm | 573.00 |
| 632 | 9 Ply - 15 mm thick Plywood |  |  |  |  |  | 1 Sqm | 823.00 |
| 633 | 9 Ply - 19 mm thick Plywood |  |  |  |  |  | 1 Sqm | 826.00 |
|  | VENEERED DECORATIVE PLYWOOD -IS 1328 |  |  |  |  |  |  |  |
|  | One Side Teak Veneer lamination of 1.0 mm thick |  |  |  |  |  |  |  |
| 634 | 3 Ply - 4 mm thick Plywood |  |  |  |  |  | 1 Sqm | 412.00 |
| 635 | 5 Ply - 6 mm thick Plywood |  |  |  |  |  | 1 Sqm | 536.00 |
| 636 | 5 Ply - 9 mm thick Plywood |  |  |  |  |  | 1 Sqm | 687.00 |
| 637 | 9 Ply - 13 mm thick Plywood |  |  |  |  |  | 1 Sqm | 876.00 |
|  | VENEERED DECORATIVE PLYWOOD-IS 1328 |  |  |  |  |  |  |  |
|  | Both Side Teak Veneer lamination of 1.0 mm thick |  |  |  |  |  |  |  |
| 638 | 5 Ply - 9 mm thick Plywood |  |  |  |  |  | 1 Sqm | 896.00 |
| 639 | 9 Ply - 12 mm thick Plywood |  |  |  |  |  | 1 Sqm | 1045.00 |
|  | RAJSHRI OR EQUIVALENT SOLID DOOR FRAME |  |  |  |  |  |  |  |
| 640 | Providing and fixing factory made polyvinyl chloride (PVC) Door Frame of the size 50 x 47mm with a wall thickness of 5mm, made out of extruded 5mm rigid PVC foam sheet, miter cut at two corners and joined with 2nos. of 150mm long brackets of 15x15mm M.S. square tube. The two vertical door profiles are to be reinforced with 19x19mm M.S. Square tube of 19 gauges. The door frame shall be fixed to the wall using 65/100mm long M.S. Screws through the frame by using PVC fasteners. A minimum of 4nos. of screws to be provided for each vertical member & minimum 2nos. for horizontal member etc. complete as per manufacturers specification and direction of Engineer-in-Charge for finished item of work |  |  |  |  |  | 1 RM | 303.00 |
|  | RAJSHRI OR EQUIVALENT SOLID PANEL DOOR SHUTTER |  |  |  |  |  |  |  |
| 641 | Providing and fixing 30mm thick Solid panel PVC door shutter consisting of frame made out of M.S. tubes of 19 gauge thickness and size of 19mm x 19mm for stiles, & 15mm x 15mm for top & bottom rails. M.S. frame shall have a coat of steel primers of approved make and manufacture. M.S. frame shall be covered with 5mm thick heat moulded PVC ‘C’ channel of size 30 x 50mm forming stiles, and 5mm thick, 75mm wide PVC sheets for top rail, lock rail & bottom rail on either side, and 10mm (5mm x 2) thick, 20mm wide cross PVC sheet as gap insert for top rail & bottom rail. Paneling of 5mm thick PVC sheet to be fitted in the M.S. frame welded / sealed to the stiles & rails with 30mm wide x 5mm thick PVC sheet beading on either side, and joined together with solvent cement adhesive etc. An additional 5mm thick PVC strip of 20mm width is to be stuck on the interior side of the ‘C’ Channel using PVC solvent adhesive. Complete as per direction of Engineer-in-charge, manufacturer’s specification & drawing for finished item of work. |  |  |  |  |  | 1 Sqm. | 2205.00 |
|  | RAJSHRI BOTH SIDE PRELAM SOLID PANEL PVC DOOR |  |  |  |  |  |  |  |
| 642 | Providing and fixing 30mm thick “RAJSHRI” or Equivalent BRAND FACTORY MADE BOTHSIDE PRELAM SOLID PANEL PVC DOOR SHUTTER consisting of frame made out of MS. Tubes of 19gauge thickness and size of 19mmX19mm for stiles, top and bottom rails. MS. Frame shall have a coat of steel primers of approved make and manufacture. MS. Frame covered with 5mm thick heat moulded “Rajshri” Prelam PVC ‘C’ channel of size 30mm thickness, 70mm width out of which 50mm shall be flat and 20mm shall be tapered in 450 angle on either side forming stiles; and 5mm thick, 95mm wide Rajshri Prelam PVC sheet out of which 75mm shall be flat and 20mm shall be tapered in 450 on the inner side to form top and bottom rail 115mm wide Rajshri Prelam PVC sheet out of which 75mm shall be flat and 20mm shall be tapered on both sides to form lock rail. Top, bottom and lock rail shall be provided either side of the panel. 10mm (5mmX2) thick, 20mmwide cross PVC sheet shall be provided as gap insert for top and bottom rail. Paneling of 5mm thick both side Prelam Pvc sheet to be fitted in the MS. Frame welded/ sealed to the stiles & rails with 7mm(5mm+2mm) thick X 15mm wide PVC sheet beading on inner side, and joined together with solvent cement adhesive etc. an additional 5mm thick PVC strip of 20mm width is to be stuck on the interior side of the ‘C’ channel using PVC solvent cement adhesive etc. complete as per direction of Engineer – in-charge, manufacture specification and drawing. |  |  |  |  |  | 1 Sqm | 2700.00 |
|  | RAJSHRI SOLD SINGLE SIDE PRELAM PANEL PVC DOOR |  |  |  |  |  |  |  |
| 643 | Providing and fixing 30mm thick “RAJSHRI” or Equivalent BRAND FACTORY MADE SOLID SINGLE SIDE PRELAM PANEL PVC DOOR SHUTTER consisting of frame made out of MS. Tubes of 19gauge thickness and size of 19mmX19mm for stiles, and 15mmX15mm for top and bottom rails. The MS. Frame shall have a coat of steel primers of approved make and manufacture. MS. Frame covered with 5mm thick heat moulded “Rajshri” single side Prelam Pvc ‘C’ channel of size 30mmX50mm to form stiles.5mm thick& 75mm wide single side prelim Pvc sheets shall be for the top, bottom and lock rails. Paneling of 5mm thick one side Pvc sheet to be fitted in the MS. Frame and sealed to the front (prelam side) stiles and rails (which should have a 5mm 45 bent portion as beading) and attached to the back(white/ivory) stiles and rails which should have a 10mm 900 bent portion as beading ) using 2nos. 15mm wide X 5mm thick Pvc sheet beading inside the 90 bent beading. All stiles, rails and beadings are to be joined to the panel using solvent cement etc. an additional 5mm thick Pvc strip of 20mm width to be stuck on the interior side of the ‘C” channel using Pvc solvent cement adhesive and 10mm (5mmX2) thick, 20mm wide cross Pvc sheet to be fitted as gap insert for top and bottom rail etc. complete as per direction of Engineer –in-Charge, manufacture specification and drawing. |  |  |  |  |  | 1 Sqm | 2450.00 |
| 644 | SUPPLY & FIXING or RAJSHRI or Equivalent SINGLE SIDE PRELAM SOLID PANEL PVC DOOR FRAME (CHOUKHAT) Providing and fixing factory made Pvc single side Prelam door frame of the size 50X47 mm with a wall thickness of 5mm, made out of extruded 5mm rigid Pvc single side prelam sheet, meter cut at two corners and joined with 2nos. of 150mm long brackets of 1515mm MS.Square tube. The two vertical door profiles are to be reinforced with 19X19mm MS. square tube of 19guage. The door frame shall be fixed to the wall using 65/100mm long MS. Screws through the frame by using Pvc fasteners. a minimum of 4nos. of screws to be provided for each vertical member and minimum 2nos. for horizontal member etc. complete as per manufacture specification and direction of Engineer-in-charge. |  |  |  |  |  | 1 RM | 340.00 |
|  | RAJSHRI MOULDED PVC DOOR – SOLID CORE |  |  |  |  |  |  |  |
| 645 | Providing and fixing 35mm thick “RAJSHRI” or Equivalent BRAND FACTORY MADE MOULDED DOOR SHUTTER consisting of solid core single leaf flush door of 30mm thickness, lipped with 15mm (5mmX3) thick X 30mm width on one stiles and top rails and 10mm (5mmX2) thick X 30mm width on the other stiles and bottom rails. The inner panel laminated with 2mm thick termite proof, water proof and fire resistant moulded Pvc sheet with 2,4,6 raised panel design in different panel and / or prelam colours on one side after routing the moulded design on flush door and 2mm prelam Pvc sheet on other side using rubber adhesive on flush door and solvent cement adhesive on Pvc lipping etc. complete as per direction of Engineer-in-Charge, manufacture specification and drawing |  |  |  |  |  | 1 Sqm | 2900.00 |
| 646 | RAJSHRI or Equivalent MOULDED PVC DOOR – WOOD FREE Providing and fixing 30mm thick Rajshri Brand factory made Moulded door shutter- wood free consisting of frame made out of MS. tubes 19gauge thickness and size of 25mmX25mm for stiles, top and bottom rails. MS. Frame shall have a coat of steel primers. The inner panel shall consist of 25mm thick High density EPS conforming to IS 4671-1984 bounded with 2mm thick termite proof, water proof and fire resistant moulded Pvc sheet with 2,4,6 raised panel design in different plain and / or prelam colours after routing to the moulded design on one side and 2mm plain and / or prelam Pvc sheet on other side of the EPS. The edge of panel to be sealed with lipping of 10mm wide PVC sheet bottom (made by sticking 2 rigid foam sheet of 5mm thickness using Pvc solvent cement) and stiles sides 25mm (5mmX5) thick and 30mm width Pvc sheet fitted along Ms. Tube for lock provision for lock height 5mm thick Pvc sheet of size 150mmX100mm fixed with upper and lower face of inner side of EPS Panel etc. Complete as per direction of Engineer – in-Charge, manufacture specification and drawing |  |  |  |  |  | 1 Sqm | 2475.00 |
|  | RAJSHRI WIRE MESH SOLID PVC DOOR |  |  |  |  |  |  |  |
| 647 | Providing and fixing 30mm thick “RAJSHRI” or Equivalent BRAND FACTORY MADE WIRE MESH SOLID PVC DOOR SHUTTER consisting of frame made out of M.S. tubes of 19 gauge thickness and size of 19mmX19mm for stiles and 15mm for top, lock & bottom rails. MS. Frame shall have a coat of steel primers of approved make and manufacture. 19mmX19mm M.S. square tubes shall be covered with 5mm thick heat moulded “Rajshri” PVC ‘C’ channel of size 30mm thickness, 90mm width out of which 75mm shall be flat and 15mm shall be flat and 15mm shall be bent in 900 on the inner side to from top and bottom rail land 130mm wide PVC sheet out of which 100mm shall be flat and 154mm shall be bent in 900 on both side to from lock rail, 10mm(5mmX2) thick, 20mm wide cross PVC sheet as gap insert for top rail & bottom rail. Panel will be of G.I. wire mesh of 24gauge to be fitted in by ’C’ channel of size 35mm X10mm on the either side in the stiles. The inner side ‘C’ channel is fixed by 19mmX19mm MS. Tube on both sides with screws. The wire mesh in top bottom & lock rail is sealed with15mm X 15mm tube with G.I. wire 10mm(5mmX2) thick PVC strip of 25mm width is to be stuck& nails on both side of G.I. wire mesh interior at the top, bottom & lock rail. An additional 5mm thick PVC strip of 20mm width is to be stuck on the interior side of the ‘C’ channel using PVC solvent cement adhesive etc. complete as per direction of Engineer-in-charge, manufacturer’s specification & drawings |  |  |  |  |  | 1 Sqm | 2200.00 |
|  | RAJSHRI SOLID PVC PARTITIONS |  |  |  |  |  |  |  |
| 648 | Providing and fixing 50mm thick “RAJSHRI” or Equivalent SEMI HEIGHT SOLID PVC PARTITION consisting of 49mm sandwich PVC panel made out of 36mm high density expanded polystyrene sheet(E.P.S.-Thermocole) stuck on both side with 2mm thick plain/printed /prelam PVC rigid foam sheets reinforced with frame work made from 35mmX5mm M.S. Angle. In between two M.S. Angles 5mm thick plain/printed/prelam heat moulded “Rajshri” PVC ‘C’ channel of size 50X100mm with feathered edges shall be fixed on the floor & 50X75mm with feathered edges plain/printed /prelam PVC ‘C’ channel. In between adjoining EPS sandwiched PVC partitions panel 5mm thickX75mm width plain/printed/prelam PVC sheet feathered at edges shall be stuck on front & back face using solvent cement etc. in places where two partitions meet to from an ‘L’ shape the panels are joined 5mm thick plain/printed/prelam heat moulded “Rajshri” PVC ‘L’ section of size 75mmX75mm with feathered edges shall be fixed on front & back side. This is done to maintain the symmetry of design. An additional 5mm thick PVC strip of 40mm width is to be stuck on the interior side of the ‘C’ Channel using PVC solvent adhesive, complete as per direction of Engineer-in-Charge, manufacture’s specification & drawing |  |  |  |  |  | 1 Sqm | 2700.00 |
|  | RAJSHRI INSULATED PVC WALL LINING |  |  |  |  |  |  |  |
| 649 | Providing and fixing “RAJSHRI” or Equivalent INSULATED PVC WALL PANELING shall consist of PVC insulated panels having wall thickness of 1mm /1.5mm/2mm PVC sheet and the desired thickness of EPS (dependent on the insulation required) screwed into the wall using screws of adequate length. The screws shall be placed at a distance of 150mm and shall be 25mm from the edge of the insulated panel on all from sides of the panel. The PVC strips of 5mm thick PVC sheet of width 75mm feathered at the edges shall be stuck using solvent cement adhesive at the places where two panels are joined, so as to cover the screw of both the panels, using solvent cement. 90 heat bent strips of 5mm thick PVC sheets of width 75mm X75mm feathered at the edges shall be stuck at wall joints / pillars etc. , complete as per direction of Engineer-in-Charge, manufacture’s specification & drawing |  |  |  |  |  | 1 Sqm | 2000.00 |
| 650 | RAJSHRI SOLID PVC WALL LINING Providing and fixing RAJSHRI or equivalent plain PVC rigid foam sheet wall paneling of thickness 5mm consisting of panels having size 1220mm X2440mm stuck on inner frame work of 25mm wide strips of 5mm thick PVC rigid foam sheet. The strips should have in their centre a grove of 6mm width and 2mm depth for sinking the head of the screw used to fix the strips to the wall and should be placed vertically at equal intervals of 405/407mm centre to c entre. The PVC strips shall be fixed to the wall using 65mm long M.S. screws through the groove in the strip by using PVC fasteners. The distance between two screws shall not be more than 300mm. a groove of 2mm may be left at the joint of two panels for aesthetic looks. This framework is done for making existing wall surface in ensured level and for sticking the wall paneling sheet (panel) on it. The exact size of the panel and correspondingly of the grid May be varied depending n the size of the wall. However it must ensured that a strips of 5mm thick PVC sheet of width 75mm feathered at the edges shall be stuck at the places where two panels are joined, using solvent cement. , complete as per direction of Engineer-in-Charge, manufacture’s specification & drawing |  |  |  |  |  | 1 Sqm | 1900.00 |
|  | RAJSHRI INSULATED PVC FALSE CELING |  |  |  |  |  |  |  |
| 651 | Providing and fixing RAJSHRI or Equivalent false ceiling consisting of 600mmX600mm, Solid PVC insulation panels having wall thickness of 1mm thick plain PVC rigid foam sheet and EPS as defined in of desired thickness depending on insulation required shall be insert in a frame work made using anodized aluminium Tee section of sizes 1”X1” (25X25mm, 19 gauge or 1mm thick) in square pattern of grid sizes of 2’X2’ (600X600mm). The aluminium framework is supported from the ceiling with the help of G.I. hook and G.I. wire/6mm.M.S. rods of required sizes to maintain proper level etc. the aluminium framework is supported on side wall with the use of aluminium 1”X1” (25X25mm) angles, complete as per direction of Engineer-in-Charge, manufacture’s specification & drawing |  |  |  |  |  | 1 Sqm | 1200.00 |
|  | DOOR FRAMES |  |  |  |  |  |  |  |
| 652 | MS. Hallow door frames manufactured by cold roll formed process steel sheet 1.25mm thick bright CRCA confirming to IS 4351 - 1976 with Hinges tower Bolts etc. |  |  |  |  |  | 1 RM | 233.00 |
|  | Flush door shutters, solid bond wood block board type with teak ply on both faces. |  |  |  |  |  |  |  |
| 653 | 25 mm thick |  |  |  |  |  | 1 Sqm | 1092.00 |
| 654 | 30 mm thick |  |  |  |  |  | 1 Sqm | 1129.00 |
| 655 | 35 mm thick |  |  |  |  |  | 1 Sqm | 1216.00 |
|  | Flush door shutters, solid bond wood block board type with commercial ply on both faces. |  |  |  |  |  |  |  |
| 656 | 25 mm thick |  |  |  |  |  | 1 Sqm | 556.00 |
| 657 | 30 mm thick |  |  |  |  |  | 1 Sqm | 626.00 |
| 658 | 35 mm thick |  |  |  |  |  | 1 Sqm | 686.00 |
| 659 | 40 mm thick |  |  |  |  |  | 1 Sqm | 817.00 |
|  | Flush door shutters, solid bond wood block board type with teak veneer on one face and commercial ply on another face (lipping) |  |  |  |  |  |  |  |
| 660 | 20 mm thick |  |  |  |  |  | 1 Sqm | 733.00 |
| 661 | 25 mm thick |  |  |  |  |  | 1 Sqm | 817.00 |
| 662 | 30 mm thick |  |  |  |  |  | 1 Sqm | 915.00 |
| 663 | 35 mm thick |  |  |  |  |  | 1 Sqm | 980.00 |
| 664 | 40 mm thick |  |  |  |  |  | 1 Sqm | 1045.00 |
|  | Plain cement bonded particle boards conforming to IS 14276 truly exterior grade (Bison panel or equivalent) |  |  |  |  |  |  |  |
| 665 | 6 mm thick |  |  |  |  |  | 1 Sqm | 178.00 |
| 666 | 8 mm thick |  |  |  |  |  | 1 Sqm | 205.00 |
| 667 | 10 mm thick |  |  |  |  |  | 1 Sqm | 237.00 |
| 668 | 12 mm thick |  |  |  |  |  | 1 Sqm | 277.00 |
| 669 | 16 mm thick |  |  |  |  |  | 1 Sqm | 365.00 |
| 670 | 20 mm thick |  |  |  |  |  | 1 Sqm | 456.00 |
| 671 | 25 mm thick |  |  |  |  |  | 1 Sqm | 569.00 |
| 672 | 30 mm thick |  |  |  |  |  | 1 Sqm | 682.00 |
| 673 | 40 mm thick |  |  |  |  |  | 1 Sqm | 912.00 |
|  | Prelaminated cement particle board both sides laminated truly exterior board (Bison Pre Lam or equivalent) |  |  |  |  |  |  |  |
| 674 | 6 mm thick |  |  |  |  |  | 1 Sqm | 430.00 |
| 675 | 8 mm thick |  |  |  |  |  | 1 Sqm | 462.00 |
| 676 | 10 mm thick |  |  |  |  |  | 1 Sqm | 498.00 |
| 677 | 12 mm thick |  |  |  |  |  | 1 Sqm | 542.00 |
| 678 | 16 mm thick |  |  |  |  |  | 1 Sqm | 599.00 |
|  | BISON DESIGNER BOARD |  |  |  |  |  |  |  |
|  | Availed Designs 1.Lumber 2.Quadra 3.Murk |  |  |  |  |  |  |  |
|  | standard size 10'-0"x4'-0" |  |  |  |  |  |  |  |
| 679 | 6mm |  |  |  |  |  | 1 Sqm | 194.00 |
| 680 | 8mm |  |  |  |  |  | 1 Sqm | 226.00 |
| 681 | 10mm |  |  |  |  |  | 1 Sqm | 253.00 |
| 682 | 12mm |  |  |  |  |  | 1 Sqm | 285.00 |
| 683 | 16mm |  |  |  |  |  | 1 Sqm | 371.00 |
|  | DECOCEIL DESIGNER CEILING BOARD |  |  |  |  |  |  |  |
|  | patterns: 1.Tika 2.Forraplus 3.Chemerahplus |  |  |  |  |  |  |  |
|  | (sizes 4x4,4x2,2x2) |  |  |  |  |  |  |  |
| 684 | 3.2 mm |  |  |  |  |  | 1 Sqm | 160.00 |
|  | EVEREST WALL BOARD MULTI PURPOSE CEMENT BOARDS (HIGH PRESSURED STEAM CURED) |  |  |  |  |  |  |  |
| 685 | 4 mm thick |  |  |  |  |  | 1 Sqm. | 112.00 |
| 686 | 6 mm thick |  |  |  |  |  | 1 Sqm. | 170.00 |
| 687 | 8 mm thick |  |  |  |  |  | 1 Sqm. | 221.00 |
| 688 | 10 mm thick |  |  |  |  |  | 1 Sqm. | 247.00 |
| 689 | 12 mm thick |  |  |  |  |  | 1 Sqm. | 320.00 |
| 690 | 14 mm thick |  |  |  |  |  | 1 Sqm. | 320.00 |
| 691 | 16 mm thick |  |  |  |  |  | 1 Sqm. | 374.00 |
| 692 | 18 mm thick |  |  |  |  |  | 1 Sqm. | 400.00 |
| 693 | 20 mm thick |  |  |  |  |  | 1 Sqm. | 455.00 |
|  | EVEREST DESIGNER BOARD |  |  |  |  |  |  |  |
| 694 | 4 mm thick |  |  |  |  |  | 1 Sqm. | 187.00 |
| 695 | 6 mm thick |  |  |  |  |  | 1 Sqm. | 280.00 |
|  | MULTIPURPOSE CEMENT BOARDS FALSE CEILING WITH EXPOSED METAL E-GRID IS 14862 : 2000 |  |  |  |  |  |  |  |
|  | Supply and fixing of Everest Board (High Pressured Steam Cured) multipurpose fiber cement board manufactured as per IS 14862 : 2000, to be fixed in E-Grid, GI suspended ceiling grid system of size 600 mm x 600 mm with, main runner 39 x 24.7 mm, cross T 29 x 24.7 mm and wall angle 22.2 x 22.2 mm the grid is suspended from the truss with GI wire and necessary hardware |  |  |  |  |  |  |  |
| 696 | Using 6 mm thick E-board |  |  |  |  |  | 1 Sqm | 630.00 |
| 697 | Using 4 mm thick E-board |  |  |  |  |  | 1 Sqm | 550.00 |
| 698 | Using 4 mm thick E-board classic* |  |  |  |  |  | 1 Sqm | 651.00 |
| 699 | Using 6 mm thick E-board Classic* |  |  |  |  |  | 1 Sqm | 760.00 |
|  | EVEREST ACOSTYLE FALSE CEILING with MINERAL FIBRE TILES Exposed Metal E-Grid |  |  |  |  |  |  |  |
|  | Providing & Fixing false ceilings using E GRID pre painted GI sections of dimensions of 38x24mm as main ‘T” and 28 x 24mm as the cross runners of 4’ and 2’along with wall angles of dimensions 24x22mm. These sections are to be suspended using 4mm dia GI rods or GI Straps from tie level (suspended using fasteners from slab/purlin etc) to form a uniform grid of 600x600mm after ensuring line and level 15mm of dimensions 595x595mm Acoustic tiles of make ‘ACOSTYLE’ of M/S Everest Industries Limited (mineral fiber acoustic tiles) to be installed in the above said grid with tile edges to be regular finished |  |  |  |  |  |  |  |
| 700 | Rhythm 95 (15 mm Tegular Edge) |  |  |  |  |  | 1 Sqm | 650.00 |
| 701 | Symphony 90 (13 mm Tegular Edge) |  |  |  |  |  | 1 Sqm | 560.00 |
| 702 | Symphony 90 (13 mm Square Edge) |  |  |  |  |  | 1 Sqm | 560.00 |
| 703 | Harmony 99 (15 mm Tegular Edge) |  |  |  |  |  | 1 Sqm | 685.00 |
|  | WALL PANELLING |  |  |  |  |  |  |  |
|  | Supply and fixing of E-board wall paneling (water, fire and Termite proof) with frame work made of GI stud profile of size 48 x 40 10 x 0.6 mm thick placed at every 610 mm C/C intervals fixed vertically in between floor and roof `U’ track of size 50 x 30 x 0.6 mm thick duly fixed with self expansion screws and with 8 mm thick E-board / 6 mm thick E-board classic cladded around with self tapping self embedding screws fixed at every 300 mm (centers) leaving a 3 mm gap between two panels of width 2’-0”. The Gap will be covered with a decorative beading. Surface will be given a smooth finish over a coat of primer. |  |  |  |  |  |  |  |
| 704 | With 8 mm E-board |  |  |  |  |  | 1 Sqm. | 837.00 |
| 705 | With 6 mm E-board Designer |  |  |  |  |  | 1 Sqm. | 903.00 |
|  | DOUBLE SKIN PARTITION |  |  |  |  |  |  |  |
| 706 | Supply and fixing double skin E-board partition with Frame work made of GI stud profile of size 48 x 40 x 10 x 0.6 mm thick placed at every 610 mm C/C intervals and 915 mm horizontal intervals duly fixed with self expansion screws, the E-board of 8 mm thick cladded around with self tapping self embedding screws fixed at every 300 mm (centers) leaving 3 mm gap between two panels of width 2’-0” (610 mm). The gap will be covered with decorative moulding surface will be given a smooth finish over cement primer. The rate is inclusive of cost and conveyance of all materials and labour charges etc. complete. |  |  |  |  |  | 1 Sqm. | 1287.00 |
|  | EVEREST SOLID WALL PANELS |  |  |  |  |  |  |  |
| 707 | Providing and fixing of 50mm thick EVEREST SOLID WALL PANEL Sandwich panels made of light Weight FRACC two 4mm thick non-asbestos fiber reinforced cement sheets on either side of the lightweight concrete core composed of Portland cement binders with tongue and groove arrangement. The bottom of the panel shall be fixed to the floor channel made up of 1 mm thick GI channels of size 14x50x25 mm fixed to the existing floor with 6x60mm nylon self expansion screws at 600 mm C/C and top of the panel fixed with ceiling channel made out of 1mm thick GI Channel of size 38x50x38mm duly fixed to the existing ceiling with 6x60mm nylon self expansion screws at 600 mm C/C. All the tongue and groove joints of the panels have to be properly cleaned and both the panels have to be fixed by applying the mixture of fly ash and sodium silicate in between and Everest jointing compound all the exposed joints. The rate includes cost of 50mm EVEREST SOLID WALL PANELS, Gl Floor and ceiling channel. The panels can be flushed by using paper tape and Everest jointing compound all complete |  |  |  |  |  | 1 Sqm | 1345.00 |
| 708 | Providing and fixing of 75mm thick EVEREST SOLID WALL PANEL Sandwich panels made of light Weight FRACC two 4mm thick non-asbestos fiber reinforced cement sheets on either side of the lightweight concrete core composed of Portland cement binders with tongue and groove arrangement. The bottom of the panel shall be fixed to the floor channel made up of 1 mm thick GI channels of size 14x75x25 mm fixed to the existing floor with 6x60mm nylon self expansion screws at 600 mm C/C and top of the panel fixed with ceiling channel made out of 1mm thick GI Channel of size 38x75x38mm duly fixed to the existing ceiling with 6x60mm nylon self expansion screws at 600 mm C/C. All the tongue and groove joints of the panels have to be properly cleaned and both the panels have to be fixed by applying the mixture of fly ash and sodium silicate in between and Everest jointing compound all the exposed joints. The rate includes cost of 75mm EVEREST SOLID WALL PANEL panels, Gl Floor and ceiling channel. The panels can be flushed by using paper tape and Everest jointing compound all complete. |  |  |  |  |  | 1 Sqm. | 1722.00 |
|  | DOOR |  |  |  |  |  |  |  |
| 709 | Supply and fixing of E-board door shutter with all-round style “U” lipping size 15 x 18 x 15 mm made from 0.8 mm thick powder coated cold rolled steel profiles with 16 mm thick E-board with necessary screws fixed at 1’-0” interval 250 mm long aldrops 2 Nos. M.S. nickel coated 250 mm long 1 No. M.S. Tower bolts including cost and conveyance of all materials at site and applying 1 coat of primer, 2 coats of luppum and 2 coats of synthetic enamel paint as per approved design for finished item of work. |  |  |  |  |  | 1 Sqm. | 1102.00 |
|  | DOOR WITH CRCA FRAME |  |  |  |  |  |  |  |
| 710 | Providing and fixing frame made from 1.2 mm powder coated cold rolled steel profiles of overall section size of 60 mm x 45 mm with 21 mm rivet for shutter with necessary stiffeners hinges and screws. E-board shutter with all-round `U’ lipping of size 15 X 18 X 15 mm made from 0.8 mm thick CRCA powder coated steel profile with 16 mm thick E-board with necessary screws fixed at every 1’-0” interval 250 mm long aldrops 2 Nos. MS Nickel coated 250 mm long 1 No. MS tower bolt including cost and conveyance. One coat of primer, 2 coats of luppum and 2 coats of synthetic enamel paint as per approved colour, labour charges etc. |  |  |  |  |  | 1 Sqm. | 1680.00 |
|  | Cup Boards Shutters using Cement Bonded Particle Board as per IS 14276 (Bison Panel or Equivalent ) |  |  |  |  |  |  |  |
| 711 | Supply and fixing of cupboard shutters and frames using 16mm thick Cement Bonded Particle Board (Bison Panel or Equivalent) duly painted both sides with one coat of primer and two coats of synthetic / enamel paint of approved shade and brand including fixing of hardware like M.S. powder coated Piano hinges of size 18mm x 18mm, brass ball catches, Aluminium tower bolts and handles , Gore locks, Fevicol, screws and nails including cost and conveyance of all materials, labour charges, all incidental and operational charges etc., complete for finished item of work at site as per the directions of Engineer in Charge. |  |  |  |  |  | 1 Sqm | 1493.00 |
|  | False Ceiling using Cement Bonded Particle Board as per IS 14276 (Bison Panel or Equivalent ) |  |  |  |  |  |  |  |
| 712 | Supply and fixing of false ceiling with galvanized and pre painted steel T section of size 24mm x 38mm x 0.4mm for main T duty pre punched to accept cross T section of size 24mm x 30mm x 0.4mm duly punched at both ends for insertion into main T. The grid size will be 610mm x 610mm. The frame work (grid) is suspended to the roof by using G.I flat of 0.60mm or 14-gauge G.I wire with necessary fixers to the roof and frame work. 8mm thick Cement Bonded Particle Board lay on the grid. The cost inclusive of necessary hardware, labour, one coat of primer (Both side) and two coats of smooth finish on visible side. |  |  |  |  |  | 1Sqm | 738.00 |
|  | WALL PANELING WITH CEMENT BONDED PARTICLE BOARD as per IS 14276 (BISON PANEL OR EQUIVALENT) |  |  |  |  |  |  |  |
| 713 | Supply and fixing bison panel wall paneling (water proof) with frame work made of GI stud section of size 48mm x 35mm x 0.55mm thick placed at every 610 mm C/C intervals vertically and at 1200mm internals horizontally fixed to the wall by means of self expansion screws and caps. 10 mm thick cement bonded particle board (Bison panel or Equivalent) is to be fixed to frame by means of self taping screws placed at every 300 mm intervals leaving 3mm gap between two panels. The cost inclusive of one coat of Altek primer and two coats of Altek smooth finish and cost & conveyance of all materials, labour charges etc. complete as per the direction of Engineer in charge. |  |  |  |  |  | 1 Sqm | 1182.00 |
|  | DOUBLE SKIN PARTITION WITH CEMENT BONDED PARTICLE BOARD as per IS 14276 (BISON PANEL OR EQUIVALENT) |  |  |  |  |  |  |  |
| 714 | Supply and fixing double skin partition made out of G.I. track section of size 50mmx 35mm x 0.55 mm for fixing to the roof and floor and stud section of size 48mm x 35mmx 0.55mm placed in track section vertically at 610mm intervals and at 1200mm intervals horizontally fixed to the wall by means of self expansion screws and caps to the wall and roof. 10mm thick Cement bonded particle board (BISON PANEL OR EQUIVALENT) is to be fixed both sides of frame work by using self taping screws fixed at every 300mm intervals to the frame work leaving 3 mm gap between two panels. The cost inclusive with one coat of Altek primer and two coats of Altek smooth finish and cost and conveyance of all materials to site, labour charges etc., complete as per the direction of Engineer in charge. |  |  |  |  |  | 1 Sqm | 1894.00 |
|  | SINGLE SHUTTER WITH CEMENT BONDED PARTICLE BOARD as per IS 14276 (BISON PANEL OR EQUIVALENT) |  |  |  |  |  |  |  |
| 715 | Supply and fixing cement bonded particle board shutter with all-round "U" section of size 12mmx18mm x12mm made from 0.6 mm thick galvanized, color quoted cold rolled steel profiles 16 mm thick panel with necessary screws fixed at every 304.8 mm intervals. The cost inclusive of MS Nickel coated Aldrops of size 255 mm long 2 Nos., 200 mm long tower bolts 1 No., 125 mm handles 2 Nos. 300 mm long "T" hinge cost and conveyance of all materials at site and applying 1 coat primer, 1 coat of luppum and 2 coats of synthetic enamel paint as per approved design for finished item of work. |  |  |  |  |  | 1 Sqm | 1398.00 |
|  | Doors Frames (Colour Coated Steel Sections) & Shutter using Cement Bonded Particle Board as per IS 14276 (Bison Panel or Equivalent) |  |  |  |  |  |  |  |
| 716 | Providing and fixing of door frame made from cold roll formed sections made of galvanized (Base steel as per IS277 with zinc of 120 Gms/Sqm) colour coated steel sections with total coated thickness of 1mm, primer coated with epoxy primer coat of 5-7 microns thick, finish paint with polyester paint of 12-16 microns. The Section for door frame is to be filled with polyurethane (P.U) form of 40kg/m3. Overall size of frame section should be of 50mm x 50mm. With necessary corner brackets, stiffeners, hinges, and screws. Cement Bonded particle board shutter (Bison Panel or Equivalent) “Single Leaf” with all round styles “U” section of size 12m x 18m x 12mm made from 0.6 mm thick galvanized, colour coated cold rolled steel profiles with 16mm thick Cement Bonded particle Board (Bison Panel or Equivalent) with necessary screws fixed at every 1’-0” (304.8mm) intervals. The cost inclusive of MS nickel coated Aldrop of size 250mm long 2nos. 200mm long tower bolt 1no., 125mm handles 2nos., cost and conveyance of all materials at site and applying one |  |  |  |  |  | 1 Sqm | 2225.00 |
|  | coat primer , one coat of luppum and two coats of synthetic enamel paint as per approved design for finished item of work |  |  |  |  |  |  |  |
|  | Box Type Cup Board using Cement Bonded Particle Board (Bison Panel or Equivalent) & Shutter using Pre-Laminated Cement Bonded Particle Board as per IS 14276 (Bison Lam or Equivalent) |  |  |  |  |  |  |  |
| 717 | Supply and fixing of box type cupboard using 20mm thick Cement Bonded Particle Board (Bison Panel or Equivalent) for making frame of size 100mm x 35mm using two sandwiched 16mm boards, one plain board and one Pre-Laminated cement bonded particle board with plain 35mm wood lipping. The Shutters are made of 16m thick Pre-Laminated Cement Bonded Particle Board (Bison Lam or Equivalent) of approved shade inclusive of hardware like M.S. powder coated Piano hinges of size ¾” x ¾” , Aluminium powder coated handles of size 4” (101.6mm), Aluminium tower bolt 4” (101.6mm) Gore locks, screws, wood lipping and Fevicol etc., complete finish item of work. |  |  |  |  |  | 1 Sqm | 3178.00 |
|  | Door Shutter with Cement Bonded Particle Board (Bison Lam or Equivalent) |  |  |  |  |  |  |  |
| 718 | Supply and fixing of door shutter made out of PVC Hollow extruded section having dimensions of 47mm x 26mm with wall thickness of 1.5mm with necessary tapering to house Bison Lam thickness of 12mm. All the four corners are jointed with PVC ‘L’ brackets of size 4’’ x 8’’ (101.6 x 203.2mm) and the PVC lipping is filled with well-seasoned country wood to hold hinges. The hardware consists of 250mm long Aldrop – 2 Nos.250mm Handle- 1 No. (all are of Aluminium). The cost is inclusive of all incidental, labour transportation charges. |  |  |  |  |  | 1 Sqm | 1459.00 |
|  | Double Skin Partition with Pre Laminated Cement Bonded Particle Board (Bison Lam or Equivalent) |  |  |  |  |  |  |  |
| 719 | Supply and fixing double skin partition made out of G.I track section of size 50 mm x 35mm x 0.55mm for fixing to the roof and floor and stud section of size 48mm x 35mm x 0.55mm placed in track section vertically at 610mm intervals and at 1200mm internals horizontally. The frame is fixed by means of self expansion screws and caps to the wall / roof. 12mm thick Pre Laminated Cement bonded particle board (Bison Lam or equivalent) is to be fixed both sides of the frame work by using 2mm thick electro plated CR flat section leaving 3mm gap between two panels. The cost inclusive of cost and conveyance of all materials to sites, other materials incidentals and labour charges as per the direction of Engineer in charges. |  |  |  |  |  | 1 Sqm | 1930.00 |
|  | Wall Paneling with Pre Laminated Cement Bonded Particle Board (Bison Lam or equivalent) |  |  |  |  |  |  |  |
| 720 | Supply and fixing of wall paneling with frame work made of G.I stud section of size 48mm x 35mm x 0.55mm thick placed at every 610mm intervals vertically and at 1200mm internals horizontally fixed to the wall by means of self expansion screws and caps. 12mm thick pre laminated Cement bonded particle board (Bison Lam or equivalent) is to be fixed to frame by means of self-taping screws placed at every 300mm intervals leaving 3mm gap between two panels. The cost inclusive of Conveyance of materials, Labour charges etc., complete as per the direction of Engineer in charge. |  |  |  |  |  | 1 Sqm | 1555.00 |
|  | Aluminium Partition with Pre Laminated Cement Bonded Particle Board (Bison Lam or Equivalent) |  |  |  |  |  |  |  |
| 721 | Supply and fixing in position aluminium glazed partitions using 5.50 mm thick plain glass to full height using with pre laminated cement bonded particle board (Bison Lam or equivalent) of 10mm thick to a height of 0.91 meter at bottom panel and remaining height with glass and aluminium sections anodized to 12 to 15 microns and of sections of size 37mm x 62mm and 1.5mm thickness with one meter centre to centre duly fixed with clip beading on both sides including fixing the frame to pillars by M.S. flats, bolts and nuts including cost and conveyance of all materials etc., complete as directed during execution. |  |  |  |  |  | 1 Sqm | 2387.00 |
|  | Specification for Bison lam windows with Pre-Painted Steel Sections |  |  |  |  |  |  |  |
|  | Supply and fixing of factory made windows fabricated from pre-painted Cold Rolled formed sections made of galvanized steel colour coated sections (Base steel as per IS-512 D quality, galvanized as per IS-277 with zinc of 120 grams/Sqm) with total coated thickness of 0.6mm primer coated with epoxy primer of 5-7 microns thick, finish painted with polyester paint of 12-16 microns and back coated with alkyd backer of 5-7 microns), the section for outer frame and centre mullion (duly filled with PU form of 40 Kg/Cum) should be of 60mm X 50mm, with provision for fixing glazed shutter. The section for glazed/ paneled shutter should be of size 47mm X 20mm and another section of size 60mm X 23mm should be provided for separating glazed and paneled portion. The sections are cut to length and miter jointed with plastic corner brackets. The size of the corner brackets for a window outer frames 50mm X 60mm. The mullion section is to be fixed to the frame with plastic mullion cap |  |  |  |  |  |  |  |
|  | 1. Stiffener plates of size 60mm X 50mm made of 1.25mm thick CRCA electroplated are to be provided to fix window frames to concrete/masonry. Gasket is to be provided for fixing glass. 10mm thick pre laminated (Bison lam or equivalent) cement bonded particle board both sides. 5mm thick colour pin headed glass is to be provided for glazing and providing guard bars of 10mm square rods duly welded at 100mm pitch to MS flat section of size 18mm and 3mm thick. Windows are to be fixed to wall by means of self expansion nylon cap(75) and electroplated wooden screws of size 100mm, together with the following (Supply and fixing) Aluminium powder coated fittings. |  |  |  |  |  |  |  |
|  | 1) 75mm long grip handles 1 no per shutter. |  |  |  |  |  |  |  |
|  | 2) 75mm long tower bolts 2 nos. per shutter. |  |  |  |  |  |  |  |
|  | 3) Heavy duty SS pivot hinges 2 nos. per shutter. |  |  |  |  |  |  |  |
|  | 4) MS powder coated telescopic stay 1 no per shutter. |  |  |  |  |  |  |  |
|  | BISON LAM WINDOWS WITH PPS SECTIONS |  |  |  |  |  |  |  |
| 722 | Single Shutter 2’0” X 4’0” outer frame section size of 50mm X 60mm shutter frame section size of 47mm X 20mm and 60mm X 23mm section provided for shutter frame mullion |  |  |  |  |  | 1 Sqm | 3456.00 |
| 723 | Double shutter 3’0” X 4’0” outer frame section size of 50mm X 60mm shutter frame section size of 47mm X 20mm, Mullion section size of 50mm X 60mm for outer frame, and 60mm X 23mm section provided for shutter frame mullion |  |  |  |  |  | 1 Sqm | 3489.00 |
| 724 | Double shutter 3’0” X 3’0” outer frame section size of 50mm X 60mm shutter frame section size of 47mm X 20mm, Mullion section size of 50mm X 60mm for outer frame, and 60mm X 23mm section provided for shutter frame mullion |  |  |  |  |  | 1 Sqm | 3842.00 |
| 725 | Double shutter 4’0” X 4’0” outer frame section size of 50mm X 60mm shutter frame section size of 47mm X 20mm, Mullion section size of 50mm X 60mm for outer frame, and 60mm X 23mm section provided for shutter frame mullion |  |  |  |  |  | 1 Sqm | 3456.00 |
| 726 | Centre fixed both side openable shutters 6’0” X 4’0” outer frame section size of 50mm X 60mm shutter frame section size of 47mm X 20mm, Mullion section size of 50mm X 60mm fixed beading section size of 18mm X 25mm, and 60mm X 23mm section provided for shutter frame mullion. |  |  |  |  |  | 1 Sqm | 3103.00 |
|  | DOUBLE DOOR SHUTTER WITH CEMENT BONDED PARTICLE BOARD as per IS 14276 |  |  |  |  |  |  |  |
|  | (BISON PANEL OR EQUIVALENT) |  |  |  |  |  |  |  |
| 727 | Supply and fixing of BISON PANEL door shutter with all round style “U” size of 20mm X 18mm X 20mm made from 0.6mm thick galvanized pre-painted cold rolled steel profiles with 16mm thick bison panel (cement bonded particle board) separating two shutters with 30mm X 18mm X 30mm Stepped lipping with necessary screws fixed at every 1’-0” (304.8mm) intervals. MS Nickel coated 250mm long aldrops 2 no’s, MS powder coated 250mm long tower bolt 2 no’s, & 2no’s MS powder coated 125mm long grip handle. Including cost and conveyance of all materials at site and applying one coat of primer, 2 coats of luppum and 2 coats of synthetic enamel paint as per approved design for finished item of work |  |  |  |  |  | 1 Sqm | 1553.00 |
|  | SINGLE SHUTTER WITH BISON LAM CEMENT BONDED PRE LAMINATED PARTICLE BOARD as per IS 14276 |  |  |  |  |  |  |  |
|  | (BISON OR EQIVALENT) |  |  |  |  |  |  |  |
| 728 | Supply and fixing of BISON LAM cement bonded pre-laminated particle board single door shutter with all round section “U” size of 20mm X 18mm X 20mm made from 0.6mm thick galvanized pre-painted cold rolled steel profiles with 16mm thick bison lam (pre-laminated cement bonded particle board) with necessary screws fixed at every 1’-0” (304.8mm) intervals. The cost inclusive of MS Nickel coated 250mm long aldrops 2 no’s, MS powder coated250mm long tower bolt 1 no’s, 2no’s MS powdercoated125mm long grip handle, 1 no’s M.S.P.C. Door stopper. Including cost and conveyance of all materials at site labour charges etc., complete finished item of work |  |  |  |  |  | 1 Sqm | 1660.00 |
|  | DOUBLE SHUTTER WITH BISON LAM CEMENT BONDED PRE LAMINATED PARTICLE BOARD as per IS 14276 |  |  |  |  |  |  |  |
|  | (BISON OR EQUIVALENT) |  |  |  |  |  |  |  |
| 729 | Supply and fixing of BISON LAM cement bonded pre-laminated particle board single door shutter with all round”U” section size of 20mm X 18mm X 20mm made from 0.6mm thick galvanized pre-painted cold rolled steel profiles with 16mm thick bison lam (pre-laminated cement bonded particle board) separating two shutter with 30mm X 20mm X 30mm stepped lipping with necessary screws fixed at every 1’-0” (304.8mm) intervals. The cost inclusive of MS Nickel coated 250mm long aldrops 2 no’s, MS powder coated 250mm long tower bolt 1 no’s, 2no’s MS powder coated 125mm long grip handle, 2 no’s M.S.P.C. Door stopper. Including cost and conveyance of all materials at site labour charges etc., complete finished item of work |  |  |  |  |  | 1 Sqm | 1778.00 |
|  | Inssula or Equivalent Mineral Fiber Ceiling Tiles : |  |  |  |  |  |  |  |
|  | Inssula Ceiling Tile of 0.595 mm x 0.595 mm of 13 mm thick Square Edge |  |  |  |  |  |  |  |
| 730 | Fissura Fine Model |  |  |  |  |  | 1 Sqm | 329.00 |
| 731 | Speckle Model |  |  |  |  |  | 1 Sqm | 367.00 |
| 732 | Textura Model |  |  |  |  |  | 1 Sqm | 405.00 |
|  | Inssula Ceiling Tile of 0.595 mm x 0.595 mm of 15 mm thick Standard Regular Edge |  |  |  |  |  |  |  |
| 733 | Fissura Fine Model |  |  |  |  |  | 1 Sqm | 424.00 |
| 734 | Speckle Model |  |  |  |  |  | 1 Sqm | 443.00 |
| 735 | Textura Model |  |  |  |  |  | 1 Sqm | 481.00 |
|  | Inssula or Equivalent Grid Suspended Ceiling System |  |  |  |  |  |  |  |
|  | Providing & fixing Pre-coated G.I. Wall angle size 19 mm x 19 mm x 0.30 mm thick of 3600 mm long fixed to the brick wall/partition at the perimeter of ceiling with nylon sleeves and screws at 610 mm centers. Then suspending main G.I. Frame "T" section size of 38 mm x 24mm x 0.30mm thick of 3600mm long from soffit in one direction with the help of Rawl Plug and G.I. wire/Rod of 4 mm diameter with 1220 mm centers G.I. Cross "T" section of size 25mm x 24mm x 0.30mm thick of 600mm long is inserted into the slots provided in main "T" at 600mm centers to form a grid of 600mm x 600mm. Inssula or equivalent 13/15mm tiles of size 0.595 mm x 0.595 mm can be laid on to the grid etc. as per specification and as directed by department Officers. |  |  |  |  |  |  |  |
|  | Using 13 mm thick Square Edge laid on to the Grid |  |  |  |  |  |  |  |
| 736 | Fissura Fine Model |  |  |  |  |  | 1 Sqm | 696.00 |
| 737 | Speckle Model |  |  |  |  |  | 1 Sqm | 741.00 |
| 738 | Textura Model |  |  |  |  |  | 1 Sqm | 784.00 |
|  | Using 15 mm thick Standard Regular Edge laid on to the Grid |  |  |  |  |  |  |  |
| 739 | Fissura Fine Model |  |  |  |  |  | 1 Sqm | 886.00 |
| 740 | Speckle Model |  |  |  |  |  | 1 Sqm | 949.00 |
| 741 | Textura Model |  |  |  |  |  | 1 Sqm | 1012.00 |
|  | V- BOARD MULTI PURPOSE CEMENT BOARDS – (HIGH PRESSURED STEAM CURED) |  |  |  |  |  |  |  |
| 742 | 4mm Thick |  |  |  |  |  | 1 Sqm | 111.00 |
| 743. | 6mm Thick |  |  |  |  |  | 1 Sqm | 160.00 |
| 744 | 8mm Thick |  |  |  |  |  | 1 Sqm | 205.00 |
| 745 | 10mm Thick |  |  |  |  |  | 1 Sqm | 237.00 |
| 746 | 12mm Thick |  |  |  |  |  | 1 Sqm | 295.00 |
| 747 | 14mm Thick |  |  |  |  |  | 1 Sqm | 330.00 |
| 748 | 16mm Thick |  |  |  |  |  | 1 Sqm | 376.00 |
| 749 | 18mm Thick |  |  |  |  |  | 1 Sqm | 425.00 |
| 750 | 20mm Thick |  |  |  |  |  | 1 Sqm | 450.00 |
|  | V- BOARD MULTI PURPOSE CEMENT BOARDS ) FALSE CEILING WITH EXPOSED METAL GRID |  |  |  |  |  |  |  |
|  | Supply and fixing of V- Board ( High Pressured Steam Cured) multipurpose fiber cement board manufactured as per IS 14862: 2000, to be fixed in grid, GI Suspended ceiling grid system of size 600 mm x 600 mm with, main runner 39 x 24.7 mm, cross T 29 x 24.7 mm and wall angle 22.2 x 22.2 mm the grid is suspended from the truss with GI wire and necessary hardware. Inclusive of all taxes |  |  |  |  |  |  |  |
| 751 | Using 4 mm thick |  |  |  |  |  | 1 Sqm | 540.00 |
| 752 | Using 6 mm thick |  |  |  |  |  | 1 Sqm | 640.00 |
| 753 | Using 8 mm thick |  |  |  |  |  | 1 Sqm | 700.00 |
|  | WALL PANELING |  |  |  |  |  |  |  |
|  | Supply and fixing of V-board wall paneling (water, fire and termite resistant) with frame work made of GI stud profile of size 48 x 40 x 10 x 0.6 mm thick placed at every 610 mm C/C intervals fixed vertically in between floor and roof ‘U’ track of size 50 x 30 x 0.6 mm thick duly fixed with self expansion screws and with 8 mm thick V- Board / Cladded around with self tapping self embedding screws fixed at every 300 mm (centers) leaving a 3 mm gap between two panels of width 2’-0”. The Gap will be covered with a decorative beading. Surface will be given a smooth finish over a coat of primer. Inclusive of all taxes |  |  |  |  |  |  |  |
| 754 | With 8 mm V- Board |  |  |  |  |  | 1 Sqm | 860.00 |
|  | DOUBLE SKIN PARTITION |  |  |  |  |  |  |  |
| 755 | Supply and fixing double skin V- Board partition with Frame work made of GI stud profile of size 48x40x 0x 0.6 mm thick placed at every 610 mm C/C intervals and 915 mm horizontal intervals duly fixed with self expansion screws, the V- Board of 8 mm thick cladded around with self tapping self embedding screws fixed at every 300 mm (centers) leaving a 3 mm gap between two panels of width 2’-0”(610 mm). The Gap will be covered with a decorative moulding surface will be given a smooth finish over cement primer. The rate Inclusive of cost and conveyance of all materials and labour charges etc., Complete. |  |  |  |  |  | 1 Sqm | 1300.00 |
|  | DOOR |  |  |  |  |  |  |  |
| 756 | Supply and fixing of V- Board door shutter with all-round style “U” lipping size 15 x 18 x 15 mm made from 0.8 mm thick powder coated cold rolled steel profiles with 16 mm thick V-board with necessary screws fixed at 1’-0” interval 250 mm long aldrops 2 nos. M.S. nickel coated 250 mm long 1 no. M.S. Tower Bolts Including cost and conveyance of all materials at site and applying 1 coat of primer, 2 coats of luppum and 2 coats of synthetic enamel paint as per approved design for finished item of work. |  |  |  |  |  | 1 Sqm | 1400.00 |
|  | DOOR WITH CRCA FRAME |  |  |  |  |  |  |  |
| 757 | Providing and fixing frame made from 1.2 mm powder coated cold rolled steel profiles of overall section size of 60 mm x 45 mm with 21 mm rivet for shutter with necessary stiffeners hinges and screws. V- Board shutter with all-round style “U” lipping of size 15 x 18 x 15 mm made from 0.8 mm thick CRCA powder coated steel profile with 16 mm thick V-board with necessary screws fixed at every 1’-0” interval 250 mm long aldrops 2 nos. M.S. nickel coated 250 mm long 1 no. M.S. Tower Bolts Including cost and conveyance. One coat of primer, 2 coats of luppum and 2 coats of synthetic enamel paint as per approved colour, labour charges etc.,. |  |  |  |  |  | 1 Sqm | 1650.00 |
|  | SINTEX OR EQUIVALENT DOOR (SINGLE PANEL) |  |  |  |  |  |  |  |
| 758 | Supply and fixing Sintex or equivalent doors are made out of PVC section and panels. The overall dimension of the same is 33mm X 47mm with usual process variation and having a wall thickness of 1.5mm with a variation of ± 0.3mm. The infill is seamless hallow multi chambered PVC single panel having an effective dimension of 762mm X 20mm having a wall thickness of 1mm ± 0.3mm complete, for finished item of work. |  |  |  |  |  | Sqm. | 2018.00 |
| 759 | Supply and fixing PVC door frames made from extruded sections in overall dimensions of 40 X 48mm having a wall thickness of 1.5mm with usual process variation of ± 0.3mm, complete, for finished item of work. |  |  |  |  |  | 1 RM. | 189.00 |
|  | SINTEX OR EQUIVALENT OPENABLE WINDOW |  |  |  |  |  |  |  |
| 760 | Supply and fixing of Sintex window shall be made out of extruded PVC section. The outer frame having overall dimensions of 40 X 48mm with a wall thickness of 1.5mm with usual process variation of ± 0.3mm. The shutter frame having with overall dimensions of 26 X 47mm with wall thickness of 1.2mm ± 0.3mm provided with 5mm glazing. The centre mullion having over all dimensions of 25 X 32.5mm ±0.3mm, complete, for finished item of work. |  |  |  |  |  | 1 Sqm. | 3008.00 |
|  | SINTEX OR EQUIVALENT SLIDING 2 TRACK WINDOWS |  |  |  |  |  |  |  |
| 761 | Supply and fixing of two track or three track sliding window the outer frame shall be 58 X 45mm with a wall thickness of 1.2mm ± 0.3mm. The sliding shutter frame is made of PVC section 57 X 32mm with overall dimensions of 1.5mm ± 0.3mm provided with 5mm glazing. Roller bearings and arrestor type handle cum lock shall be provided, complete, for finished item of work. |  |  |  |  |  | 1 Sqm. | 2680.00 |
|  | SINTEX OR EQUIVALENT FALSE CEILING |  |  |  |  |  |  |  |
| 762 | Supply and fixing of PVC false ceiling on MS/GI made skeleton framework. The frame shall be 24 X 48mm MS tube section fixed along with the wall. The entire frame work shall be concealed with PVC section having overall dimensions of 6 X 250mm with a wall thickness of 1mm ± 0.3mm with tongue and groove system, provided with necessary arrangements for electrical connections, complete, for finished item of work. |  |  |  |  |  | 1 Sqm. | 1335.00 |
|  | SINTEX OR EQUIVALENT WALL PANELLING |  |  |  |  |  |  |  |
| 763 | Supply and fixing of wall paneling on the framework of wooded rippers of 24 X 24mm of required size. The entire framework shall be paneled with PVC section having overall dimensions of 6 X 250mm with a wall thickness of 1mm ± 0.3mm. The panels are joined with tongue and groove system. Necessary cutouts for electrical connections to be provided at required places complete, for finished item of work. |  |  |  |  |  | 1 Sqm. | 1273.00 |
|  | SINTEX OR EQUIVALENT PVC CABINETS |  |  |  |  |  |  |  |
| 764 | Supply and fixing of PVC cupboards for kitchen and bedrooms shall be made out of multi chambered PVC sections having overall dimensions of 20 X 762mm with a wall thickness of 1.2mm ± 0.3mm. The cabinets shall be made as per the requirements of the room with groove system. The PVC section surface shall be plain and laminated finish to have good look, completed, for finished item of work. |  |  |  |  |  | 1 Sqm. | 4050.00 |
|  | SINTEX OR EQUIVALENT SMC SIGN BOARDS |  |  |  |  |  |  |  |
|  | Supply of hot pressed compression moulded SMC sign plates for making road signs as per the approved dimensions of National Highway Authority Of India, complete, for finished item of work. |  |  |  |  |  |  |  |
| 765 | Circular sign plate Dia 400mm. |  |  |  |  |  | Each. | 506.00 |
| 766 | Circular sign plate Dia 600mm. |  |  |  |  |  | Each. | 1330.00 |
| 767 | Circular sing plate Dia 870mm |  |  |  |  |  | Each. | 2665.00 |
| 768 | Triangular sign plate 600mm. |  |  |  |  |  | Each. | 683.00 |
| 769 | Triangular sign plate 900mm. |  |  |  |  |  | Each. | 1542.00 |
| 770 | Square sign plate 600mm. |  |  |  |  |  | Each. | 1236.00 |
| 771 | Square sign plate 1000mm. |  |  |  |  |  | Each. | 4591.00 |
|  | SINTEX OR EQUIVALENT PVC PARTITION |  |  |  |  |  |  |  |
| 772 | Supply and fixing of PVC Double Skin Partition. The entire framework shall be carried out by mild steel section having overall dimensions of 24 X 48mm with a wall thickness of 1.2mm. The entire framework shall be concealed with PVC section having overall dimensions of 6 X 250mm with a wall thickness of 1mm ± 0.3mm both sides. Glass provision/Door provision can be provided along the partition wherever required. Partitions can be made full or half as per requirement complete, for finished item of work. |  |  |  |  |  | Sqm. | 2219.00 |
|  | SINTEX OR EQUIVALENT WATER STORATGE TANKS |  |  |  |  |  |  |  |
| 773 | Supply of Sintex or equivalent Polyethylene water storage tank double layer confirming to ISI 12701/96 mark with lid. |  |  |  |  |  | Ltr. | 7.00 |
|  | SINTEX FRONTURA DOOR SHUTTER (25MM THICK) |  |  |  |  |  |  |  |
| 774 | Supply and fixing of Sintex Frontura door shutter consisting of Sintex make or equivalent FRP (Fiber Reinforced Plastic) panels on either sides with depressed paneled design for door having overall wall thickness of 1.5mm + 0.35 mm with usual process variation. The core of the door shutter shall be filled with Polyurethane foam (PUF) injected with the help of Hydraulic Injection method, wooden black of 35mm x 31mm shall be provided on all four side of the door shutter. Necessary wooden blacks shall be provided to take up necessary hardwares and fixtures etc. with steel wire mesh at the center of the door the rate inclusive of all taxes and finished item work. Max size: 838mm (W) x 2060mm(H) Colour GOLDEN BROWN. |  |  |  |  |  | 1 Sqm. | 3120.00 |
|  | SINTEX FRONTURA DOOR SHUTTER (35MM THICK) |  |  |  |  |  |  |  |
| 775 | Supply and fixing of Sintex Frontura door shutter consisting of Sintex make or equivalent FRP (Fiber Reinforced Plastic) panels on either sides with depressed paneled design for door having overall wall thickness of 1.5mm + 0.35 mm with usual process variation. The core of the door shutter shall be filled with Polyurethane foam (PUF) injected with the help of Hydraulic Injection method, wooden black of 35mm x 31mm shall be provided on all four side of the door shutter. Necessary wooden blacks shall be provided to take up necessary hardwares and fixtures etc. with steel wire mesh at the center of the door the rate inclusive of all taxes and finished item work Max size: 838mm (W) x 2060 (H)Colour Golden Brown |  |  |  |  |  | 1 Sqm. | 3400.00 |
|  | SINTEX ENDURA DOOR SHUTTER (26MM THICK) |  |  |  |  |  |  |  |
| 776 | Supply and fixing of Sintex Door shutter made from specially Hot Press Moulded SMC ( Sheet moulding compound) sheets, Polyurethane core and PVC Frame section SMC sheets shall be made from SMC conforming to IS-13410 sheet shall have an average thickness of 1.5mm + 0.35mm.Core of the door shutter shall be filled with high density polyurethane foam, injected with the help of hydraulic injection method shutter frame to be made of PVC extruded section confirming to IS:10151:1982 of size 26mm x 47mm with a wall thickness of 1.5mm + 0.3mm corresponding to Sintex code DWUF 305. The hardware locations shall be reinforced with special polymeric reinforcement wooden blocks of suitable size. The corners of the shutter frame shall be reinforced with Polymeric ‘L’ shaped angle of size 105mm x 230mm corresponding to Sintex code PR-02 the lock rail shall be made from PVC extruded sections having overall dimensions of 26mm x 47mm 1.5mm + 0.3mm, with usual process variation corresponding to Sintex code DWUF 305 Combining two numbers. Wooden pieces are inserted on the periphery. Suitable MS Tube reinforcement shall be provided on vertical sides of shutter frame the rate inclusive of all taxes and finished item work Colour: White & Grey |  |  |  |  |  | 1 Sqm. | 2600.00 |
|  | PVC DOOR SHUTTER (Nandi or Equivalent) |  |  |  |  |  |  |  |
| 777 | Supply and fixing of door shutters made of rigid PVC extruded hollow section(Nandi or equivalent) of 20mm X 200 mm with the wall thickness of 1.0mm +/- 0.1mm equally divided into 4 no's with tongue and groove locking arrangements. The shutter frame is made of 58 X 24 mm with the wall thickness of 1.5mm +/- 0.15mm section mitre-cut and joined at 4 corners with 125mm X 225mm plastic brackets. The shutter shall be horizontally reinforced with 2 no's of 8mm PVC rods. Teak wood batons shall be reinforced inside the door shutter during the fabrication of the door shutter at those points wherever the hardware is fixed on to the door shutter. |  |  |  |  |  | 1 Sqm. | 1427.00 |
| 778 | Supply and fixing of door shutters made of rigid PVC extruded hollow section(Nandi or equivalent) of 20mm X 200 mm with the wall thickness of 1.0mm +/- 0.1mm equally divided into 4 no's with tongue and groove locking arrangements. The shutter frame is made of 30mm X 79mm with the wall thickness of 1.5mm +/-0.15mm section mitre-cut and joined at 4 corners with 125mm X 225mm plastic brackets. The shutter shall be horizontally reinforced with 2 no's of 8mm PVC rods. Teak wood batons shall be reinforced inside the door shutter during the fabrication of the door shutter at those points wherever the hardware is fixed onto the door shutter. |  |  |  |  |  | Sqm. | 1631.00 |
|  | PVC DOOR FRAME (Nandi or Equivalent) |  |  |  |  |  |  |  |
| 779 | Supply and fixing of door frame made of hollow extruded PVC section (Nandi or equivalent) having dimensions of 40 X 57mm with the wall thickness of 2mm +/- 0.2mm duly reinforced with seasoned wood plank at the hinges side. The door frame top 2 corners shall be metri-cut/ welded. |  |  |  |  |  | 1 RM | 195.00 |
| 780 | Supply and fixing of door frame made of hollow extruded PVC section (Nandi or equivalent) having dimensions of 40 X 46 mm with the wall thickness of 2mm +/- 0.2mm duly reinforced with seasoned wood plank at the hinges side. The door frame top 2 corners shall be mitre cut/welded |  |  |  |  |  | 1 RM | 170.00 |
|  | NCL or Equivalent PRE- PAINTED STEEL, WINDOWS, VENTILATORS & DOOR FRAMES. |  |  |  |  |  |  |  |
|  | Supply and fixing of windows & top Hung and fixed louvered ventilators made of pre - painted steel (base steel as per IS 513 of -0.6 mm thick 'D" quality, galvanized as per IS 277 with zinc of 120 GSM) primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be 46 x 52 mm section for shutter should be 46 x 46 mm section for mullion should be 46 x 70 mm, and section for beading should be 18 x 25 mm and section for louvered ventilation should be 33 x57mm Box section. The windows should be paneled with 5mm thick plain float glass & 4 mm pinhead glass for ventilators with Ethyl propylene Diamine monomer Gasket (EPDM). The sections are to be cut to length mitre joined with corner brocket centre mullions are to be fixed using mullion cap. Handle made of high grade aluminium powder coated and nylon receiver. Gaskets made of Ethyl propylene Diamine monomer(EPDM). |  |  |  |  |  |  |  |
|  | Corner brackets made of CRCA with Zinc Phosphate. Mullion caps made of glass filled nylon. The above frames should be fixed to the concrete /masonry wall by means of self expanding screws. including 10 mm square guard bars with 6" pitch complete for finished item of work. |  |  |  |  |  |  |  |
|  | Windows |  |  |  |  |  |  |  |
| 781 | Single shutter 2'0 x 4'0 (609.6 x 1219.2 mm ) Outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm |  |  |  |  |  | 1 Sqm | 4446.00 |
| 782 | Double shutter 3'-0" x 4'-0" (914.4mm x 1219.2 mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm |  |  |  |  |  | 1 Sqm | 4446.00 |
| 783 | Double shutter 3'-0" x 4'-0" (914.4mm x 1219.2 mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4712.00 |
| 784 | Double shutter 4'-0" x 4 '-0" (1219.2x12.19.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm |  |  |  |  |  | 1 Sqm | 4446.00 |
| 785 | Double shutter 4'-0" x 4 '-0" (1219.2x1219.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4712.00 |
| 786 | Double shutter 4'-6" x 4 '-6" (1371.6x1371.6mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4200.00 |
| 787 | Centre fixed both side openable shutter 5'-0" x 4'-0" (1524x1219.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm fixed beading section size of 18 x 25 mm |  |  |  |  |  | 1 Sqm | 4200.00 |
| 788 | Centre fixed both side openable shutter 6'-0" x 4'-0" (1828.8x1219.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm fixed beading section size of 18 x 25 mm |  |  |  |  |  | 1 Sqm | 4200.00 |
| 789 | Centre fixed both side openable shutter 6'-0" x 4'-6" (1828.8x1371.6mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm fixed beading section size of 18 x 25 mm |  |  |  |  |  | 1 Sqm | 4200.00 |
|  | Ventilators : |  |  |  |  |  |  |  |
| 790 | Top Hung 2 '-0" x 2'-0" (609.6x609.6mm) outer frame section size of 46mm x 52 mm shutter frame section size of 46mm x 46 mm |  |  |  |  |  | 1 Sqm | 4870.00 |
| 791 | Top Hung 4 '-0" x 2'-0" (1219.2x609.6mm) outer frame section size of 46 x 52 m shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4870.00 |
| 792 | Top Hung 4 '-0" x 3'-0" (1219.2x914.4mm) outer frame section size of 46 x 52m m shutter frame section size of 46 x 46 mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4870.00 |
| 793 | Fixed louvers 2'-0" x 2'-0" (609.6x609.6mm)(Box section) outer frame section size of 33 x 57 mm |  |  |  |  |  | 1 Sqm | 3334.00 |
| 794 | Fixed louvers 4'-0" x 2'-0" (1219.2x609.6mm)(Box section) outer frame section size of 33 x 57 mm Mullion section size of 33 x 57 |  |  |  |  |  | 1 Sqm | 3334.00 |
| 795 | Fixed louvers 4'-0" x 3'-0" (1219.2x914.4mm) (Box section) outer frame section size of 33 x 57 mm Mullion section size of 33 x 57 |  |  |  |  |  | 1 Sqm | 3334.00 |
|  | NCL or Equivalent ECO 3000 SERIES WINDOWS (SUITABLE FOR RESIDENTIAL BUILDINGS WITH GRILL PROVISION) |  |  |  |  |  |  |  |
|  | Providing and Fixing of windows made of pre-painted steel (Base steel as per IS 513 of 0.6mm thick galvanized as per IS 277 with Zinc of 120 GSM). Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be of 48x50mm, centre mullion should be of 48x50mm, section for shutter should be of 47x20mm and Fixed glass beading section should be of 12x12mm. Outer frame and mullions to have rebate for Glazed shutter and a 20mm provision for Guard bars/Grills. The sections are to be cut to length meter joined with corner bracket. Centre mullion is to be fixed with mullion cap. Seccolor Handle, seccolor stay, 2 nos. of Stainless Steel heavy duty Pivot hinges shall be provided per shutter. The windows should be paneled with 5mm thick plain float glass. Rubber Gaskets are provided all around the glass. The above frames should be fixed to the concrete/masonry wall by means of self expanding screws, Including 10mm Square guard bars with 6” (152.4mm) pitch etc., complete for finished item of work. |  |  |  |  |  |  |  |
| 796 | Single shutter Window 2’0”x4’0” (609.6mm x 1219.2mm). Outer frame section size of 48x50mm shutter frame section size of 47x20mm. |  |  |  |  |  | 1 Sqm | 4093.00 |
| 797 | Double shutter Window with vertical mullion 3’0” x 4’0” ( 914.4mm x 1219.2mm) outer frame section size of 48x50mm shutter frame section size of 47x20mm. Mullion section size of 48x50mm. |  |  |  |  |  | 1 Sqm | 4359.00 |
| 798 | Double shutter Window with vertical member4’0” x 4’0” (1219.2mm x 1219.2mm) outer frame section size of 48x50mm shutter frame section size of 47x20mm and mullion section should be of 48x50mm. |  |  |  |  |  | 1 Sqm | 4359.00 |
| 799 | Centre fixed both side openable shutter window 5’0”x4’0” (1524mm x1219.2mm). Outer frame section size of 48x50mm. Shutter frame section size of 47x20mm. Mullion section size of 48x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 3846.00 |
| 800 | Centre fixed both side openable shutter window 6’0”x4’0” (1828.8mm x1219.2mm). Outer frame section size of 48x50mm. Shutter frame section size of 47x20mm. Mullion section size of 48x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 3846.00 |
|  | NCL or Equivalent ECO 4000 SERIES WINDOWS (SUITABLE FOR RESIDENTIAL BUILDINGS WITH GRILL & FLYMESH PROVISION) |  |  |  |  |  |  |  |
|  | Providing & Fixing of windows made of pre-painted steel (Base Steel as per IS 513 of 0.6mm thick galvanized as per IS 277 with zinc of 150 GSM). Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be of 72x55mm, centre mullion should be of 72x50mm, Section for fixed glass beading section should be of 12x12mm and section for shutters should be of 47x20mm. Outer frame & mullion sections to have rebate for glazed shutters, fly mesh and a 20mm provision for guard bars/grills. Fly mesh shutter section should be of 20x40mm. The sections are to be cut to length meter joined with corner bracket. Centre mullions are to be fixed with mullion cap, seccolor stay, seccolor handles, seccolor latch 2 Nos of heavy duty stainless steel pivot hinges shall be provided per Shutter. The windows should be paneled with 5mm thick plain float glass and S.S. Mesh for fly mesh shutter(304 grade). Rubber Gaskets are provided all around the glass. The above frames should be fixed to the concrete/masonry wall by means of self expanding screws. Including 10mm Square guard bars with 6” (152.4mm) pitch , complete for finished item of work |  |  |  |  |  |  |  |
| 801 | Single shutter Window 2’0”x4’0” (609.6mm x 1219.2mm). Outer frame section size of 72x50mm shutter frame section size of 47x20mm. |  |  |  |  |  | 1 Sqm | 5434.00 |
| 802 | Double shutter Window with vertical mullion 3’0” x 4’0” ( 914.4mm x 1219.2mm) outer frame section size of 72x50mm shutter frame section size of 47x20mm. Mullion section size of 72x50mm. |  |  |  |  |  | 1 Sqm | 5699.00 |
| 803 | Double shutter Window with vertical member4’0” x 4’0” (1219.2mm x 1219.2mm) outer frame section size of 72x50mm shutter frame section size of 47x20mm and mullion section should be of 72x50mm. |  |  |  |  |  | 1 Sqm | 5699.00 |
| 804 | Centre fixed both side openable shutter window 5’0”x4’0” (1524mm x1219.2mm). Outer frame section size of 72x50mm. Shutter frame section size of 47x20mm. Mullion section size of 72x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 5188.00 |
| 805 | Centre fixed both side openable shutter window 6’0”x4’0” (1828.8mm x1219.2mm). Outer frame section size of 72x50mm. Shutter frame section size of 47x20mm. Mullion section size of 72x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 5188.00 |
|  | Fly mesh Shutters |  |  |  |  |  |  |  |
|  | Supply & fixing of Fly mesh shutters of size 20 x 40 mm fly proof mesh with std. Spn (S.S.) fly mesh including cost and conveyance of all materials, and labour charges etc. complete for finished item of work. |  |  |  |  |  |  |  |
| 806 | Single shutter 2'-0" x 4'-0" (609.6x1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 1022.00 |
| 807 | Double shutter 3'-0" x 4'-0" (914.4 x 1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 1022.00 |
| 808 | Double shutter 4'-0" x 4'-0" (1219.2 x 1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 1022.00 |
|  | Supply & fixing of Fly mesh shutters of size 20 x 40 mm fly proof mesh with Netlon fly mesh including cost and conveyance of all materials, and labour charges etc. complete for finished item of work. |  |  |  |  |  |  |  |
| 809 | Single shutter 2'-0" x 4'-0" (609.6x1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 881.00 |
| 810 | Double shutter 3'-0" x 4'-0" (914.4 x 1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 881.00 |
| 811 | Double shutter 4'-0" x 4'-0" (1219.2 x 1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 881.00 |
|  | Door frames |  |  |  |  |  |  |  |
|  | Supply & fixing of Door frames made of all formed sections of 1.25mm thick CRCA section, size should be 50x60mm with 32mm rebate. The corner of the frame should be welded. The frame should be painted with one coat of primer and finished painted with pure polyester paint. The frame should be provided with approved quality butt hinges of 3 Nos. complete for finished item. |  |  |  |  |  |  |  |
| 812 | Single shutter 3'-0" x 7'-0" (914.4x2133.6mm) outer frame section size of 50 x 60 mm |  |  |  |  |  | 1RM | 204.00 |
| 813 | Double shutter 5'-0" x 7'-0" (1524 x 2133.6mm) outer frame section size of 50 x 60 mm |  |  |  |  |  | 1 RM | 204.00 |
|  | Supplying & fixing of Door frame made of roll formed section of 1.25 mm thick CRCA section, size should be 50 x 60 mm with 32 mm rebate. The corner of the frame should be welded. The frame should be painted with one coat of primer and finished painted with pure polyester paint. The frame should be provided with approved quality butt hinges of 3 Nos. Complete for finished item of work. |  |  |  |  |  |  |  |
| 814 | Single shutter 3'-0" x 7'-0" (914.4x2133.6mm) outer frame section size of 50 x 75 mm |  |  |  |  |  | 1 RM | 232.00 |
| 815 | Single shutter 5'-0" x 7'-0" (1524x2133.6mm) outer frame section size of 50 x 75 mm |  |  |  |  |  | 1 RM | 232.00 |
|  | NCL or Equivalent Structural Glazing: |  |  |  |  |  |  |  |
|  | Providing and Fixing Structural Glazing fabricated from Roll formed sections made of Pre-painted Steel / Powder coated (Base steel as per IS 513 of ‘D’ quality, galvanized as per IS 277 with Zinc of 120 Gm/Sqm) with total coated thickness of 0.72mm. The glass holding section made of 304 grade stainless steel of 0.6mm thick. Galvanized steel sections are to be sued as stiffeners inside the colour coated steel/powder coated sections as per the design requirement. Design calculations are made of suit wind pressures as given in IS 875. Primer coat with epoxy primer of 5 – 7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with Alkyd backer of 5-7 microns or powder coated with pure polyester powder up to 50-60 microns thick. The vertical section should be of 50mm x100mm x 0.72mm, 33mm x 58mm x 0.72mm for frame horizontal section, stiffener section should be 48mm x 98mm x 1mm, cover profiles sections should be of 20mm x 58mm x 0.58mm, Glass holding section should be of 37mm x 37mm & 37mm x 18mm. Including 5mm thick Ocean Blue reflective glass. Brackets made of CRCA powder coated/Electroplated should be used to connect vertical to horizontal, vertical to slab, to fix verticals at top & bottom as per site requirement. |  |  |  |  |  |  |  |
|  | Gasket made of Ethyl Propylene Diamine Monomer. Natural cure, with Good U.V. Resistance Silicon to be used. Wall fixing of sections to concrete/masonry wall should be with self-expanding cap & screws. The rate is inclusive of cost and conveyance of all materials to site, all labour charges, incidental charges, cost of all consumables etc. and scaffolding charges, form work etc. complete for finished item of work. |  |  |  |  |  |  |  |
| 816 | Structural Glazing: |  |  |  |  |  | 1 Sqm | 5003.00 |
|  | NCL or Equivalent Top Hung in Structural Glazing: |  |  |  |  |  |  |  |
|  | Providing and Fixing Top Hung shutters in Structural Glazing fabricated from Roll formed sections made of Pre-painted Steel / Powder coated (Base steel as per IS 513 of ‘D’ quality, galvanized as per IS 277 with Zinc of 120 Gm/Sqm) with total coated thickness of 0.72mm. The glass holding section made of 304 grade stainless steel of 0.6mm thick. Galvanized steel sections are to be used as stiffeners inside the colour coated steel/powder coated sections as per the design requirement. Design calculations are made to suit wind pressures as given in IS 875. Primer coat with epoxy primer of 5-7 microns thick and back coated with Alkyd backer of 5-7 microns or powder coated with pure polyester powder up to 50-60 microns thick. The outer frame should be of 20mm x 58mm x 0.6mm, Shutter section should be of 58mm x 33mm x 0.72 mm, stiffener section should be 56mm x 31mm x 1mm. EARL Bihari Hinges – 2 Nos for each shutter and Aluminium Stays – 2 Nos. for each shutter, Aluminium Handle – 1 no. for each shutter. The rate is inclusive of cost and conveyance of all materials to site, all labour charges, incidental charges, cost of all consumables etc. and scaffolding charges, form work etc. complete for finished item of work |  |  |  |  |  |  |  |
| 817 | Top Hung Shutter |  |  |  |  |  | 1 Sqm | 4025.00 |
|  | NCL or Equivalent Partitions: |  |  |  |  |  |  |  |
|  | Supply and fixing of partitions made of seccolor pre-painted steel (base steel as per IS 513 of 0.6mm thick ‘D’ quality, galvanized as per IS 277 with Zinc of 120 GSM). Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick alkyd backer. Section for outer frame should be 46x52mm, section for shutter should be 46x46mm, section for mullion should be 46x70mm, section for beading should be 18x25mm and section for middle and bottom rail (Lock Rail and Kick Rail) of size should be 23x130mm. The partition should be paneled with 9mm pre-laminated particle board to a height of 0.91m from the bottom, and remaining height with 5mm thick plain glass. with Ethyl Propylene Diamine Monomer (EPDM) Gaskets. The sections are to be cut to length, mitre joined with corner bracket Centre mullions are to be fixed using mullion cap. Gaskets made of Ethyl Propylene Diamine Monomer (EPDM). Corner brackets made of CRCA with zinc phosphating. Mullion caps made of Glass filled nylon. The above frames should be fixed to the concrete/masonry wall by means of self expanding screws, and complete for finished item of work. |  |  |  |  |  |  |  |
| 818 | Fixed Partition: Outer frame section size of 46x52mm, mullion section should be of 46x70mm, fixed beading should be of 18x25mm. |  |  |  |  |  | 1 Sqm | 3420.00 |
| 819 | Fixed Partition with Door: Outer frame section size of 46x52mm, mullion section should be of 46x70mm, fixed beading should be of 18x25mm, section for shutter should be of 46x46mm and section for middle & bottom should be of size 23x130mm |  |  |  |  |  | 1 Sqm | 3901.00 |
| 820 | Providing and fixing door frames fabricated from sections made of galvanized steel powder coated (base steel as per IS 513, galvanised as per IS 217 with zinc of 120 grams / square meter). The factory made section should be with a Powder Coating of pure Polyester Powder of 50-60 microns thick with total coated thickness of 1.25 mm. Size of door frame section should be of 105x60mm with a rebate of 38mm to accommodate 35mm thick door shutter. The frame section should have groove to insert gasket. The section of the door and fan light frame is to be filled with polyurethane foam. The frame section should be cut to length, and joined by way of welding |  |  |  |  |  | 1 RM | 558.00 |
|  | Each frame should be provided with: |  |  |  |  |  |  |  |
|  | a) 2mm thick MS powder coated butt hinges of 100nn long – 3 Nos. for single shutter, 6 Nos. for double shutter. |  |  |  |  |  |  |  |
|  | b) CRCA Electroplates stiffeners of 6 Nos. – 3 in each vertical. |  |  |  |  |  |  |  |
|  | c) 40mm x 200mm x 1.2mm hold fasts screwed to stiffeners – 6 Nos. with split end tail. |  |  |  |  |  |  |  |
|  | d) Nylon Aldrop Receiver – 1 No. for single shutter frame. e)Tie Rod – 1 No |  |  |  |  |  |  |  |
|  | Curtain Glazing NCL |  |  |  |  |  |  |  |
|  | Supply and fixing of curtain Glazing made of pre painted steel (base steel as per IS 513 of 0.6 mm thick D quality, galvanized as per IS 277 with Zinc of 120 GSM) Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12- 16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be 46 x 52 mm, section for mullion should be 46 x 70 mm and section for beading should be 18 x 25 mm. The Glazing should be paneled with 5 mm thick plain float Glass with Ethyl Propylene diameine Monomer (EPDM) Gaskets. The sections are to be cut to length, mitre joined with corner bracket. Centre mullions are to be fixed suing mullion cap. Handle made of high grade Aluminium powder coated and nylon receiver. Gaskets made of Ethyl propylene Diamine Monomer (EPDM) Corner brackets made of CRCA with zinc phosphating. Mullion caps made of Glass filled nylon. The above frames should be fixed to the concrete / masonry wall be means of self expanding screws. & complete for finished item. |  |  |  |  |  |  |  |
| 821 | Fixed Glazing 2'-0" x 2'-0" (609.6x609.6mm) grid outer frames section size of 46 x 52 mm |  |  |  |  |  | 1 Sqm | 4111.00 |
|  | Mullion section size of 46 x 70mm |  |  |  |  |  |  |  |
|  | Beading section size of 18 x 25 mm |  |  |  |  |  |  |  |
| 822 | Fixed Glazing 2'-0" x 3'-0" (609.6x914.4mm) grid outer frame section size of 46 x 52 mm |  |  |  |  |  | 1 Sqm | 4111.00 |
|  | Mullion section size of 46 x 70 mm |  |  |  |  |  |  |  |
|  | Beading section size of 18 x 25 mm |  |  |  |  |  |  |  |
| 823 | Fixed Glazing 3'-0" x 3'-0" (914.4x914.4mm) grid outer frame section size of 46 x 52 mm |  |  |  |  |  | 1 Sqm | 3498.00 |
|  | Mullion section size of 46 x 70 mm |  |  |  |  |  |  |  |
|  | Beading section size of 18 x 25 mm |  |  |  |  |  |  |  |
| 824 | Fixed Glazing 3'-0" x 4'-0" (914.4x1219.2mm) grid outer frame section size of 46 x 52 mm |  |  |  |  |  | 1 Sqm | 3498.00 |
|  | Mullion section size of 46 x 70 mm |  |  |  |  |  |  |  |
|  | Beading section size of 18 x 25 mm |  |  |  |  |  |  |  |
|  | NCL or Equivalent Sliding Windows |  |  |  |  |  |  |  |
| 825 | Providing and Fixing Sliding Window fabricated from Roll formed sections made of galvanized Steel colour coated/powder coated (Base Steel as per IS 513 ‘D’ quality, galvanized as per IS 277 with zinc of 120 GM/Sq.mtr) with total coated thickness of 0.60mm. Primer coat with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with Alkyd backer of 5-7 microns or powder coated with pure polyester powder up to 50-60 microns thick. Section for external frame bottom should be of 79x44mm, external frame sides and top should be of 69x24mm, guide for top and bottom should be of 69x26mm (Stainless Steel). Section for shutter should be of 26x30mm. The windows should be paneled with 5mm thick plain float glass. Corner bracket for internal and external frame made of glass filled nylon. Gaskets are to be made of Ethyl Propylene Diamine Monomer (EPDM).The sections are to be cut to length, joined and assembled by means of corner bracket. The above frames should be fixed to the concrete/masonry walls by means of self expanding brackets & screws including 10mm square guard bars with 6” pitch (152.4mm) and all Taxes complete for finished item of work |  |  |  |  |  | 1 Sqm | 4712.00 |
|  | ELIXIR OR EQUALENT DOOR FRAMES |  |  |  |  |  |  |  |
|  | Supply and fixing of factory made Internal door frames without threshold, fabricated from pre-painted cold roll formed section 100mm x 60 mm made of galvanized ( base steel as per IS – 513 ‘D’ quality, galvanized as per IS-277 with Zinc of 120 gms/Sqm) steel colour coated sections with total coated thickness of 1 mm, primer coated with epoxy primer of 5-7 microns thick, finish painted with polyester paint of 12-16 microns and back coated with alkyd backer of 5-7 microns. The frame section is to be filled with polyurethane (P.U. Foam) of 40 Kg/Cum; and the sections are cut to length and miter joined by means of 1.25mm thick CRCA electroplated corner brackets of size 75 x 75 mm and 1.25mm thick electroplated stiffeners of size 150mm for fixing hold fasts and hinges together with the following MS Power coated fitting (Supply and fixing ) of approved make a) 6 Nos M.S. Z. Type hold fasts made of 30mm x 3 mm MS flat of 230mm long, 3 Nos each side expandable PVC covered screw type fasteners |  |  |  |  |  |  |  |
|  | b) 2.5 mm x 300 mm long ‘T’ hinges / 2.5mm x 50 mm x 100mm two way but hinges (3 Nos for single leaf or 2 Nos for Double leaf /leaf ) |  |  |  |  |  |  |  |
| 826 | c) 12 mm Dia MS tie rod at bottom with necessary size of nuts on either side |  |  |  |  |  | 1 RM | 385.00 |
|  | ELIXIR SUPER SERIES OR EQUIVALENT PRE – PAINTED STEEL WINDOWS / VENTILATORS & DOOR FRAMES |  |  |  |  |  |  |  |
|  | Supply and fixing of windows & top hung and fixed louvered Ventilators made of pre – painted steel (base steel as per IS 513 of -0.6 mm thick ‘D” quality, galvanized as per IS 277 with zinc of 120 GSM) primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint, of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be 46 x 52 mm, section for shutter should be 46 x 46 mm, section for mullion should be 46x70 mm, and section for beading should be 18x25 mm and section for louvered ventilator should be 33x57 mm Box section. The windows should be paneled with 5 mm thick plain float glass & 4 mm pinhead glass for ventilators with Ethyl propylene Diamine monomer Gasket (EPDM). The sections are to be cut to length, mitre joined with corner bracket and centre mullions are to be fixed using mullion cap. Handle made of high grade aluminum powder coated and nylon receiver. Gaskets made of Ethyl propylene Diamine monomer(EPDM). Corner brackets made of CRCA with Zinc phosphate. Mullion caps made of glass filled nylon. The above frames should be fixed to the concrete / masonry wall by means of self expanding screws. Including 10 mm square bars with 6” pitch complete finished item of work including all taxes. |  |  |  |  |  |  |  |
|  | WINDOWS |  |  |  |  |  |  |  |
| 827 | Single shutter 2’0x4’0(609.6x1219.2 mm) Outer frame section size of46x52 mm shutter frame section size of 46x46mm |  |  |  |  |  | 1 Sqm | 4252.00 |
| 828 | Double shutter 3’- 0” X 4’ – 0” (914.4MM X 1219.2MM) outer frame section size of 46x52 mm shutter frame section size of 46x46 mm |  |  |  |  |  | 1 Sqm | 4252.00 |
| 829 | Double shutter 3’- 0” x 4’- 0” (914.4mm x 1219.2 mm) outer frame section size of 46 x 52 mm shutter frame section size of 46x46 mm mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4506.70 |
| 830 | Double shutter 4’ – 0” x 4’ – 0” (1219.2x12.19.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm |  |  |  |  |  | 1 Sqm | 4252.00 |
| 831 | Double shutter 4’-0” x 4’-0” (1219.2x1219.2mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm mullion section size of 46 x 70mm |  |  |  |  |  | 1 Sqm | 4506.00 |
| 832 | Double shutter 4’-6” x 4’-6” (1371.6 x 1371.6mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46mm Mullion section size of 46 x 70 mm |  |  |  |  |  | 1 Sqm | 4020.00 |
| 833 | Centre fixed both side openable shutter 5’-0” x 4’-0”(1524x1219.2mm) outer frame section size of 46x52 mm shutter frame section size of 46 x 46 mm mullion section size of 46 x 70 mm fixed beading section size of 18 x 25mm |  |  |  |  |  | 1 Sqm | 4020.00 |
| 834 | Centre fixed both side openable shutter 6’-0” x 4’-0” (1828.8 x 1219.2 mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm mullion section size of 46 x 70 mm fixed beading section size of 18 x 25mm |  |  |  |  |  | 1 Sqm | 4020.00 |
| 835 | Centre fixed both side openable shutter 6’-0” x 4’-6” (1828.8 x 1371.6 mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm mullion section size of 46 x 70 mm fixed beading section size of 18 x 25mm |  |  |  |  |  | 1 Sqm | 4020.00 |
|  | Ventilators : |  |  |  |  |  |  |  |
| 836 | Top hung 2’ – 0” x 2’ -0” (609.6 x 609.6mm) outer frame section size of 46 mm x 52mm shutter frame section size of 46mm x 46mm |  |  |  |  |  | 1 Sqm | 4659.00 |
| 837 | Top hung 4’-0” x 2’-0” (1219.2 x 609.6mm) outer frame section size of 46 x 52 m shutter frame section size of 46x46mm mullion section size of 46 70 mm |  |  |  |  |  | 1 Sqm | 4659.00 |
| 838 | Top hung 4’-0” x 3’=0” ( 1219.2 x 914.4mm) outer frame section size of 46 x 52 mm shutter frame section size of 46 x 46 mm mullion section size of 46 x 70mm |  |  |  |  |  | 1 Sqm | 4659.00 |
| 839 | Fixed louvers 2’-0” x 2’-0”(609.6x 609.6mm)(Box section) outer frame section size of 33x57mm |  |  |  |  |  | 1 Sqm | 3188.00 |
| 840 | Fixed louvers 4’-0” x 2’-0” (1219.2 x 609.6mm) ) (Box section) outer frame section size of 33x57mm mullion section size of 33 x 57 |  |  |  |  |  | 1Sqm | 3188.00 |
| 841 | Fixed lovers 4’-0” x 3’-0” (1219.2 x 914.4 mm) (Box section) outer frame section size of 33 x 57 mm mullion section size of 33 x 57 |  |  |  |  |  | 1 Sqm | 3188.00 |
|  | ELIXIR E 1000 SERIES OR EQIVALENT WINDOWS (SUITABLE FOR RESIDENTIAL BUILDINGS WITH GRILL PROVISION) |  |  |  |  |  |  |  |
|  | Providing and fixing of windows made of pre-painted steel (base steel as per IS 513 of 0.6mm thick galvanized as per IS 277 with zinc of 120 GSM). Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be of 48x50mm, centre mullion should be of 48x50mm, section for shutter should be of 47x20mm and fixed glass beading section should be of 12x12mm. Outer frame and mullions to have rebate for glazed shutter and a 20mm provision for guard bars/grills. The sections are to be cut to length meter joined with corner bracket. Centre mullion is to be fixed with mullion cap. Handle, 2 nos of stainless steel heavy duty pivot hinges shall be provided for shutter. The window should be paneled with 5mm thick plain float glass. Rubber gaskets are provided all around the glass. The above frames should be fixed to the concrete/masonry wall by means of self expanding screws, including 10mm square guard bars with 6” (152.4 mm) pitch etc., complete for finished item of work including of all taxes. |  |  |  |  |  |  |  |
| 842 | Single shutter window 2’0”x4’0” (609.6mm x 1219.2mm).outer frame section size of 48x50mm shutter frame section size of 47x20mm. |  |  |  |  |  | 1 Sqm | 3915.00 |
| 843 | Double shutter window with vertical mullion 3’0” x 4’0” (914.4mm x 1219.2mm) outer frame section size of 48x50mm shutter frame section size of 47x20mm. mullion section size of 48x50mm |  |  |  |  |  | 1 Sqm | 4170.00 |
| 844 | Double shutter window with vertical member 4’0” x 4’0” (1219.2mm x 1219.2mm) outer frame section size of 48x50mm shutter frame section size of 47x20mm and mullion section should be of 48x50mm |  |  |  |  |  | 1 Sqm | 4170.00 |
| 845 | Centre fixed both side openable shutter window 5’0”x4’0” (1524mm x 1219.2mm). Outer frame section size of 48x50mm. Shutter frame section size of 47x20mm. Mullion section size of 48x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 3678.00 |
| 846 | Centre fixed both side openable shutter window 6’0”x4’0” (1828.8mm x 1219.2mm). outer frame section size of 48x50mm. shutter frame section size of 47x20mm. Mullion section size of 48x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 3678.00 |
|  | ELIXIR E-2000 SERIES, OR EQUIVALENT WINDOWS (SUITABLE FOR RESIDENTIAL BUILDINGS WITH GRILL & FLYMESH PROVISION) |  |  |  |  |  |  |  |
|  | Providing & fixing of windows made of pre-painted steel (Base steel as per IS 513 of 0.6mm thick galvanized as per IS 277 with zinc of 150 GSM). Primer coated with epoxy primer of 5-7 microns thick, finish painted with a polyester paint of 12-16 microns thick and back coated with 5-7 microns thick alkyd backer. Section for outer frame should be of 72x55mm, centre mullion should be of 72x50mm, section for fixed glass beading should be of 12x12mm and section for shutter should be of 47x20mm. Outer frame & mullion sections to have rebate for glazed shutters, fly mesh and 20 mm provision for guard bars/ grills fly mesh shutter section should be of 20x40mm. The sections are to be cut to length Mitre joined with corner bracket. Center mullions to be fixed with mullion cap. Elixir or equivalent stay, handles latch 2nos of heavy duty stain less steel pivot hinges shall be provided per shutter. The windows should be paneled with 5 mm thick plain float glass and S.S. Mesh for fly mesh shutter (304 grade). Rubber Gaskets are provided all around the glass. The above frames should be fixed to the concrete/masonry wall by means of self expanding screws. Including 10mm square guard bars with 6” (152.4mm) pitch, complete for finished item of work including of all taxes. |  |  |  |  |  |  |  |
| 847 | Single shutter window 2’0”x4’0” (609.6mm x 1219.2mm). Outer frame section size of 72x50mm shutter frame section size of 47x20mm. |  |  |  |  |  | 1 Sqm | 5308.00 |
| 848 | Double shutter window with vertical mullion 3’0” x 4’0” (914.4mm x 1219.2mm) outer frame section size of 72x50mm shutter frame section size of 47x20mm. Mullion section size of 72x50mm. |  |  |  |  |  | 1 Sqm | 5452.00 |
| 849 | Double shutter window with vertical mullion 4’0” x 4’0” (1219.2mm x 1219.2mm) outer frame section size of 72x50mm shutter frame section size of 47x20mm and mullion section should be of 72x50mm. |  |  |  |  |  | 1 Sqm | 5452.00 |
| 850 | Centre fixed both side openable shutter window 5’0”x4’0”(1524mm x 1219.2mm).outer frame section size of 72x50mm. Shutter frame section size of 47x20mm and mullion section size of 72x50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 4969.00 |
| 851 | Centre fixed both side openable shutter window 6’0” x 4’0” (1828.8mm x 1219.2mm). Outer frame section size of 72x50mm.shutter frame section size of 72 x 50mm. Fixed beading section size of 12x12mm. |  |  |  |  |  | 1 Sqm | 4969.00 |
|  | Fly mesh shutters |  |  |  |  |  |  |  |
|  | Supply & fixing of fly mesh shutters of size 20x40mm fly proof mesh with std. Spn (S.S) fly mesh including cost and conveyance of all materials, and labour charges etc. Complete for finished item of work including of all taxes. |  |  |  |  |  |  |  |
| 852 | Single shutter 2’-0” x 4’-0” ( 609.6 x 1219.2mm) Shutter section size 20 x 40mm |  |  |  |  |  | 1 Sqm | 978.00 |
| 853 | Double shutter 3’-0” x 4’-0” (914.4 x 1219.2mm) shutter section size 20 x 40mm |  |  |  |  |  | 1 Sqm | 978.00 |
| 854 | Double shutter 4’-0” x 4’-0” (1219.2 x 1219.2mm) shutter section size 20 x 40mm |  |  |  |  |  | 1 Sqm | 978.00 |
|  | Supply & fixing of fly mesh shutters of size 20 x 40 mm fly proof mesh with Netlon fly mesh including cost and conveyance of all materials, and labour charges etc. complete for finished item of work including of all taxes. |  |  |  |  |  |  |  |
| 855 | Single shutter 2’-0” x 4’-0” (609.6 x 1219.2mm) shutter section size 20 x 40mm |  |  |  |  |  | 1 Sqm | 842.00 |
| 856 | Double shutter 3’-0” x 4’= 0” ( 914.4 x 1219.2mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 842.00 |
| 857 | Double shutter 4’-0” x 4’-0” (1219.2 x 1219.2 mm) shutter section size 20 x 40 mm |  |  |  |  |  | 1 Sqm | 842.00 |
|  | ELIXIR PEB OR EQUIVALENT PRE ENGINEERED BUILDINGS FOR A MAXIMUM SPAN OF 9 MTRS |  |  |  |  |  |  |  |
|  | Design, fabrication & supply of light weight pre engineered buildings for a maximum span of 9 mtrs with 50x50x2.8 mm M.S hollow square tubes for columns, trusses & purlins walls with sand which panels made up of 2 Nos 10mm thick cement bonded particle/ fiber board’s with 25mm thick 12 kg/cum density expanded polystyrene in between encased in 1.25 mm thick 24x7x26 mm ‘W’ tracks in bottom & top and ‘H’ section for sides with doors, windows & ventilators, roofing with 26 gauge pre painted profiled sheets fixed with self tapping/ drilling screws, painting with enamel paint for steel items, OBD for interiors and ace for exteriors & providing base plates, templates & anchor bolts along with civil foundation designs & drawings for the finished item of work. Inclusive of all Taxes and Duties. |  |  |  |  |  |  |  |
|  | SPECIFICATION FOR LIGHT WEIGHT PRE - ENGINEERED BUILDINGS. FOR MAXIMUM SPAN OF 9 MTRS. |  |  |  |  |  |  |  |
| 858 | Steel Structure: - Structure is made up of 50x50X 2.8 m/33x66x2.8mm thick hollow M.S. sections for Trusses, columns, and purloins. |  |  |  |  |  | 1 RM | 275.00 |
| 859 | 10 mm thick 200x200 mm M.s. plates for base plates, 250 mm long 12mm Φ anchor bolts with plate washer and nuts complete with one coat of Asian or equivalent red oxide paint. |  |  |  |  |  | Each | 450.00 |
| 860 | Sand which panels: Sand which panels of 50 mm thick with 25 mm Thermocole of 12 kg / cum, fixed in between 2 Nos 10 mm cement bonded wood particle / fiber board’s properly fixed together with 30 mmx30mmx30mm wood pieces along width & length wherever necessary |  |  |  |  |  | 1 Sqm | 1050.00 |
| 861 | U' Tracks & 'H' Section:- Bottom & Top 'U' Tracks are made of 1.25 mm thick 120 GSM G.I. Sheet of size 24x51x26 mm size. |  |  |  |  |  | 1 RM | 108.00 |
| 862 | Fabricated 'H' sections are made up of 2 Nos 1.25 mm thick C.R. Of 40 mm width for flanges and 1 Nos of 2 mm thick C.R. of 50 mm width plate for web. will be used |  |  |  |  |  | 1 RM | 150.00 |
| 863 | Sand which Panel joinery is made up of "Elixir" couplers or equivalent made from 0.6 mm G.I. roll formed section to hold 2 Nos of 10 mm board on both sides with 25 mm Thermocole in between. |  |  |  |  |  | 1RM | 175.00 |
| 864 | Window:- windows are made up of 51 x51 mm step 'U' section made from 1.25 thick MS Cold rolled Steel Sheet and Windows Shutters & Grill are made of 47x20 mm size pre painted section with 4 mm glass complete with necessary hardware. Grills are made from 8 mm square rod at 100 mm spacing. |  |  |  |  |  | 1 Sqm | 3200.00 |
| 865 | Door Frames :-51 X 51 Stepped ' U' section made from 1.25 mm thick C.R. section with a 32 mm rebate for 30 mm flush door and Door Shutters with 30 mm thick block board / flush doors with boiling water proof plywood complete with necessary Hard ware. |  |  |  |  |  | 1 Sqm | 3000.00 |
| 866 | Ventilators:- Ventilators are made of 51x51 mm step 'U' section made From 1.25 mm thick C.R. sheet with fixed louvers and glass. |  |  |  |  |  | 1 Sqm | 3600.00 |
| 867 | Painting:-1) Steel structure is to be painted with 2 coats of Grade I enamel paint. |  |  |  |  |  | 1 Sqm | 80.00 |
| 868 | 2) Exterior walls are painted with one coast of primer and 2 coats of ACE" or equivalent paint. |  |  |  |  |  | 1 Sqm | 70.00 |
| 869 | 3) Interiors are painted with 1+2 coat of O.B.D. |  |  |  |  |  | 1 Sqm | 60.00 |
| 870 | Toilets: - Toilets are provided as per plan with a provision for fixing the wash basin, W.C. etc only is made with M.S. steel section secured to the walls with toilet bracket provision excluding cost of WC/ Wash basin etc |  |  |  |  |  | Each | 600.00 |
| 871 | Sun shades: - Sun shades will be provided wherever necessary with 300 mm more than the width of window. 25x25 mm angle frame to cover 450 mm in the front and the sides. 10mm cement bonded particle board will be fixed on the frame work and sealed with green paste or equivalent. |  |  |  |  |  | Each | 2500.00 |
| 872 | Roofing:- Roofing will be provided with 0.5mm thick galvanized / pre painted G.I. profiled sheets fixed to the purlins with 14 size self drilling screws with neoprene washer. Side laps are stitched with self tapping / drilling screws. End laps are to be sealed with 25x3 mm Butyl tape. The sheets are provided with anti capillary grove. |  |  |  |  |  | 1 Sqm | 520.00 |
|  | Supply and fixing of factory made Bathroom door frames without threshold, fabricated from pre-painted cold roll formed section 50mm x 50 mm made of galvanized (base steel as per IS – 513 ‘D’ quality, galvanized as per IS-277 with Zinc of 120 gms/Sqm) steel colour coated sections with total coated thickness of 1 mm, primer coated with epoxy primer of 5-7 microns thick, finish painted with polyester paint of 12-16 microns and back coated with alkyd backer of 5-7 microns. The frame section is to be filled with polyurethane (P.U. Foam) of 40 Kg/Cum; and the sections are cut to length and miter joined by means of 1.25mm thick CRCA electroplated corner brackets of size 75 x 75 mm and 1.25mm thick electroplated stiffeners of size 150mm for fixing hold fasts and hinges together with the following MS Power coated fitting (Supply and fixing) of approved make. |  |  |  |  |  |  |  |
|  | a) 6 Nos M.S. Z. Type hold fasts made of 30mm x 3 mm MS flat of 230mm long, 3 Nos each side expandable PVC covered screw type fasteners |  |  |  |  |  |  |  |
|  | b) 2.5 mm x 300 mm long ‘T’ hinges / 2.5mm x 50 mm x 100mm two way but hinges (3 Nos for single leaf or 2 Nos for Double leaf /leaf ) |  |  |  |  |  |  |  |
| 873 | c) 12 mm Dia MS tie rod at bottom with necessary size of nuts on either side. |  |  |  |  |  | 1 RM | 327.00 |
|  | SPECIFICATION OF HARDWYN OR EQUIVALENT DOOR CLOSER ISI MARKED |  |  |  |  |  |  |  |
| 874 | Providing and fixing IS:3564 marked Aluminium Die cast body tubular type universal hydraulic door closer, Hardwyn make (Classic Queen) with necessary accessories and screws etc complete |  |  |  |  |  | Each | 549.00 |
| 875 | Providing and fixing IS:3564 marked Aluminium Extruded Section body tubular type universal hydraulic door closer with double speed adjustment, Hardwyn make (Mytel) with necessary accessories and screws etc complete |  |  |  |  |  | Each | 732.00 |
| 876 | Providing and fixing IS:3564 marked Aluminium Extruded Section body tubular type universal hydraulic door closer with double speed adjustment, Hardwyn make (Eddy) with necessary accessories and screws etc complete |  |  |  |  |  | Each | 862.00 |
| 877 | Providing and fixing IS:3564 marked Aluminium Extruded Section body tubular type universal hydraulic door closer with double speed adjustment, Hardwyn make (Gazel) with necessary accessories and screws etc complete |  |  |  |  |  | Each | 902.00 |
|  | SPECIFICATION OF HARDWYN OR EQUIVALENT FLOOR SPRING ISI MARKED |  |  |  |  |  |  |  |
| 878 | Providing and fixing double action hydraulic floor spring of approved brand manufacture IS : 6315 marked, Hardwyn make (U-32) for doors including cost of cutting floors as required, embedding in floors and S.S cover plates with Aluminium Pivot and single piece M.S. Sheet outer box with slide plate etc. complete (Weight Capacity up to 130 Kgs) |  |  |  |  |  | Each | 2182.00 |
| 879 | Providing and fixing double action hydraulic floor spring of approved brand manufacture IS : 6315 marked, Hardwyn make (U-32) for doors including cost of cutting floors as required, embedding in floors and Brass cover plates with Aluminium Pivot and single piece M.S. Sheet outer box with slide plate etc. complete (Weight Capacity up to 130 Kgs) |  |  |  |  |  | Each | 2313.00 |
| 880 | Providing and fixing double action hydraulic floor spring of approved brand manufacture Hardwyn make (U-88) for doors including cost of cutting floors as required, embedding in floors and S.S cover plates with Aluminium Pivot and single piece M.S. Sheet outer box with slide plate etc. complete (Weight Capacity up to 80 Kgs) |  |  |  |  |  | Each | 1634.00 |
| 881 | Providing and fixing double action hydraulic floor spring of approved brand manufacture Hardwyn make (U-88) for doors including cost of cutting floors as required, embedding in floors and Brass cover plates with Aluminium Pivot and single piece M.S. Sheet outer box with slide plate etc. complete (Weight Capacity up to 80 Kgs) |  |  |  |  |  | Each | 1764.00 |
|  | ALUMINIUM DOORS AND WINDOWS |  |  |  |  |  |  |  |
| 882 | Supplying Aluminium Door frames with rectangular box hollow section (including annodization cost) |  |  |  |  |  | 1 Kg | 250.00 |
| 883 | Supplying Aluminium anodized openable sliding window, Ventilator (including annodization cost) |  |  |  |  |  | 1 Kg | 250.00 |
| 884 | A.C sheet Corrugated 6 mm thick |  |  |  |  |  | 1 Sqm | 188.00 |
| 885 | A.C sheet plain 4 m thick |  |  |  |  |  | 1 Sqm | 135.00 |
| 886 | A.C sheet plain 6 m thick |  |  |  |  |  | 1 Sqm | 140.00 |
| 887 | Sun Control film to the glazed windows partition including labour charges. |  |  |  |  |  | 1 Sqm | 285.00 |
| 888 | Aluminium Venetian Blinds horizontal 25.4 mm wide with all accessories. |  |  |  |  |  | 1 Sqm | 727.00 |
| 889 | Aluminium Venetian Blinds horizontal 20.0 mm wide with all accessories. |  |  |  |  |  | 1 Sqm | 693.00 |
| 890 | Venetian blinds Vertical blinds 100 mm wide with all accessories. |  |  |  |  |  | 1 Sqm | 1176.00 |
| 891 | S&F of marblex flooring with 2.0 mm thick in rolls. |  |  |  |  |  | 1 Sqm | 369.00 |
| 892 | S&F Vinyl Flooring with 1.5 mm thick in rolls |  |  |  |  |  | 1 Sqm | 369.00 |
|  | Providing Chemical Water proof treatment with Brick bat coba & chemical cement mortars to exposed RCC roof slab surfaces to required slopes mixed with water proofing chemical compound, laid over roof slab and brickbat coba treatment in chemical cement mortar 120/100 mm thick at the centre and 80/60 mm thick at the edges finished smooth with a floating coat of chemical mixed cement mortar as per the manufacturer specification including cost and conveyance of all materials like sand, chemical water proofing compound, water etc., to site, including seigniorage charges and operational, incidental, and labour charges for mixing mortar, laying, lift charges, rendering smooth curing including rounding off junctions of wall and slab etc., complete for finished item of work. (with a guarantee of 10 years) |  |  |  |  |  |  |  |
| 893 | 100 mm Average thick |  |  |  |  |  | 1 Sqm | 475.00 |
| 894 | 80 mm Average thick |  |  |  |  |  | 1 Sqm | 400.00 |
|  | Supplying and fixing of Unplasticised Poly Vinyl Chloride (UPVC) sliding windows duly manufactured using UPVC reinforced profiles of 60 mm x 62 mm x 2.5 mm for outer frames, 77 mm x 38 mm x 2.5 mm for sliding shutter frames the profiles shall be extruded online with properties of impact resistance, UV Stability, corrosion resistance, colour fastness, chemical resistance, low thermal expansion, fire retarding, aging resistant, sea water resistant, capable of mounting single/ double glazing system as per requirements, structurally reinforced with galvanized, hot dip galvanized up to 50 microns suitable passivated for corrosion resistance for tropical deployment of minimum thickness of 1 mm and shall not be exposed to any drainage routing, prefabricated & welded through fusion welding and assembled in factory, the window sash shall be fitted with 5 mm thick tinted/ float glass of reputed make duly fixed with ethylene propylene di-monomer (EPDM) weathering seal resistant to tear & decay and exposure to ozone, ultra violet radiation and tropical climates and accessories like clipping locking system made of aluminium 1 No., per set of sashes and the system is to be installed at the site using anchor fasteners, silicon rubber sealant of approved make all-round the outer frame from inside and outside to absorb and buffer expansion of wall, the construction of window system shall have a suitable water drainage facility and shall allow easy glazing/ deglazing at site etc., including cost and conveyance of all materials, accessories, labour charges for transportation, erection at site with templates for casement sizing complete for finished item of work (Guarantee for all manufacturing defects including accessories but excluding Glass panes) |  |  |  |  |  |  |  |
| 895 | A) SLIDING WINDOW - Single Glazed - Tinted Sash |  |  |  |  |  | 1 Sqm | 4630.00 |
| 896 | B) SLIDING WINDOW - Single Glazed - Plain Sash |  |  |  |  |  | 1 Sqm | 4520.00 |
| 897 | C) SLIDING WINDOW - Double Glazed - Tinted Sash |  |  |  |  |  | 1 Sqm | 6340.00 |
| 898 | D) SLIDING WINDOW - Double Glazed - Plain Sash |  |  |  |  |  | 1 Sqm | 6095.00 |
|  | Supplying and fixing of Unplasticised Poly Vinyl Chloride (UPVC) Casement windows duly manufactured using UPVC reinforced profiles of 60 mm x 56 mm x 2.5 mm for outer frames, 60 mm x 70 mm x 2.5 mm for casement shutter frames and the mullion sections are of 60 mm x 70 mm x 2.5 mm the profiles shall be extruded online with properties of impact resistance, UV Stability, corrosion resistance, colour fastness, chemical resistance, low thermal expansion, fire retarding, aging resistant, sea water resistant, capable of mounting single/ double glazing system as per requirements, structurally reinforced with galvanized, hot dip galvanized up to 50 microns suitable passivated for corrosion resistance for tropical deployment of minimum thickness of 1 mm and shall not be exposed to any drainage routing, prefabricated & welded through fusion welding through fusion welding and assembled in factory, the window sash shall be fitted with 5 mm thick tinted/ float glass of reputed make duly fixed with ethylene propylene di-monomer (EPDM) weathering seal resistant to tear & decay and exposure to ozone, ultra violet radiation and tropical climates and accessories like friction stay hinges of stainless steel grade 304 provided with friction adjusting device and self wiping features 2 Nos., per sash, handle with mazak alloy casting 1 No. per sash, multipoint locking system suitably concealed 1 No., per sash provided with raiser wedges for smooth operation and the system is to be installed at the site using anchor fasteners, silicon rubber sealant of approved make all-round the outer frame from inside and outside to absorb and buffer expansion of wall, the construction of window system shall have a suitable water drainage facility and shall allow easy glazing/ deglazing at site etc., including cost and conveyance of all materials, accessories, labour charges for transportation, erection at site with templates for casement sizing complete for finished item of work (Guarantee for all manufacturing defects including accessories but excluding Glass panes) |  |  |  |  |  |  |  |
| 899 | A) CASEMENT WINDOW - Single Glazed - Tinted Sash |  |  |  |  |  | 1 Sqm | 5485.00 |
| 900 | B) CASEMENT WINDOW - Single Glazed - Plain Sash |  |  |  |  |  | 1 Sqm | 5235.00 |
| 901 | C) CASEMENT WINDOW - Double Glazed - Tinted Sash |  |  |  |  |  | 1 Sqm | 7085.00 |
| 902 | D) CASEMENT WINDOW - Double Glazed - Plain Sash |  |  |  |  |  | 1 Sqm | 6800.00 |
|  | Supplying and fixing of Unplasticised Poly Vinyl Chloride (UPVC) Curtain Walling systems duly manufactured using UPVC reinforced profiles of 60 mm x 56 mm x 2.5 mm for outer frames, 60 mm x 70 mm x 2.5 mm for casement shutter frames and the mullion sections are of 60 mm x 70 mm x 2.5 mm or 60 mm x 62 mm x 2.5 mm for sliding shutter outer frames and 77 mm x 38 mm x 2.5 mm for sliding shutter frames wherever necessary the profiles shall be extruded online with properties of impact resistance, UV Stability, corrosion resistance, colour fastness, chemical resistance, low thermal expansion, fire retarding, aging resistant, sea water resistant, capable of mounting single/ double glazing system as per requirements, structurally reinforced with galvanized, hot dip galvanized up to 50 microns suitable passivated for corrosion resistance for tropical deployment of minimum thickness of 1 mm and shall not be exposed to any drainage routing, prefabricated & welded through fusion welding through fusion welding and assembled in factory, the window sash shall be fitted with 5 mm thick tinted/ float glass of reputed make duly fixed with ethylene propylene di-monomer (EPDM) weathering seal resistant to tear & decay and exposure to ozone, ultra violet radiation and tropical climates and accessories for casement |  |  |  |  |  |  |  |
|  | window - friction stay hinges of stainless steel grade 304 provided with friction adjusting device and self wiping features 2 Nos., per sash, handle with mazak alloy casting 1 No. per sash, multipoint locking system suitably concealed 1 No., per sash provided with raiser wedges for smooth operation for sliding window - clipping locking system 1 No. made of aluminium per set of sashes and the system is to be installed at the site using anchor fasteners, silicon rubber sealant of approved make all-round the outer frame from inside and outside to absorb and buffer expansion of wall, the construction of window system shall have a suitable water drainage facility and shall allow easy glazing/ deglazing at site etc., including cost and conveyance of all materials, accessories, labour charges for transportation, erection at site with templates for casement sizing complete for finished item of work (Guarantee for all manufacturing defects including accessories but excluding Glass panes) |  |  |  |  |  |  |  |
| 903 | A)CURTAIN WALLING-Single Glazed-Tinted Sash |  |  |  |  |  | 1 Sqm | 5360.00 |
| 904 | B)CURTAIN WALLING-Single Glazed - Plain Sash |  |  |  |  |  | 1 Sqm | 5125.00 |
| 905 | C)CURTAIN WALLING-Double Glazed-Tinted Sash |  |  |  |  |  | 1 Sqm | 7570.00 |
| 906 | D) CURTAIN WALLING-Double Glazed-Plain Sash |  |  |  |  |  | 1 Sqm | 6990.00 |
|  | Supply of (UPVC ) Un-Plasticized Poly Vinyl Chloride Doors with single/ double shutters and door frame made of hallow extruded (UPVC ) Plasticize poly Vinyl Chloride Profiles Suitable for Tropical deployment , the profiles are with multi chambered Constructions, the nominal thickness of outer wall is 2.5mm and the door frame sizes of 60 mm x 56 mm x 2.5mm duly reinforced with seasoned wood plank at the door hinges and the door frame top 2 courses shall be welded through fusion welding and assembled in factory inclusive of Unplasticised Poly Vinyl Chloride (UPVC) Door shutter frames duly manufactured using UPVC reinforced profiles of 60 mm x 110 mm x 2.5 mm for casement shutter frames and the mullion sections are of 60 mm x 70 mm x 2.5 mm the profiles shall be extruded online with properties of impact resistance, UV Stability, corrosion resistance, colour fastness, chemical resistance, low thermal expansion, fire retarding, aging resistant, sea water resistant, capable of mounting single/ double glazing system as per requirements, structurally reinforced with galvanized, hot dip galvanized up to 50 microns suitable passivated for corrosion resistance for tropical deployment of minimum thickness of 1 mm and shall not be exposed to any drainage routing, prefabricated & welded through fusion welding through fusion welding and assembled in factory, the door sash shall be fitted with glass and or particle board sheets of standard make duly fixed with ethylene propylene di-monomer (EPDM) weathering seal resistant to tear & decay and exposure to ozone, ultra violet radiation and tropical climates and accessories for doors are Dr. Hahn hinges to adjust sash in 3 axes, handle with mazak alloy casting 1 No. per sash, ferro locking system and key made of mild steel zinc plated with yellow passivated along with powder coated handles and mortise lock of standard make and the system is to be installed at the site using anchor fasteners, silicon rubber sealant of approved make all-round the outer frame from inside and outside to absorb and buffer expansion of wall, the construction of window system shall have a suitable water drainage facility and shall allow easy glazing/ deglazing at site etc., including cost and conveyance of all materials, accessories, labour charges for transportation, erection at site with templates for casement sizing complete for finished item of work for Swing Door or Pivot Door excluding cost Glass panes and or Particle board sheets (Guarantee for all manufacturing defects including accessories but excluding Glass panes) |  |  |  |  |  |  |  |
| 907 | DOUBLE DOOR |  |  |  |  |  | 1 Sqm | 7470.00 |
| 908 | SINGLE DOOR |  |  |  |  |  | 1 Sqm | 7180.00 |
|  | Sintex Door shutter, with shutter frame and infill panel made from PVC material confirming to IS: 10151-1982, The shutter frame made out of PVC Extruded sections having overall dimension of 47mm x 33mm with wall thickness of 1.2mm + 0.3mm with usual process variation corresponding to the Sintex code number DWUF 306. The shutter frame is given tapered shape to provide better grip to the infill panel end to drain out the water The Infill panel is made out of PVC |  |  |  |  |  |  |  |
|  | Sections having Overall Dimensions of 200mmX 20mm with wall thickness of 1mm + 0.2mm with usual process variation corresponding to the Sintex code number SPUF 101. The shutter frames shall be mitre cut and joined at the corner. Polymeric “ L” type corners shall be provided at the corners to take up hardware & fixtures and shall be fixed with 10-20mm counter sunk fully threaded parallel shank steel screws. All the hardware locations shall be reinforced with special polymeric Bars as shown in the drawings. Stickers showing locations of different Hardware shall be pasted on the shutter. The infill panels shall be joined by tongue and groove method, |  |  |  |  |  |  |  |
| 909 | WOODEN AND BP GREY COLOUR. |  |  |  |  |  | 1 Sqm | 925.00 |
|  | Door shutter made out of PVC material confirming to IS: 10151-1982. The Infill Panel is made out of seamless one piece multi chamber PVC extruded sections having overall dimensions of 610mm x 20mm with wall thickness of 1mm + 0.2mm with usual process variation corresponding to the Sintex code number SPUF 128. The top and bottom edges of the shutter to be covered with end cap of the size 20mm x 11mm with usual process variation corresponding to Sintex Code Number DWUF 327. The door shutter to be reinforced with special polymeric reinforcements as shown in the drawing to take up necessary hardware’s and fixtures. Stickers indicating the locations of hardware will be pasted at appropriate places, . |  |  |  |  |  |  |  |
| 910 | WOODEN AND BP GREY COLOUR. |  |  |  |  |  | 1 Sqm | 1100.00 |
|  | RATES OF WORKS |  |  |  |  |  |  |  |
|  | A. DISMANTLING |  |  |  |  |  |  |  |
|  | Dismantling clearing away and carefully stacking materials useful for reuse. |  |  |  |  |  |  |  |
| 911 | Brick or stone masonry in clay up to 3 M height |  |  |  |  |  | 1 Cum | 75.00 |
| 912 | Brick or stone masonry in clay over 3 M height |  |  |  |  |  | 1 Cum | 90.00 |
| 913 | Brick or stone masonry in lime motor or brick masonry in cement mortar up to 3 M height |  |  |  |  |  | 1 Cum | 128.00 |
| 914 | Brick in lime mortar wall up to 3 M height |  |  |  |  |  | 1 Cum | 75.00 |
| 915 | Brick or stone masonry in lime motor or brick masonry in cement mortar over 3 M height |  |  |  |  |  | 1 Cum | 128.00 |
| 916 | Brick in lime mortar wall up to 3 M height |  |  |  |  |  | 1 Cum | 75.00 |
| 917 | Stone masonry in cement mortar up to 3 M height |  |  |  |  |  | 1 Cum | 181.00 |
| 918 | Stone masonry in cement mortar over 3 M height |  |  |  |  |  | 1 Cum | 217.00 |
| 919 | Terracing work in roofs or floors |  |  |  |  |  | 10 Sqm | 90.00 |
| 920 | Flat stones in roof or floors including lifting |  |  |  |  |  | 10 Sqm | 75.00 |
| 921 | Pan tiled or Mangalore tiled roof without roof timbers. |  |  |  |  |  | 10 Sqm | 68.00 |
| 922 | Flat and pan tiles or Mangalore tiles over flat tiles without roof timbers |  |  |  |  |  | 10 Sqm | 75.00 |
| 923 | Old thatched roofing including tying materials into small bundles for reuse. |  |  |  |  |  | 10 Sqm | 23.00 |
| 924 | Wrought and framed timber in roofs or floors |  |  |  |  |  | 1 Cum | 113.00 |
| 925 | Doors and windows including removal of frame, hinges and fastening. |  |  |  |  |  | 1 Sqm | 38.00 |
| 926 | Old lime mortar plaster |  |  |  |  |  | 10 Sqm | 23.00 |
| 927 | Old cement mortar plaster |  |  |  |  |  | 10 Sqm | 26.00 |
| 928 | Dry rough stone revetment for aprons and stacking within 40 M lead |  |  |  |  |  | 1 Cum | 36.00 |
| 929 | Revetment for aprons and stacking grouted within 40 lead |  |  |  |  |  | 1 Cum | 26.00 |
| 930 | Lime concrete |  |  |  |  |  | 1 Cum | 120.00 |
| 931 | Surki or cement concrete |  |  |  |  |  | 1 Cum | 187.00 |
| 932 | Kadapa slabs or Shahabad stone slabs flooring on sand bed |  |  |  |  |  | 10 Sqm | 38.00 |
| 933 | reinforced cement concrete |  |  |  |  |  | 1 Cum | 1103.00 |
| 934 | Through scraping of old plastered surface |  |  |  |  |  | 10 Sqm | 23.00 |
| 935 | Washing of plastered surface with soap, soda and water (or with soda lime and water) |  |  |  |  |  | 10 Sqm | 8.00 |
| 936 | clean removal of lime plaster from walls and raking out joints 20mm deep or from terraced roof and raking out joints 100 mm deep |  |  |  |  |  | 10 Sqm | 24.00 |
| 937 | Clean removal of cement plaster from walls and raking out joint 200 mm deep |  |  |  |  |  | 10 Sqm | 25.00 |
|  | Bailing out water for earth work excavation /concrete for foundation below water level. (For building items only) |  |  |  |  |  |  |  |
|  | a) other than sandy soil |  |  |  |  |  |  |  |
| 938 | 0.0 M to 1.0 M |  |  |  |  |  | 1 cum | 100.00 |
| 939 | 1.0 M to 2.0 M |  |  |  |  |  | 1 cum | 118.00 |
| 940 | 2.0 M to 3.0 M |  |  |  |  |  | 1 cum | 138.00 |
|  | b) Sandy soil |  |  |  |  |  |  |  |
| 941 | 0.0 M to 1.0 M |  |  |  |  |  | 1 cum | 159.00 |
| 942 | 1.0 M to 2.0 M |  |  |  |  |  | 1 cum | 246.00 |
| 943 | 2.0 M to 3.0 M |  |  |  |  |  | 1 cum | 298.00 |
|  | (E) ORDINARY CONCRETE as per IS - 456 |  |  |  |  |  |  |  |
|  | Supply and placing of the Ready Mix Concrete as per IS-4926:2003 of following grades with minimum cement content as per IS-456:2000 and Standard Mix Design as per IS-10262:1982 with coarse aggregate of nominal size 20 mm and fine aggregate of Zone-II as per the Mix Design approved by the Department from standard suppliers including cost and conveyance, vertical pumping up to 5 floors/ horizontal pumping up to 60.0 M for buildings/ pavements etc., works for basic cost excluding leveling, grading, spreading, curing, centering, vibrating, compacting, finishing etc as required |  |  |  |  |  |  |  |
| 944 | M - 10 (CEMENT CONTENT - 220 Kgs) |  |  |  |  |  | 1 Cum | 3045.00 |
| 945 | M - 15 (CEMENT CONTENT - 280 Kgs) |  |  |  |  |  | 1 Cum | 3445.00 |
| 946 | M - 20 (CEMENT CONTENT - 300 Kgs) |  |  |  |  |  | 1 Cum | 3528.00 |
|  | Note: Variation in cement content for mixes other than specified above has to be adjusted in the rates as per the corresponding approved rate of Cement per Kg. |  |  |  |  |  |  |  |
| 947 | Extra rate for pumping for each additional floor above 5 Floors / for each 12.0 M additional length over 60.0 M for horizontal pumping to be added in the datas |  |  |  |  |  | 1 cum | 37.00 |
| 948 | Extra rate for pumping with Boom Pump up to 24.0 M vertical height from average ground level |  |  |  |  |  | 1 cum | 138.00 |
|  | For Standard Concrete and High Strength Concretes vide IS-456, rates may be arrived as per the design mixes. |  |  |  |  |  |  |  |
|  | (F) labour charges for wrought and put up including fixing in position frames, shutters for doors, windows, ventilators etc. |  |  |  |  |  |  |  |
| 949 | Fully paneled doors external or internal |  |  |  |  |  | 1 Sqm | 473.00 |
| 950 | Glazed and paneled doors |  |  |  |  |  | 1 Sqm | 457.00 |
| 951 | Framed and planked doors |  |  |  |  |  | 1 Sqm | 447.00 |
| 952 | Ledged, braced and planked doors, |  |  |  |  |  | 1 Sqm | 383.00 |
| 953 | Windows glazed |  |  |  |  |  | 1 Sqm | 457.00 |
| 954 | Windows paneled |  |  |  |  |  | 1 Sqm | 601.00 |
| 955 | Windows ledged, braced and planked |  |  |  |  |  | 1 Sqm | 383.00 |
| 956 | Windows framed and planked |  |  |  |  |  | 1 Sqm | 364.00 |
| 957 | Ventilators glazed, fixed with double frame with wire netting. |  |  |  |  |  | 1 Sqm | 457.00 |
| 958 | Swing ventilators glazed |  |  |  |  |  | 1 Sqm | 492.00 |
| 959 | Weld mesh doors and windows |  |  |  |  |  | 1 Sqm | 383.00 |
| 960 | G.I or .AC sheet door & windows |  |  |  |  |  | 1 Sqm | 193.00 |
| 961 | Flush door shutters of any thickness including wrought & put up of wooden door frames, fixing the door shutter to the frame, fixing the fixtures to the door shutter, etc complete |  |  |  |  |  | 1 Sqm | 383.00 |
| 962 | Flush door shutters of any thickness fixing the door shutter to the frame, fixing the fixtures to the door shutter |  |  |  |  |  | 1 Sqm | 288.00 |
|  | (G) MISCELLANEOUS ITEMS |  |  |  |  |  |  |  |
| 963 | Preparing gabions with jungle wood |  |  |  |  |  | Each | 3.00 |
| 964 | Planting plants |  |  |  |  |  | Each | 2.00 |
| 965 | Pruning plants |  |  |  |  |  | Each | 2.00 |
| 966 | Maintaining avenues including fencing, weeding, mulching, watering etc per plant. |  |  |  |  |  | Each | 6.00 |
| 967 | - do - When scattered in hot weather with extra conveyance from March to June |  |  |  |  |  | Each | 7.00 |
| 968 | Hoisting or lowering of slab |  |  |  |  |  | 1 cum | 92.00 |
| 969 | Dressing faces of granite stone (double line dressing) |  |  |  |  |  | 1 Sqm | 152.00 |
| 970 | Dressing faces of other than granite stone(double line dressing) |  |  |  |  |  | 1 Sqm | 95.00 |
|  | Expansion Joint fillers |  |  |  |  |  |  |  |
|  | SUPPLY OF HIGH PERFORMANCE EXPANSION JOINT FILLER BOARD. CONFORMING TO M.O.S.T "SILFLEX" CAPCELL HD-100 |  |  |  |  |  |  |  |
| 971 | A. 18MM THICK |  |  |  |  |  | 1 Sqm | 653.00 |
| 972 | B. 20MM THICK |  |  |  |  |  | 1 Sqm | 732.00 |
| 973 | C. 25MM THICK |  |  |  |  |  | 1 Sqm | 915.00 |
|  | SUPPLY OF EXPANSION JOINT FILLER BOARD FOR BUILDINGS, COLUMNS, BEAMS AND SLABS "ARMOUR BOARD" SILFILL |  |  |  |  |  |  |  |
| 974 | A. 20MM THICK |  |  |  |  |  | 1 Sqm | 262.00 |
| 975 | B. 25MM THICK |  |  |  |  |  | 1 Sqm | 327.00 |
| 976 | C. 50MM THICK |  |  |  |  |  | 1 Sqm | 653.00 |
|  | SUPPLY OF BACK UP TO COLD APPLIED SEALANT IN CONSTRUCTION AND LONGITUDINAL JOINT BACK UP ROAD SILSEAL |  |  |  |  |  |  |  |
| 977 | A. 10MM OD |  |  |  |  |  | 1 RM | 10.00 |
| 978 | B. 12MM OD |  |  |  |  |  | 1 RM | 12.00 |
| 979 | C. 15MM OD |  |  |  |  |  | 1 RM | 15.00 |
| 980 | D. 20MM OD |  |  |  |  |  | 1 RM | 21.00 |
| 981 | E. 25MM OD |  |  |  |  |  | 1 RM | 26.00 |
|  | (H) SCAFFOLDING |  |  |  |  |  |  |  |
| 982 | For superstructure in first floor over the rate of foundation and basement |  |  |  |  |  | 1 Sqm | 40.00 |
| 983 | For second floor over the rate of first floor superstructure |  |  |  |  |  | 1 Sqm | 48.00 |
| 984 | For third floor over the rate of II nd floor superstructure |  |  |  |  |  | 1 Sqm | 65.00 |
| 985 | For each additional floor over third floor |  |  |  |  |  | 1 Sqm | 29.00 |
|  | Scaffolding for plastering of walls and ceiling or painting of roof timber or walls, where height is more than 3 M |  |  |  |  |  |  |  |
| 986 | 1st and 2nd floor |  |  |  |  |  | 10 Sqm | 44.00 |
| 987 | 2nd and 3rd floor |  |  |  |  |  | 10 Sqm | 88.00 |
| 988 | 3rd and 4th floor |  |  |  |  |  | 10 Sqm | 132.00 |
| 989 | Extra for every additional floor over 4th floor |  |  |  |  |  | 10 Sqm | 32.00 |
|  | Centering charges |  |  |  |  |  |  |  |
| 990 | Centering Charges for RCM/ Facia Railing 50 to 75 mm thickness |  |  |  |  |  | 1 Sqm | 102.00 |
|  | Providing Centering, shuttering and scaffolding with props and steel plate as per the approved shuttering plan and other accessories as per the norms and stability calculations including cost and conveyance of all accessories etc complete for finished item of work |  |  |  |  |  |  |  |
| 991 | A. FOOTINGS, Templates, Bed Blocks |  |  |  |  |  | 1 cum | 715.00 |
| 992 | B. PEDESTALS |  |  |  |  |  | 1 cum | 963.00 |
| 993 | C. PLINTH BEAMS, LINTELS of any span |  |  |  |  |  | 1 cum | 1071.00 |
| 994 | D. Using Cashewrina Ballies and Wooden runners & staging including all bracings, cross members etc., complete for following items of work: |  |  |  |  |  |  |  |
|  | ITEM OF WORK | Up to |  | Height of the scaffolding to support structure (unsupported height of Structure from Ground Level) |  |  |  |  |
|  |  | Unit |  | 3.66 M | 4.27 M | 4.88 M | 5.49 M | 7.32 M |
| 995 | CHAJJAS of any thick | 1 sqm |  | 138.00 | 156.00 | 174.00 | 211.00 | 266.00 |
| 996 | COLUMNS | 1 cum |  | 1587.00 | 1798.00 | 2010.00 | 2433.00 | 3067.00 |
| 997 | BEAMS (Rectangular, L & T beams) | 1 cum |  | 1509.00 | 1710.00 | 1912.00 | 2314.00 | 2918.00 |
| 998 | SLABS up to 150 mm | 1 sqm |  | 169.00 | 192.00 | 215.00 | 260.00 | 328.00 |
| 999 | SLABS above 150 mm | 1 sqm |  | 209.00 | 236.00 | 264.00 | 320.00 | 403.00 |
| 1000 | SLABS above 300 mm | 1 cum |  | 543.00 | 615.00 | 688.00 | 832.00 | 1049.00 |
| 1001 | GRID/ WAFFLE SLABS | 1 sqm |  | 266.00 | 301.00 | 337.00 | 408.00 | 514.00 |
| 1002 | HELICAL STAIRCASE | 1 sqm |  | 317.00 | 359.00 |  |  |  |
| 1003 | ARCHES up to 1.5 M (Beams & Lintels) | 1 cum |  | 2573.00 | 2916.00 | 3259.00 | 3945.00 | 4975.00 |
| 1004 | Above 1.5 m Span (Beams & Lintels) | 1 cum |  | 2611.00 | 2959.00 | 3307.00 | 4003.00 | 5047.00 |
| 1005 | DOMES of any Span | 1 cum |  | 2774.00 | 3144.00 | 3514.00 | 4253.00 | 5363.00 |
|  | E. Using Adjustable Jack screw Props and Acro span beams including all pipe bracings, clamps, couplings, pre-fabricated steel "H" frames, cross members etc., complete for following items of work: |  |  |  |  |  |  |  |
|  | ITEM OF WORK |  | Up to | Height of the scaffolding to support structure (Unsupported height of Structure from Ground Level) |  |  |  |  |
|  |  |  | Unit | 3.66 M | 4.27 M | 4.88 M | 5.49 M | 7.32 M |
| 1006 | CHAJJAS of any thick |  | 1 sqm | 220.00 | 238.00 | 257.00 | 293.00 | 348.00 |
| 1007 | COLUMNS |  | 1 cum | 1587.00 | 1798.00 | 2010.00 | 2433.00 | 3067.00 |
| 1008 | BEAMS (Rectangular, L & T beams) |  | 1 cum | 2415.00 | 2616.00 | 2817.00 | 3219.00 | 3823.00 |
| 1009 | SLABS up to 150 mm |  | 1 sqm | 271.00 | 294.00 | 316.00 | 362.00 | 429.00 |
| 1010 | SLABS above 150 mm |  | 1 sqm | 334.00 | 362.00 | 389.00 | 445.00 | 528.00 |
| 1011 | SLABS above 300 mm |  | 1 cum | 868.00 | 941.00 | 1013.00 | 1158.00 | 1375.00 |
| 1012 | GRID/ WAFFLE SLABS |  | 1 sqm | 426.00 | 461.00 | 497.00 | 567.00 | 674.00 |
| 1013 | HELICAL STAIRCASE |  | 1 sqm | 507.00 | 549.00 |  |  |  |
| 1014 | ARCHES up to 1.5 M (Beams & Lintels) |  | 1 cum | 4117.00 | 4460.00 | 4803.00 | 5489.00 | 6518.00 |
| 1015 | Above 1.5 m Span (Beams & Lintels) |  | 1 cum | 4177.00 | 4525.00 | 4873.00 | 5570.00 | 6614.00 |
| 1016 | DOMES of any Span |  | 1 cum | 4438.00 | 4808.00 | 5178.00 | 5918.00 | 7028.00 |
|  | Note: 1. Centering charges and relevant Specifications to be clearly mentioned in the RCC Specifications, datas and contract agreements. |  |  |  |  |  |  |  |
|  | 2. For unsupported heights more than 7.32 M, a 10% increase may be allowed over the rate for every rise of 0.61 M. |  |  |  |  |  |  |  |
| 1017 | Labour charges for making 50 to 75 mm thick RCM Paradah walls (RCM Drop walls) including tying Rabbit (chicken) wire mesh to the existing mild steel/ HYSD Steel reinforcement applying mortar lumps, finishing, plastering both faces, sponge finishing etc complete Labour charges only excluding cost of materials & steel reinforcement |  |  |  |  |  | 1 Sqm | 180.00 |
| 1018 | Labour charges for fixing A.C sheets including cost of J. Bolts bitumen washers etc. with or without ridges. |  |  |  |  |  | 10 sqm | 472.00 |
| 1019 | Labour charges for fabricating, erecting, fixing aluminium partitions, doors, windows, work-station frames including cutting, fabricating, assembling, fixing in position with necessary screws excluding cost of materials |  |  |  |  |  | 1 Sqm | 362.00 |
| 1020 | Labour charges for making, fabricating office furniture, racks, work-station furniture with ply-wood sheets, decolam/ veneer laminations including fixing cutting, making, assembling, fixing hinges, fixing fixtures, fixing locks, pasting sheets/ laminations fixing in position with necessary screws excluding cost of materials, accessories, fittings |  |  |  |  |  | 1 Sqm | 317.00 |
| 1021 | Labour charges for making, fabricating kitchen furniture, cup-boards with shutters, box racks, with ply-wood sheets, decolam/ veneer laminations including fixing cutting, making, assembling, fixing hinges, fixing fixtures, fixing locks, pasting sheets/ laminations fixing in position with necessary screws excluding cost of materials, accessories, fittings |  |  |  |  |  | 1 Sqm | 484.00 |
| 1022 | Mixing concrete as per design mix in Mechanical Weigh Batching units up to 15 cum/hr capacity |  |  |  |  |  | 1 Cum | 100.00 |
| 1023 | Mixing concrete as per design mix in Electronic Weigh Batching units up to 15 cum/hr capacity |  |  |  |  |  | 1 Cum | 120.00 |
|  | (j) LIFT CHARGES |  |  |  |  |  |  |  |
|  | For RCC items |  |  |  |  |  |  |  |
| 1024 | Between 1st and 2 nd floor |  |  |  |  |  | 1 cum | 55.00 |
| 1025 | Between 2nd and 3rd floor |  |  |  |  |  | 1 cum | 137.00 |
| 1026 | Between 3rd and 4th floor |  |  |  |  |  | 1 cum | 254.00 |
| 1027 | Between 4th and 5th floor |  |  |  |  |  | 1 cum | 400.00 |
| 1028 | Between 5th and 6th floor |  |  |  |  |  | 1 cum | 436.00 |
| 1029 | between 6ht and 7th floor |  |  |  |  |  | 1 cum | 545.00 |
| 1030 | Between 7th and 8th floor |  |  |  |  |  | 1 cum | 635.00 |
| 1031 | Between 8th and 9th floor |  |  |  |  |  | 1 cum | 708.00 |
| 1032 | Between 9th and 10th floor |  |  |  |  |  | 1 cum | 817.00 |
|  | LIFT CHARGES FOR BRICK MASONRY & STONE MASONRY |  |  |  |  |  |  |  |
| 1033 | Between 1st and 2nd floor |  |  |  |  |  | 1 cum | 46.00 |
| 1034 | Between 2nd and 3rd floor |  |  |  |  |  | 1 cum | 65.00 |
| 1035 | Between 3rd and 4th floor |  |  |  |  |  | 1 cum | 83.00 |
| 1036 | Add extra for every additional floor above 4th floor |  |  |  |  |  | 1 cum | 26.00 |
|  | Lift charges for impervious coat up to 26 mm thick plastering with impermo compound or Shahabad stone flooring: |  |  |  |  |  |  |  |
| 1037 | Between 1st and 2nd floor |  |  |  |  |  | 10 Sqm | 23.00 |
| 1038 | Between 2nd and 3rd floor |  |  |  |  |  | 10 Sqm | 41.00 |
| 1039 | Between 3rd and 4th floor |  |  |  |  |  | 10 Sqm | 60.00 |
| 1040 | Add extra for every additional floor above 4th floor |  |  |  |  |  | 10 Sqm | 20.00 |
|  | Lift charges for plastering |  |  |  |  |  |  |  |
| 1041 | Between 1st and 2nd floor |  |  |  |  |  | 10 Sqm | 11.00 |
| 1042 | Between 2nd and 3rd floor |  |  |  |  |  | 10 Sqm | 19.00 |
| 1043 | Between 3rd and 4th floor |  |  |  |  |  | 10 Sqm | 29.00 |
| 1044 | Add extra for every additional floor above 4th floor |  |  |  |  |  | 10 Sqm | 13.00 |
|  | Providing, Designing, cutting, bending and fixing aluminium composite cladding of approved make (Alucopanel, alucobonel, Reynobond, Lennox Durabuid) on external facade of size as shown with water tight system either curved or straight on concrete/ masonry surfaces skin material 0.25 mm thick aluminium sheet (300 5 H6) cover material natural polyethylene aluminium cladding panel to be of approved colour / shade fixed with extruded aluminium basis frame (50x25x1.5mm) angle cleats, weather sealants, rivets, GI brackets all as approved, using suitable chemical/ anchor bolts on structural steel work including necessary accessories complete in all respects, where level difference is shown dummy structural steel backup frame shall be provided including the finished surface shall be protected with 80 microns self adhesive peel off film with two layers of white and black tested to withstand at least 6 months exposure to local weather condition, without loosing the original peel off characteristic or causing stains or other damages with silicon sealant of Dow Corning 789 or Wacker EL 305 or GE etc., including all scaffolding and labour charges etc., complete for finished item of work |  |  |  |  |  |  |  |
| 1045 | ACP Cladding - 3 mm thk |  |  |  |  |  | 1 Sqm | 2162.00 |
| 1046 | ACP Cladding - 4 mm thk |  |  |  |  |  | 1 Sqm | 2657.00 |
|  | Providing, Designing, cutting, bending and fixing aluminium composite panel cladding of approved make (Alstrong legame, Reynobond, Alpolic, Alcomat) on external facade of size as shown with water tight system either curved or straight on concrete/ masonry surfaces the panel should be of 4 mm thick with combination of aluminium skin material of 0.5 mm thick aluminium sheet of alloy grade 3105 H 14 cover material natural polyethylene aluminium cladding panel coated on external side with PVDF (Poly Vinyledene Fluoride) of KYNAR PVDF coating containing 70% KYNAR 500 resins for anti-fungal & anti bacterial properties and the central core should be with opaque/ white polyethylene virgin grade, UV stabilized with a combination of 70% LDPE grade 22 FA 002 and 30% HDPE grade E5201and the core should be bonded to the aluminium skin on both sides with DUPONT adhesive film of 30 to 50 μ to be of approved colour / shade |  |  |  |  |  |  |  |
|  | fixed with extruded aluminium basis frame (50x25x1.5mm) angle cleats, weather sealants, rivets, GI brackets all as approved, using suitable chemical/ anchor bolts on structural steel work including necessary accessories complete in all respects, the surface protection with 80 microns self adhesive peel off film with two layers of white and black tested to withstand at least 6 months exposure to local weather condition, without loosing the original peel off characteristic or causing stains or other damages with silicon sealant of Sil Proof NB/ Wacker EL 305 or Dow Corning 789 or Wacker EL 305 or GE etc., including all scaffolding and labour charges etc., excluding cost of structural steel backup frame complete for finished item of work |  |  |  |  |  |  |  |
| 1047 | ACP Cladding - 4 mm thk |  |  |  |  |  | 1 Sqm | 3450.00 |
| Note: SSR rates for Building items are purely for guidance for the officers for preparation of estimates vide para.113 of APPWD code. Authority who prepares the estimate is however responsible for proper rates as envisaged in para.45 of APPWD code. |  |  |  |  |  |  |  |  |
| Rates: Supply Rates for all the materials are based on market rates plus Sales Tax/ purchase VAT. Works Contract VAT/ Composite tax on finished items may be allowed separately as applicable. |  |  |  |  |  |  |  |  |

###### SANITARY

###### AND

WATER SUPPLY ITEMS

(BUILDINGS)

| ###### SANITARY & WATER SUPPLY ITEMS |  |  |  |
| --- | --- | --- | --- |
| S. No | Description of item | Unit | SS RATE FOR 2008-09 |
| 1 | 2 | 3 | 4 |
|  | S.W.G. PIPES & FITTINGS |  |  |
| 1 | Supplying, laying, jointing and testing 101.6 mm SWG pipes of ISI make conforming to ISI 651 & 4127 with airtight cement joints in CM 1.5:1 prop. Including excavation of trenches and socket pits in any soil (except rock requiring blasting) up to 1524 mm(5'0") depth and refilling with watering and tamping. | 1 RM | 212.00 |
| 2 | Supplying, laying, jointing and testing 101.6 mm SWG pipes of ISI make conforming to ISI 651 & 4127 with airtight cement joints in CM 1:5:1 prop. including excavation of trenches and socket pits in any soil (except rock requiring blasting) upto 914.4 mm (3'0") depth and refilling with watering and tamping. | 1 RM | 177.00 |
| 3 | Supplying, laying, jointing and testing 152.4 mm SWG pipes of ISI make conforming to ISI 651 & 4127 with airtight cement joints in CM 1.5:1 prop. including excavation of trenches and socket pits in any soil (except rock requiring blasting) up to 1524 mm(5'0") depth and refilling with watering and tamping. | 1 RM | 301.00 |
| 4 | labour charges only for laying, jointing and testing SWG pipes of any dia with airtight cement joints including excavation of trenches and socket pits in any soil up to 1524 mm (5'-0") depth (except rock requiring blasting) and refilling with watering and tamping. | 1 RM | 89.00 |
| 5 | a)Supplying , laying, jointing and testing 152.4 mm CC bed 1:2:4 Prop: using 19.05 mm gauge graded granite metal chips and cradling 101.6 mm SWG pipe line all round to have 76.2 mm thick cover over the sockets. | 1 RM | 70.00 |
| 6 | b) - do - labour charges only | 1 RM | 7.00 |
| 7 | a)Supplying , laying, jointing and testing 152.4 mm CC bed 1:2:4 Prop: using 19.05 mm gauge graded granite metal chips and cradling 152.4 mm SWG pipe line all round to have 76.2 mm thick cover over the sockets. | 1 RM | 121.00 |
| 8 | b) - do - labour charges only | 1 RM | 35.00 |
| 9 | a) Supplying & Laying 101.6 mm SWG channels with cement mortar joints with suitable brick masonry for fixing the channels in position | 1 RM | 67.00 |
| 10 | b) - do - labour charges only | 1 RM | 24.00 |
| 11 | a) Supplying & Laying 152.4 mm SWG channels with cement mortar joints with suitable brick masonry for fixing the channels in position | 1 RM | 126.00 |
| 12 | b) - do - labour charges only | 1 RM | 34.00 |
|  | Supply of SWG pipe of ISI make conforming to ISI 651 |  |  |
| 13 | 4" dia (101.6mm) | 1 RM | 111.00 |
| 14 | 6" dia (152.4mm) | 1 RM | 191.00 |
| 15 | 8" dia (203.2mm) | 1 RM | 295.00 |
| 16 | 10" dia (254 mm) | 1 RM | 542.00 |
| 17 | 12" dia (300 mm) | 1 RM | 780.00 |
| 18 | 14" dia (350 mm) | 1 RM | 1062.00 |
|  | Supply of SWG Bends of ISI make conforming to IS standards |  |  |
| 19 | 100 mm dia | Each | 60.00 |
| 20 | 150 mm dia | Each | 111.00 |
| 21 | 200 mm dia | Each | 209.00 |
| 22 | 250 mm dia | Each | 423.00 |
| 23 | 300 mm dia | Each | 537.00 |
| 24 | 350 mm dia | Each | 671.00 |
|  | Supply of SWG Junctions (T's & Y's) dia in mm of ISI make conforming to IS standards |  |  |
|  | Length of barrel 60 cm |  |  |
| 25 | 100 x 100 | Each | 111.00 |
| 26 | 150 x 100 | Each | 165.00 |
| 27 | 150 x 150 | Each | 179.00 |
| 28 | 200 x 100 | Each | 255.00 |
| 29 | 200 x 150 | Each | 299.00 |
| 30 | 200 x 200 | Each | 343.00 |
| 31 | 250 x 100 | Each | 403.00 |
| 32 | 250 x 150 | Each | 532.00 |
| 33 | 250 x 200 | Each | 659.00 |
| 34 | 250 x 250 | Each | 788.00 |
| 35 | 300 x 100 | Each | 575.00 |
| 36 | 300 x 150 | Each | 736.00 |
| 37 | 300 x 200 | Each | 898.00 |
| 38 | 300 x 250 | Each | 1060.00 |
| 39 | 300 x 300 | Each | 1221.00 |
| 40 | 350 x 100 | Each | 718.00 |
| 41 | 350 x 150 | Each | 920.00 |
| 42 | 350 x 200 | Each | 1122.00 |
| 43 | 350 x 250 | Each | 1323.00 |
| 44 | 350 x 300 | Each | 1526.00 |
| 45 | 350 x 350 | Each | 1526.00 |
|  | Supply of SWG Plugs dia in mm of ISI make conforming to IS standards |  |  |
| 46 | 100 | Each | 22.00 |
| 47 | 150 | Each | 43.00 |
| 48 | 200 | Each | 81.00 |
| 49 | 250 | Each | 139.00 |
| 50 | 300 | Each | 221.00 |
| 51 | 350 | Each | 277.00 |
| 52 | Supplying & fixing 150 mm x 100 mm SWG gully traps of ISI make conforming to ISI 651 & 4127 with C.I Grating & Constg. Brick masonry in CM 1:6 prop. Intermediate chamber and fitted with 304.8 mm x 288.6mm (12"x9") C.I Frame and hinged cover | Each | 419.00 |
|  | Supply of SWG Gully Traps (Round & Square) of ISI make conforming to IS standards |  |  |
| 53 | 150 x 100 mm | Each | 170.00 |
| 54 | Labour charges for fixing and laying the SWG Junctions/ Plugs/ Bends of any dia with airtight cement joints as per standrad practice | Each | 18.00 |
|  | MAN HOLES & INSPECTION CHAMBERS - AS PER - IS 4111 : Part 1 : 1986 |  |  |
| 55 | Constructing 904.0 mm (3’0”) dia solid block masonry inspection chamber as per IS - 4111: Part-1:1986 with cement mortar (1:6) prop using fly-ash blocks of 225 mm thick from approved source having a minimum crushing strength of 10 N/sq.mm including plastering with cement mortar 1:3 prop; ½” thick both inside and outside fitted with 20” dia RCC manhole covers and frames including excavating pits up to a depth of 904 mm (3'-0") in all sorts of soils (exculding rock) and laying cement conrete (1:4:8) 150 mm thick using 40 mm HBG Metal and P.C.C. 1:2:4 benching and channel 100 mm thick as per Standard specification and including cost and conveyance of all materials like cement, sand, bricks, water etc., to site, cost of seigniorage charges on all materials and all incidental and operational, labour charges like mixing cement mortar, constructing masonry, lift charges, curing etc., complete for finished item of work as per Standard specification. | Each | 4648.00 |
| 56 | Constructing 904.0 mm (3’0”) dia solid block masonry inspection chamber as per IS - 4111: Part-1:1986 with cement mortar (1:6) prop using fly-ash blocks of 225 mm thick from approved source having a minimum crushing strength of 10 N/sq.mm including plastering with cement mortar 1:3 prop; ½” thick both inside and outside fitted with 20” dia RCC manhole covers and frames including excavating pits up to a depth of 1524 mm (5'-0") in all sorts of soils (exculding rock) and laying cement conrete (1:4:8) 150 mm thick using 40 mm HBG Metal and P.C.C. 1:2:4 benching and channel 100 mm thick as per Standard specification and including cost and conveyance of all materials like cement, sand, bricks, water etc., to site, cost of seigniorage charges on all materials and all incidental and operational, labour charges like mixing cement mortar, constructing masonry, lift charges, curing etc., complete for finished item of work as per Standard specification. | Each | 7225.00 |
| 57 | Constructing 904.0 mm (3’0”) dia brick masonry inspection chamber as per IS - 4111: Part-1:1986 with cement mortar (1:6) prop using 2nd Class Clay Bricks of 225 mm thick from approved source having a minimum crushing strength of 5 N/sq.mm including plastering with cement mortar 1:3 prop; ½” thick both inside and outside fitted with 20” dia RCC manhole covers and frames including excavating pits up to a depth of 904 mm (3'-0") in all sorts of soils (exculding rock) and laying cement conrete (1:4:8) 150 mm thick using 40 mm HBG Metal and P.C.C. 1:2:4 benching and channel 100 mm thick as per Standard specification and including cost and conveyance of all materials like cement, sand, bricks, water etc., to site, cost of seigniorage charges on all materials and all incidental and operational, labour charges like mixing cement mortar, constructing masonry, lift charges, curing etc., complete for finished item of work as per Standard specification. | Each | 4096.00 |
| 58 | Constructing 904.0 mm (3’0”) dia brick masonry inspection chamber as per IS - 4111: Part-1:1986 with cement mortar (1:6) prop using 2nd Class Clay Bricks of 225 mm thick from approved source having a minimum crushing strength of 5 N/sq.mm including plastering with cement mortar 1:3 prop; ½” thick both inside and outside fitted with 20” dia RCC manhole covers and frames including excavating pits up to a depth of 1524 mm (5'-0") in all sorts of soils (exculding rock) and laying cement conrete (1:4:8) 150 mm thick using 40 mm HBG Metal and P.C.C. 1:2:4 benching and channel 100 mm thick as per Standard specification and including cost and conveyance of all materials like cement, sand, bricks, water etc., to site, cost of seigniorage charges on all materials and all incidental and operational, labour charges like mixing cement mortar, constructing masonry, lift charges, curing etc., complete for finished item of work as per Standard specification. | Each | 6160.00 |
| 59 | Constructing 914.4 mm x 457.2 mm (3'0"x1'6") brick in CM 1:6 prop. Masonry Inspection chamber up to 914.4 mm (3'0") and fitted with light weight 914.4 mm x 457.2 mm (3'0"x1'6") C.I frame and cover of 40 Kg. | Each | 4355.00 |
| 60 | Constructing 457.2 mm x 457.2 mm (1'6"x1'6") brick in CM 1:6 prop. Masonry. Inspection chamber upto 914.4 mm (3'0") and fitted with light weight 457.2 mm x 457.2 mm (1'6"x1'6") C.I frame and cover of 20 Kg. | Each | 2626.00 |
| 61 | a)Making drainage connection in the existing inspection including all repairs | Each | 88.00 |
| 62 | b) - do - labour charges only | Each | 48.00 |
|  | C.I. PIPES & FITTINGS - AS PER - IS 1729:1979 |  |  |
|  | Supplying and fixing of C.I. Single/ Double Socket soil pipes ISI marked conforming to IS:1729-1979 of reputed make with cement caulked joints and fixing as per site requirements including painting black with Japan paint and fixing with necessary wooden blocks fixed in the masonry walls in cement mortar (1½:1) and including fixing of M.S. clamps if necessary with required no.of Bombay nails etc., complete |  |  |
| 63 | 3" dia (76.2 mm) Single Socket | 1 RM | 545.00 |
| 64 | 4" dia (101.6mm) Single Socket | 1 RM | 622.00 |
| 65 | 6" dia (152.4mm) Single Socket | 1 RM | 1088.00 |
| 66 | 3" dia (76.2 mm) Double Socket | 1 RM | 595.00 |
| 67 | 4" dia (101.6mm) Double Socket | 1 RM | 680.00 |
| 68 | 6" dia (152.4mm) Double Socket | 1 RM | 1149.00 |
| 69 | Labour charges only for fixing of C.I. Single/ Double Socket soil pipes of any dia with cement caulked joints and fixing as per site requirements including painting black with Japan paint and fixing with necessary wooden blocks fixed in the masonry walls in cement mortar and including fixing of M.S. clamps if necessary with required nails | 1 RM | 150.00 |
|  | Supplying & fixing CI Plain bends Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
| 70 | 3" dia (76.2 mm) | Each | 178.00 |
| 71 | 4" dia (101.6 mm) | Each | 230.00 |
| 72 | 6" dia (152.4 mm) | Each | 448.00 |
|  | Supplying & fixing CI Plug (Door) bends Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
| 73 | 3" dia (76.2 mm) | Each | 240.00 |
| 74 | 4" dia (101.6 mm) | Each | 281.00 |
| 75 | 6" dia (152.4 mm) | Each | 592.00 |
| 76 | Labour charges only for fixing CI Plain/ Plug (Door) bends with cement caulked joints and fixing as per site requirements with standard practice | Each | 48.00 |
|  | Supplying & fixing CI soil single junctions Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
| 77 | 76.2 mm (3") CI soil single junction with plug | Each | 322.00 |
| 78 | 101.6 mm (4") CI soil single junction with plug | Each | 439.00 |
| 79 | 152.4 mm (6") CI soil single junction with plug | Each | 1025.00 |
| 80 | 76.2 mm (3") CI soil plain single junction | Each | 278.00 |
| 81 | 101.6 mm (4") CI soil plain single junction | Each | 381.00 |
| 82 | 152.4 mm (6") CI soil plain single junction | Each | 915.00 |
| 83 | Labour charges only fixing CI soil single junctions with cement caulked joints and fixing as per site requirements with standard practice | Each | 63.00 |
|  | Supplying & fixing CI soil Double junctions Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
| 84 | 76.2 mm (3") CI soil Double junction with plug | Each | 501.00 |
| 85 | 101.6 mm (4") CI soil Double junction with plug | Each | 580.00 |
| 86 | 152.4 mm (6") CI soil Double junction with plug | Each | 1432.00 |
| 87 | 76.2 mm (3") CI soil plain Double junction | Each | 436.00 |
| 88 | 101.6 mm (4") CI soil plain Double junction | Each | 538.00 |
| 89 | 152.4 mm (6") CI soil plain Double junction | Each | 1342.00 |
| 90 | Labour charges only fixing CI soil Double junctions with cement caulked joints and fixing as per site requirements with standard practice | Each | 58.00 |
|  | Supplying & fixing CI soil Offsets Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
|  | 75 mm Offset |  |  |
| 91 | for 76.20 mm (3") C.I. Pipe | Each | 214.00 |
| 92 | for 101.6 mm (4") C.I. Pipe | Each | 299.00 |
| 93 | for 152.4 mm (6") C.I. Pipe | Each | 651.00 |
|  | 115 mm Offset | Each |  |
| 94 | for 76.20 mm (3") C.I. Pipe | Each | 254.00 |
| 95 | for 101.6 mm (4") C.I. Pipe | Each | 339.00 |
| 96 | for 152.4 mm (6") C.I. Pipe | Each | 667.00 |
|  | 150 mm Offset | Each |  |
| 97 | for 76.20 mm (3") C.I. Pipe | Each | 294.00 |
| 98 | for 101.6 mm (4") C.I. Pipe | Each | 381.00 |
| 99 | for 152.4 mm (6") C.I. Pipe | Each | 682.00 |
| 100 | Labour charges only fixing CI soil Offsets with cement caulked joints and fixing as per site requirements with standard practice | Each | 29.00 |
|  | Supplying & fixing CI floor traps 1st quality ISI marked conforming to IS:1729-1979 with C.P. Grating fixing with white cement as per site requirements with standard practice |  |  |
| 101 | 100 mm dia Inlet- 75 mm (3") outlet pipe | Each | 291.00 |
| 102 | 100 mm dia Inlet - 100 mm (4") outlet pipe | Each | 348.00 |
|  | Supplying & fixing CI Nahani traps 1st quality ISI marked conforming to IS:1729-1979 with C.P. Grating fixing with white cement as per site requirements with standard practice | Each |  |
| 103 | 100 mm dia Inlet- 75 mm (3") outlet pipe - 4 Kgs | Each | 236.00 |
| 104 | Labour charges only for fixing CI Nahani/ Floor traps with C.P. Grating fixing with white cement as per site requirements with standard practice | Each | 38.00 |
| 105 | Boxing of Nahany traps in RCC floors | Each | 123.00 |
| 106 | Boxing of W.C Trap in R.C.C floors | Each | 247.00 |
|  | Supplying & fixing CI Collars (double socket) Ist quality ISI marked conforming to IS:1729-1979 with cement caulked joints and fixing as per site requirements with standard practice |  |  |
| 107 | for 76.20 mm (3") C.I. Pipe | Each | 130.00 |
| 108 | for 101.6 mm (4") C.I. Pipe | Each | 163.00 |
| 109 | for 152.4 mm (6") C.I. Pipe | Each | 468.00 |
|  | VITREOUS CHINAWARE - SANITARY APPLIANCES as per IS:2556 - 1974 |  |  |
|  | Supplying & Fixing Indian pattern white glazed W.C 1st quality ISI marked conforming to IS:2556-Part-3-1981 with "P" or "S" trap Hindware/ Parryware/ Neycer - ISI Mark |  |  |
| 110 | 580 mm Long | Each | 698.00 |
| 111 | 630 mm Long | Each | 698.00 |
| 112 | Labour charges only for fixing Indian pattern W.C with "P" or "S" trap | Each | 175.00 |
|  | Supplying & Fixing Orissa Pan white glazed W.C 1st quality ISI marked conforming to IS:2556-Part-3-1981 with "P" or "S" trap Hindware/ Parryware/ Neycer - ISI Mark | Each |  |
| 113 | 580 mm x 440 mm | Each | 820.00 |
| 114 | 630 mm x 450 mm | Each | 900.00 |
| 115 | Labour charges only Fixing Orissa pan W.C with "P" or "S" trap | Each | 219.00 |
| 116 | Supplying & Fixing porcelain foot rest for squatting I.W.C. (Rectangular pair) of sizes 250 mm x 125 mm x 15 mm thick each foot rest 1st quality ISI marked conforming to IS:2556-Part-10:1974 | Pair | 153.00 |
| 117 | Labour charges only for fixing porcelain foot rest for squatting I.W.C. (Rectangular pair) | Pair | 40.00 |
| 118 | Providing Brick masonry seat for Orissa pan/ Indian W.C. using 2nd class bricks as per standard practice | Each | 198.00 |
| 119 | Providing Cement Concrete Squatting plate for Orissa pan/ Indian W.C. in P.C.C. 1:2:4 as per standard practice | Each | 60.00 |
| 120 | Supplying & Fixing low down porcelain flushing tank 10 Lit Capacity Hindustan Sanitary ware/ parry/ Neycer with internal components | Each | 1670.00 |
| 121 | Labour charges only for fixing low down porcelain flushing tank with internal components | Each | 318.00 |
| 122 | Supplying and Fixing European Water Closet of 1st quality conforming to IS:2556-Part-2-1973 of Hindustan / Neycer or Parryware make white glazed with 'P' trap | Each | 1099.00 |
| 123 | Supplying and Fixing European Water Closet of 1st quality conforming to IS:2556-Part-2-1973 of Hindustan / Neycer or Parryware make white glazed with 'S' trap | Each | 1152.00 |
| 124 | Labour charges only for fixing European Water Closet with 'P' or 'S' trap as per standard practice | Each | 159.00 |
| 125 | Supplying and Fixing best Indian make plastic seat and lid for European water closets with rubber or plastic Buffers as per IS 2548-1996 | Each | 339.00 |
| 126 | Labour charges only for fixing plastic seat and lid for European water closet | Each | 48.00 |
|  | Supply, Installation and commissioning approved make floor mounted EWC (suit) CASCADE model 'P' or 'S' trap with dual flush porcelain cistern fixed on EWC with all internal parts of dual flush cistern, ultra solid seat cover of approved make with rubber buffer and cap 15 mm angle stop cock 450 mm long PVC inter connection pipe wall flanges all of approved make etc. complete for finished item of work in all respects |  |  |
| 127 | a) white | Each | 5996.00 |
| 128 | b) Extra for colour - Grade - I/ Grade - II | Each | 2625.00 |
| 129 | c) Extra for spl. Colour - Grade - III | Each | 5024.00 |
|  | Supplying & Fixing Indian make Flat Back Wash Hand Basin (HSW/Parryware/ Neycer) 1st quality conforming to IS:2556-Part-4:1972 with waste fittings like rubber plug, chain, 32 mm nominal size C.P. Fitting with parallel pipe thread conforming to IS:2963-1979 and fitted with 15 mm nominal bore Chromium Plated Pillar Tap of 1st quality Indian make 300 grams Seiko/ Esso or equivalent complete with standard CI brackets including wooden block |  |  |
| 130 | 660 x 460 mm - Single C.P. Pillar cock | Each | 1505.00 |
| 131 | 630 x 450 mm - Single C.P. Pillar cock | Each | 1314.00 |
| 132 | 630 x 450 mm - Double C.P. Pillar cock | Each | 1465.00 |
| 133 | 550 x 400 mm - Single C.P. Pillar cock | Each | 905.00 |
| 134 | 550 x 400 mm - Double C.P. Pillar cock | Each | 1055.00 |
| 135 | 450 x 300 mm - Single C.P. Pillar cock | Each | 772.00 |
| 136 | 450 x 300 mm - Double C.P. Pillar cock | Each | 923.00 |
| 137 | Labour charges only for fixing Indian make Flat Back Wash Hand Basin with waste fittings like rubber plug, chain, 32 mm nominal size C.P. Fitting Pillar Taps complete with standard CI brackets including wooden block | Each | 223.00 |
| 138 | Supplying and Fixing Pedestal for Wash Hand Basin (H.S.W / Parry / Earthenware) | Each | 803.00 |
| 139 | Labour charges only for fixing pedestal for wash hand basin | Each | 80.00 |
| 140 | Supplying & Fixing 31.75 mm C.P bottle trap (Heavy type) Parryware or equivalent | Each | 315.00 |
| 141 | Labour charges only for fixing bottle trap to the wash hand basin | Each | 31.00 |
| 142 | Supplying & Fixing white glazed flat back Bowl urinals of size 440 mm x 265 mm x 315 mm with internal flushing rim fixed with screws complete Indian make (HSW/Parry/Neycer) conforming to IS:2556-1995 | Each | 577.00 |
|  | Supplying & Fixing white glazed flat back half stall urinals 1st quality conforming to IS:2556-1995 with standard C.P. Spreader of overall size 95 x 95 x 57 mm conforming to IS:2556-Part6, Sec-6-1974 fixed with screws complete Indian make (HSW/Parry/Neycer) |  |  |
| 143 | 410 x 265 x 305 mm | Each | 1736.00 |
| 144 | 590 x 375 x 390 mm | Each | 1962.00 |
| 145 | Labour charges only Fixing flat back Bowl/ Half stall urinals with fixing C.P. Spreader and screws complete | Each | 88.00 |
|  | Supplying & Fixing Indian make white glazed vitreous china porcelain sink conforming to IS:2556-Part-5-1994 on cantilever brackets with waste fittings like rubber plug, chain, 32 mm nominal size C.P. Fitting with parallel pipe thread conforming to IS:2963-1979 |  |  |
| 146 | 600 x 400 x 250 | Each | 2800.00 |
| 147 | 750 x 450 x 250 | Each | 3248.00 |
| 148 | Labour charges only for fixing Vitreous china porcelain sinks with waste fittings, rubber plug, chain, cantilever brackets etc. | Each | 239.00 |
| 149 | a) S & F 609.6 x 457.2 x 254 mm RCC terrazzo finished sink (or constructed at site with 50.8mm thick) brass plug and chain including CI cantilever brackets | Each | 504.00 |
| 150 | b) - do - labour charges only | Each | 159.00 |
|  | TAPS & FAUCETS |  |  |
| 151 | a) S & F 12.7 mm NP stop cock Indian make 300 grams Seiko or equivalent | Each | 178.00 |
| 152 | b) - do - labour charges only | Each | 17.00 |
| 153 | a) S & F concealed stop cock 12.7 mm (1/2") Indian Make Seiko or equivalent | Each | 245.00 |
| 154 | b) - do - labour charges only | Each | 21.00 |
| 155 | a) S & F angle stop cock 12.7 mm dia first quality Indian Make Seiko or equivalent | Each | 178.00 |
| 156 | b) - do - labour charges only | Each | 21.00 |
| 157 | a) S&F 12.7 mm N.P bib tap Indian make 300 grams Seiko or equivalent | Each | 178.00 |
| 158 | b) - do - labour charges only | Each | 17.00 |
| 159 | a) S&F 12.7mm N.P bib tap Indian make 250 grams Seiko or equivalent | Each | 138.00 |
| 160 | b) - do - labour charges only | Each | 15.00 |
| 161 | a)S & F 31.75 mm brass plumber Union best Indian first quality | Each | 36.00 |
| 162 | b) - do - labour charges only | Each | 7.00 |
| 163 | a) S & F 12.7 mm brass stop cock Indian make 400 grams | Each | 132.00 |
| 164 | b) - do - labour charges only | Each | 15.00 |
| 165 | a) S & F 12.7 mm brass stop cock Indian make 300 grams | Each | 127.00 |
| 166 | b) - do - labour charges only | Each | 15.00 |
| 167 | a) S & F 12.7 mm brass bib cock Indian make 400 grams | Each | 143.00 |
| 168 | b) - do - labour charges only | Each | 15.00 |
| 169 | a) S & F 12.7 mm brass bib cock Indian make 300 grams | Each | 132.00 |
| 170 | b) - do - labour charges only | Each | 16.00 |
| 171 | S & F CP long body bib cock fancy type deluxe 300 grams dia 12.7 mm Indian make Seiko or equivalent | Each | 270.00 |
| 172 | S & F 12.7mm dia high neck pillar cock 1st quality Indian make 400 grams | Each | 398.00 |
| 173 | a) S & F 12.7 mm NP elbow action bib cock 600 grams | Each | 388.00 |
| 174 | b) - do - labour charges only | Each | 25.00 |
| 175 | a) S&F 12.7 m NP elbow action pillar cock 600 grams | Each | 388.00 |
| 176 | b) - do - labour charges only | Each | 22.00 |
| 177 | S&F 12.7 mm dia NP pillar cock Ist quality Indian make Seiko or equivalent | Each | 204.00 |
| 178 | S&F 12.7 mm dia NP push cock Ist quality | Each | 179.00 |
| 179 | S&F 12.7 mm dia NP lift cock Ist quality | Each | 143.00 |
| 180 | a)S & F 38.1 mm C.P waste coupling Half or Full Thread 1st Quality Indian make Parryware or equivalent | Each | 150.00 |
| 181 | b) - do - labour charges only | Each | 17.00 |
| 182 | a) S & F 31.75 mm PVC bottle trap | Each | 105.00 |
| 183 | b) - do - labour charges only | Each | 15.00 |
| 184 | a) S& F 38.1 mm brass plumber union | Each | 48.00 |
| 185 | b) - do - labour charges only | Each | 7.00 |
| 186 | a)S & F 19.05 mm NP stop cock Indian make with ISI mark 400 grams | Each | 224.00 |
| 187 | b) - do - labour charges only | Each | 15.00 |
|  | G.I. PIPES & FITTINGS |  |  |
| 188 | S& F GI union 12.7 mm Ist Quality | Each | 29.00 |
| 189 | S& F GI union 19.05 mm Ist Quality | Each | 35.00 |
| 190 | S& F GI union 25.4 mm Ist Quality | Each | 53.00 |
| 191 | S& F GI union 31.75 mm Ist Quality | Each | 66.00 |
| 192 | S& F GI union 38.1 mm Ist Quality | Each | 79.00 |
| 193 | S& F GI union 50.8 mm Ist Quality | Each | 97.00 |
| 194 | S& F GI connector 25.4 mm dia | Each | 58.00 |
| 195 | S& F 12.7 m GI 'T' A class Ist quality | Each | 36.00 |
| 196 | S& F 31.75 mm dia GI bend Ist quality | Each | 53.00 |
| 197 | S & F 31.75 mm dia GI 'T' 1st Quality | Each | 52.00 |
| 198 | a) S & F 25.4 mm copper ball cock ith PVC ball Indian make ISI mark | Each | 303.00 |
| 199 | b) - do - labour charges only | Each | 31.00 |
| 200 | a) S & F 19.05 mm copper ball cock ith PVC ball Indian make ISI mark | Each | 248.00 |
| 201 | b) - do - labour charges only | Each | 29.00 |
| 202 | a)S & F 12.7 mm copper ball cock ith PVC ball Indian make ISI mark | Each | 160.00 |
| 203 | b) - do - labour charges only | Each | 31.00 |
| 204 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 25 mm NB Size | Each | 530.00 |
| 205 | b) - do - labour charges only | Each | 31.00 |
| 206 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 20 mm NB Size | Each | 360.00 |
| 207 | b) - do - labour charges only | Each | 31.00 |
| 208 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 15 mm NB Size | Each | 266.00 |
| 209 | b) - do - labour charges only | Each | 31.00 |
| 210 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 32 mm NB Size | Each | 793.00 |
| 211 | b) - do - labour charges only | Each | 31.00 |
| 212 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 40 mm NB Size | Each | 1081.00 |
| 213 | b) - do - labour charges only | Each | 48.00 |
| 214 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 50 mm NB Size | Each | 1561.00 |
| 215 | b) - do - labour charges only | Each | 64.00 |
| 216 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 65 mm NB Size | Each | 2977.00 |
| 217 | b) - do - labour charges only | Each | 80.00 |
| 218 | a) S & F Gunmetal Gate (GM peet) valve as per IS - 778 Class - I, Indian make heavy type - 80 mm NB Size | Each | 4213.00 |
| 219 | b) - do - labour charges only | Each | 95.00 |
| 220 | a) Supplying & fixing Gunmetal (GM) Horizontal Check (Non return) Valve as per IS-778 I-class heavy duty - 15 mm NB Size | Each | 233.00 |
| 221 | b) - do - labour charges only | Each | 40.00 |
| 222 | a) Supplying & fixing Gunmetal (GM) Horizontal Check (Non return) Valve as per IS-778 I-class heavy duty - 25 mm NB Size | Each | 497.00 |
| 223 | b) - do - labour charges only | Each | 48.00 |
| 224 | a) Supplying & fixing Gunmetal (GM) Horizontal Check (Non return) Valve as per IS-778 I-class heavy duty - 40 mm NB Size | Each | 1092.00 |
| 225 | b) - do - labour charges only | Each | 80.00 |
| 226 | a) Supplying & fixing Gunmetal (GM) Horizontal Check (Non return) Valve as per IS-778 I-class heavy duty - 50 mm NB Size | Each | 1569.00 |
| 227 | b) - do - labour charges only | Each | 80.00 |
| 228 | a) Supplying & fixing Gunmetal (GM) Horizontal Check (Non return) Valve as per IS-778 I-class heavy duty - 20 mm NB Size | Each | 322.00 |
| 229 | b) - do - labour charges only | Each | 38.00 |
| 230 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 15 mm NB Size | Each | 177.00 |
| 231 | b) - do - labour charges only | Each | 38.00 |
| 232 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 20 mm NB Size | Each | 237.00 |
| 233 | b) - do - labour charges only | Each | 38.00 |
| 234 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 25 mm NB Size | Each | 314.00 |
| 235 | b) - do - labour charges only | Each | 38.00 |
| 236 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 32 mm NB Size | Each | 469.00 |
| 237 | b) - do - labour charges only | Each | 38.00 |
| 238 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 40 mm NB Size | Each | 705.00 |
| 239 | b) - do - labour charges only | Each | 38.00 |
| 240 | a) Supplying & fixing Gunmetal (GM) Vertical Check (Non return) Valve as per IS-778 I-class heavy duty - 50 mm NB Size | Each | 1021.00 |
| 241 | b) - do - labour charges only | Each | 38.00 |
| 242 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 15 mm NB Size | Each | 276.00 |
| 243 | b) - do - labour charges only | Each | 38.00 |
| 244 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 20 mm NB Size | Each | 360.00 |
| 245 | b) - do - labour charges only | Each | 38.00 |
| 246 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 25 mm NB Size | Each | 529.00 |
| 247 | b) - do - labour charges only | Each | 38.00 |
| 248 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 32 mm NB Size | Each | 959.00 |
| 249 | b) - do - labour charges only | Each | 38.00 |
| 250 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 40 mm NB Size | Each | 1440.00 |
| 251 | b) - do - labour charges only | Each | 38.00 |
| 252 | a) S & F Gunmetal (GM) Ball valve with SS Ball and SS Spindle as per IS - Class - I, Indian make heavy type - 50 mm NB Size | Each | 2519.00 |
| 253 | b) - do - labour charges only | Each | 38.00 |
| 254 | a) S& F 15 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 118.00 |
| 255 | b) - do - labour charges only | 1 RM | 29.00 |
| 256 | a) S& F 20 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 136.00 |
| 257 | b) - do - labour charges only | 1 RM | 29.00 |
| 258 | a) S& F 25 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 178.00 |
| 259 | b) - do - labour charges only | 1 RM | 29.00 |
| 260 | a) S& F 32 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 249.00 |
| 261 | b) - do - labour charges only | 1 RM | 29.00 |
| 262 | a) S& F 40 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 281.00 |
| 263 | b) - do - labour charges only | 1 RM | 31.00 |
| 264 | a) S& F 50 mm Nominal Bore GI pipe Light Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 306.00 |
| 265 | b) - do - labour charges only | 1 RM | 48.00 |
| 266 | a) S& F 15 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 131.00 |
| 267 | b) - do - labour charges only | 1 RM | 29.00 |
| 268 | a) S& F 20 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 146.00 |
| 269 | b) - do - labour charges only | 1 RM | 29.00 |
| 270 | a) S& F 25 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 191.00 |
| 271 | b) - do - labour charges only | 1 RM | 29.00 |
| 272 | a) S& F 32 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 269.00 |
| 273 | b) - do - labour charges only | 1 RM | 29.00 |
| 274 | a) S& F 40 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 301.00 |
| 275 | b) - do - labour charges only | 1 RM | 31.00 |
| 276 | a) S& F 50 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 ISI mark with GI fittings including the cost of pipe & its fittings & labour charges complete | 1 RM | 324.00 |
| 277 | b) - do - labour charges only | 1 RM | 48.00 |
| 278 | S & F 65 mm Nominal Bore GI pipe Medium Grade properties & weight as per IS 1239 with GI fittings such as elbows tees couplings, nipples, plugs including excavation for trenches and refilling the trenches complete except for GI bends union and GI connectors with checkout and socket Tata or Zenith make or equivalent. | 1 RM | 575.00 |
| 279 | a)S & F 12.7 mm holder bat clamps | Each | 9.00 |
| 280 | b) - do - labour charges only | Each | 5.00 |
| 281 | a)S & F 19.05 mm holder bat clamps | Each | 14.00 |
| 282 | b) - do - labour charges only | Each | 5.00 |
| 283 | a)S & F 25.4 mm holder bat clamps | Each | 17.00 |
| 284 | b) - do - labour charges only | Each | 5.00 |
| 285 | a)S & F 38.1 mm holder bat clamps | Each | 21.00 |
| 286 | b) - do - labour charges only | Each | 6.00 |
| 287 | a) S & F 12.7 x 152.0 mm NP shower rose heavy | Each | 132.00 |
| 288 | b) - do - labour charges only | Each | 14.00 |
| 289 | a) S & F 12.7 x 137.0 mm NP shower rose heavy | Each | 127.00 |
| 290 | b) - do - labour charges only | Each | 16.00 |
| 291 | a) S & F 12.7 x 101.6 mm NP shower rose heavy | Each | 121.00 |
| 292 | b) - do - labour charges only | Each | 16.00 |
| 293 | a) S & F 12.7 x 88.9 CP fancy shower | Each | 215.00 |
| 294 | b) - do - labour charges only | Each | 16.00 |
| 295 | a) Supplying & fixing 12.7 mm GI drain pipe with plug for tanks (Mud valve) including making a hole | Each | 30.00 |
| 296 | b) - do - labour charges only | Each | 16.00 |
| 297 | a) Supplying & fixing 19.05 mm GI drain pipe with plug for tanks (Mud valve) including making a hole | Each | 44.00 |
| 298 | b) - do - labour charges only | Each | 15.00 |
| 299 | a) Supplying & fixing 25.4 mm GI drain pipe with plug for tanks (Mud valve) including making a hole | Each | 52.00 |
| 300 | b) - do - labour charges only | Each | 17.00 |
| 301 | a) Taking branch water connection from the existing 25.4 mm GI pipe including cutting threading, etc, complete with fitting | Each | 102.00 |
| 302 | b) - do - labour charges only | Each | 29.00 |
| 303 | a) Taking branch water connection from the existing 19.05 mm GI pipe including cutting threading, etc, complete with fitting, | Each | 58.00 |
| 304 | b) - do - labour charges only | Each | 26.00 |
| 305 | a) Taking branch water connection from the existing 12.7 mm GI pipe including cutting threading, etc, complete with fitting, | Each | 44.00 |
| 306 | b) - do - labour charges only | Each | 22.00 |
| 307 | Supplying & fixing leather washers for 12.7 mm bib tap and stop cock | Each | 1.00 |
| 308 | Supplying & fixing leather washers for 19.05 mm bib tap and stop cock | Each | 3.00 |
| 309 | Supplying & Fixing 12.7 mm fibre washers | Each | 3.00 |
| 310 | Supplying & Fixing 19.05 mm fibre washers | Each | 3.00 |
|  | PVC PIPES & FITTINGS |  |  |
| 311 | a) Providing & Placing on Terrace (at all floor levels) polyethylene water storage tank with Double layer approved brand & manufacture with cover and suitable locking arrangement & making necessary holes for inlet & outlets and over flow pipes but without fittings & base support for tanks | 1 Lit | 6.00 |
| 312 | b) - do - labour charges only | 1 Lit | 0.38 |
| 313 | Supply of Plastic storage tank lid with hinges | Each | 60.00 |
| 314 | Supply & Fixing 76.2 mm dia PVC elbow Ist quality | Each | 37.00 |
| 315 | Supply & fixing 31.75 mm dia PVC flexible waste pipe of 914.4 mm length of Ist quality | Each | 21.00 |
| 316 | Supply & Fixing 38.1 mm dia PVC solid waste pipe outlet | Each | 70.00 |
| 317 | Supply & fixing Fancy PVC shelf including all fittings | Each | 168.00 |
| 318 | Supply & fixing of PVC low level system parry ware, slim line with internal components & short bend. |  |  |
| 319 | a) 10 Litres capacity Single Flush | Each | 943.00 |
| 320 | b) 10 litres capacity Dual Flush | Each | 1173.00 |
| 321 | c) 8 Litres capacity single Flush | Each | 633.00 |
|  | PRINCE/SUDHAKAR PVC/SWR PIPES & FITTINGS (OR EQUIVALENT) |  |  |
|  | Prince/ Sudhakar make or equivalent quality PVC /SWR pipes & fittings ( as per ISI standards) 4kg/cm2 |  |  |
| 322 | 75 mm dia 3 M Single Socket pipe | Each | 202.00 |
| 323 | 90 mm dia 3 M single socket pipe | Each | 320.00 |
| 324 | 110 mm dia 3 M Single socket pipe | Each | 383.00 |
| 325 | 75 mm dia 3 M Double socket pipe | Each | 228.00 |
| 326 | 90 mm dia 3 M Double socket pipe | Each | 363.00 |
| 327 | 110 mm dia 3 M Double socket pipe | Each | 444.00 |
| 328 | 75 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 142.00 |
| 329 | 90 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 219.00 |
| 330 | 110 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 272.00 |
| 331 | 75 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 151.00 |
| 332 | 90 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 232.00 |
| 333 | 110 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 282.00 |
| 334 | 75 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 104.00 |
| 335 | 90 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 160.00 |
| 336 | 110 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 192.00 |
| 337 | 75 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 88.00 |
| 338 | 90 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 129.00 |
| 339 | 110 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 166.00 |
| 340 | 75 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 53.00 |
| 341 | 90 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 89.00 |
| 342 | 110 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 110.00 |
| 343 | 160 MM dia 3 M Double socket pipe | Each | 911.00 |
|  | Prince/ Sudhakar make or equivalent quality PVC /SWR pipes & fittings ( as per ISI standards) 6kg/cm2 |  |  |
| 344 | 75 mm dia 3 M Single Socket pipe | Each | 342.00 |
| 345 | 90 mm dia 3 M single socket pipe | Each | 550.00 |
| 346 | 110 mm dia 3 M Single socket pipe | Each | 655.00 |
| 347 | 75 mm dia 3 M Double socket pipe | Each | 374.00 |
| 348 | 90 mm dia 3 M Double socket pipe | Each | 593.00 |
| 349 | 110 mm dia 3 M Double socket pipe | Each | 723.00 |
| 350 | 75 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 224.00 |
| 351 | 90 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 349.00 |
| 352 | 110 mm dia 1828.8 mm (6'-0") Single Socket pipe | Each | 415.00 |
| 353 | 75 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 239.00 |
| 354 | 90 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 363.00 |
| 355 | 110 mm dia 1828.8 mm (6'-0") Double Socket pipe | Each | 448.00 |
| 356 | 75 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 162.00 |
| 357 | 90 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 251.00 |
| 358 | 110 mm dia 1219. mm (4'-0") Double Socket pipe | Each | 306.00 |
| 359 | 75 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 140.00 |
| 360 | 90 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 205.00 |
| 361 | 110 mm dia 914.4mm (3'-0") Double Socket pipe | Each | 248.00 |
| 362 | 75 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 90.00 |
| 363 | 90 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 141.00 |
| 364 | 110 mm dia 609.6mm (2'-0") Double Socket pipe | Each | 179.00 |
| 365 | 160 mm dia 3 M Double socket pipe | Each | 1490.00 |
|  | Prince/ Sudhakar make or equivalent quality U.V resistant UPVC) SWR pipe fittings (as per ISI standards ) |  |  |
|  | Plain Bend 87.5 Degrees. |  |  |
| 366 | 75 mm dia | Each | 39.00 |
| 367 | 90 mm dia | Each | 56.00 |
| 368 | 110 mm dia | Each | 65.00 |
|  | Door Bend 87.5 Degrees. |  |  |
| 369 | 75 mm dia | Each | 53.00 |
| 370 | 90 mm dia | Each | 75.00 |
| 371 | 110 mm dia | Each | 84.00 |
|  | 45 Degree Bend |  |  |
| 372 | 75 mm dia | Each | 30.00 |
| 373 | 90 mm dia | Each | 43.00 |
| 374 | 110 mm dia | Each | 56.00 |
|  | Cleaning Pipe |  |  |
| 375 | 75 mm dia | Each | 53.00 |
| 376 | 90 mm dia | Each | 77.00 |
| 377 | 110 mm dia | Each | 93.00 |
|  | Single Tee |  |  |
| 378 | 75 mm dia | Each | 47.00 |
| 379 | 90 mm dia | Each | 70.00 |
| 380 | 110 mm dia | Each | 85.00 |
|  | Single Tee (With Door) |  |  |
| 381 | 75 mm dia | Each | 57.00 |
| 382 | 90 mm dia | Each | 85.00 |
| 383 | 110 mm dia | Each | 103.00 |
|  | Double Tee |  |  |
| 384 | 75 mm dia | Each | 99.00 |
| 385 | 90 mm dia | Each | 144.00 |
| 386 | 110 mm dia | Each | 205.00 |
|  | Double Tee (With Door) |  |  |
| 387 | 75 mm dia | Each | 158.00 |
| 388 | 90 mm dia | Each | 226.00 |
| 389 | 110 mm dia | Each | 233.00 |
|  | Single ' Y' |  |  |
| 390 | 75 mm dia | Each | 58.00 |
| 391 | 90 mm dia | Each | 93.00 |
| 392 | 110 mm dia | Each | 108.00 |
|  | Single ' Y' (With Door) |  |  |
| 393 | 75 mm dia | Each | 75.00 |
| 394 | 90 mm dia | Each | 117.00 |
| 395 | 110 mm dia | Each | 135.00 |
|  | Double ' Y' |  |  |
| 396 | 75 mm dia | Each | 81.00 |
| 397 | 90 mm dia | Each | 125.00 |
| 398 | 110 mm dia | Each | 142.00 |
|  | Double ' Y' (With Door ) |  |  |
| 399 | 75 mm dia | Each | 104.00 |
| 400 | 90 mm dia | Each | 163.00 |
| 401 | 110 mm dia | Each | 183.00 |
|  | Coupler |  |  |
| 402 | 75 mm dia | Each | 28.00 |
| 403 | 90 mm dia | Each | 40.00 |
| 404 | 110 mm dia | Each | 45.00 |
|  | Reducer |  |  |
| 405 | 90 x 75 mm dia | Each | 47.00 |
| 406 | 110 x 75 mm dia | Each | 45.00 |
| 407 | 90 x 110 mm dia | Each | 63.00 |
|  | Vent Cowl |  |  |
| 408 | 63 mm dia | Each | 7.00 |
| 409 | 75 mm dia | Each | 11.00 |
| 410 | 90 mm dia | Each | 13.00 |
| 411 | 110 mm dia | Each | 15.00 |
|  | Pipe Clip |  |  |
| 412 | 40 mm dia | Each | 9.00 |
| 413 | 50 mm dia | Each | 11.00 |
| 414 | 63 mm dia | Each | 11.00 |
| 415 | 75 mm dia | Each | 12.00 |
| 416 | 90 mm dia | Each | 13.00 |
| 417 | 110 mm | Each | 14.00 |
| 418 | 160 mm dia | Each | 28.00 |
|  | P Trap |  |  |
| 419 | P' Trap ( Small without Air vent ) 110 mm | Each | 149.00 |
| 420 | P' Trap ( Small with Air vent ) 110 mm | Each | 164.00 |
| 421 | P' Trap (Big with Air vent ) 110 mm | Each | 188.00 |
| 422 | P' Trap ( Big without Air vent ) 110 mm | Each | 173.00 |
| 423 | Floor Trap (Bell Mouth ) 110 mm | Each | 179.00 |
| 424 | Bell Mouth Trap 7 " ( 177.8 mm) | Each | 261.00 |
| 425 | 4" (101.6 mm ) Nahany Trap ( Without Jali with inlet) | Each | 89.00 |
| 426 | 3" (76.2 mm ) Nahany Trap ( without Jali without inlet) | Each | 69.00 |
| 427 | 4" (101.6 mm ) Nahany Trap ( without inlet with out Jali) | Each | 85.00 |
| 428 | Mini Trap | Each | 42.00 |
| 429 | 3" (76.2 mm ) Nahany Trap with Jali | Each | 81.00 |
| 430 | 4" (101.6 mm ) multi floor trap with Jali | Each | 114.00 |
| 431 | 4" (101.6) multi floor trap with out Jalli | Each | 89.00 |
| 432 | Round Jali | Each | 19.00 |
| 433 | Square tile & Jalli | Each | 30.00 |
| 434 | Pipe connector ( with LIP ring) | Each | 119.00 |
| 435 | Pipe connector ( with out LIP ring) | Each | 92.00 |
|  | Floor Traps |  |  |
| 436 | 4" (101.6 mm ) Floor Trap - | Each | 109.00 |
| 437 | 5" (127.mm ) floor Trap | Each | 116.00 |
|  | Door cap |  |  |
| 438 | 75 mm dia | Each | 45.00 |
| 439 | 90 mm dia | Each | 50.00 |
| 440 | 110 mm dia | Each | 58.00 |
|  | Socket Plug |  |  |
| 441 | 75 mm dia | Each | 42.00 |
| 442 | 90 mm dia | Each | 48.00 |
| 443 | 110 mm dia | Each | 50.00 |
|  | Rubber Ring for socket |  |  |
| 444 | 75 mm dia | Each | 6.00 |
| 445 | 90 mm dia | Each | 7.00 |
| 446 | 110 mm dia | Each | 9.00 |
|  | Door Socket |  |  |
| 447 | 75 mm dia | Each | 7.00 |
| 448 | 90 mm dia | Each | 9.00 |
| 449 | 110 mm dia | Each | 11.00 |
| 450 | LIP ring 110 mm | Each | 55.00 |
|  | Rubber Lubricant |  |  |
| 451 | 100 Grams | Each | 23.00 |
| 452 | 250 Grams | Each | 51.00 |
| 453 | 500 Grams | Each | 93.00 |
| 454 | 1000 Grams | Each | 173.00 |
| 455 | P' Trap with lip Ring | Each | 236.00 |
|  | Solvent Cement |  |  |
| 456 | 50 ml | Each | 12.00 |
| 457 | 100 ml | Each | 20.00 |
| 458 | 250 ml | Each | 47.00 |
| 459 | 500 ml | Each | 91.00 |
| 460 | 1000 ml | Each | 179.00 |
|  | Internal Reducer. |  |  |
| 461 | 110mm x 90 mm | Each | 71.00 |
| 462 | 160 mm x 110 mm | Each | 177.00 |
|  | CENTRIFUGAL CAST IRON PIPES & FITTNGS AS PER IS: 3989 - 1984 |  |  |
|  | CI (spun) soils Waste & Ventilating pipe (3 M) |  |  |
| 463 | 75 mm dia | Each | 1024.00 |
| 464 | 100 mm dia | Each | 1364.00 |
| 465 | 150 mm dia | Each | 2501.00 |
|  | CI (spun) soils Waste & Ventilating pipe (3 M) Double socket |  |  |
| 466 | 75 mm dia | Each | 1137.00 |
| 467 | 100 mm dia | Each | 1535.00 |
|  | 9 feet (2743.2 mm ) pipes |  |  |
| 468 | 75 mm dia | Each | 922.00 |
| 469 | 100 mm dia | Each | 1230.00 |
|  | 8 feet (2438.4) mm ) pipes |  |  |
| 470 | 75 mm dia | Each | 824.00 |
| 471 | 100 mm dia | Each | 1096.00 |
|  | 6 feet (1828.8 mm ) pipes |  |  |
| 472 | 75 mm dia | Each | 671.00 |
| 473 | 100 mm dia | Each | 875.00 |
|  | 6 feet (1828.8 mm ) pipes Double socket |  |  |
| 474 | 75 mm dia | Each | 705.00 |
| 475 | 100 mm dia | Each | 875.00 |
|  | Cut pipes (pieces) 1 foot (304.8 mm) |  |  |
| 476 | 75 mm dia | Each | 244.00 |
| 477 | 100 mm dia | Each | 281.00 |
|  | Cut pipes (pieces) 1'.6" (457.2 mm ) |  |  |
| 478 | 75 mm dia | Each | 350.00 |
| 479 | 100 mm dia | Each | 403.00 |
| 480 | 150 mm dia | Each | 769.00 |
|  | Cut pipes (pieces ) 2 feet (609.6 mm) |  |  |
| 481 | 75 mm dia | Each | 483.00 |
| 482 | 100 mm dia | Each | 534.00 |
| 483 | 150 mm dia | Each | 963.00 |
|  | Cut pipes (pieces ) 3 feet (914.4 mm) |  |  |
| 484 | 75 mm dia | Each | 741.00 |
| 485 | 100 mm dia | Each | 999.00 |
|  | Cut pipes (pieces ) 4 feet (1219.2 mm) |  |  |
| 486 | 75 mm dia | Each | 815.00 |
| 487 | 100 mm dia | Each | 1220.00 |
|  | Plain Bend |  |  |
| 488 | 75 mm dia | Each | 251.00 |
| 489 | 100 mm dia | Each | 296.00 |
| 490 | 150 mm dia | Each | 706.00 |
|  | Door Bend |  |  |
| 491 | 75 mm dia | Each | 260.00 |
| 492 | 100 mm dia | Each | 337.00 |
| 493 | 150 mm dia | Each | 772.00 |
|  | Plain Bend 45 Degrees. |  |  |
| 494 | 75 mm dia | Each | 251.00 |
| 495 | 100 mm dia | Each | 296.00 |
| 496 | 150 mm dia | Each | 706.00 |
|  | Door Bend 45 Degrees. |  |  |
| 497 | 75 mm dia | Each | 260.00 |
| 498 | 100 mm dia | Each | 337.00 |
| 499 | 150 mm dia | Each | 772.00 |
| 500 | Horn Bend Plain 101.6 x 50.8 mm (4" x 2") 100 mm dia | Each | 512.00 |
| 501 | Horn Bend door 101.6 x 50.8 mm (4" x 2") 100 mm dia | Each | 597.00 |
|  | Heel Rest Bend |  |  |
| 502 | 75 mm dia | Each | 268.00 |
| 503 | 100 mm dia | Each | 343.00 |
| 504 | 150 mm dia | Each | 727.00 |
|  | Single Branch Plain "T" |  |  |
| 505 | 75 mm dia | Each | 337.00 |
| 506 | 100 mm dia | Each | 456.00 |
| 507 | 150 mm dia | Each | 939.00 |
|  | Single Branch Door "T" |  |  |
| 508 | 75 mm dia | Each | 397.00 |
| 509 | 100 mm dia | Each | 528.00 |
| 510 | 150 mm dia | Each | 992.00 |
|  | Single Branch plain "Y" |  |  |
| 511 | 75 mm dia | Each | 397.00 |
| 512 | 100 mm dia | Each | 528.00 |
| 513 | 150 mm dia | Each | 1164.00 |
|  | Single Branch Door "Y" |  |  |
| 514 | 75 mm dia | Each | 436.00 |
| 515 | 100 mm dia | Each | 583.00 |
| 516 | 150 mm dia | Each | 1255.00 |
|  | Double Branch Plain "T" |  |  |
| 517 | 75 mm dia | Each | 461.00 |
| 518 | 100 mm dia | Each | 621.00 |
|  | Double Branch Door "T" |  |  |
| 519 | 75 mm dia | Each | 514.00 |
| 520 | 100 mm dia | Each | 647.00 |
|  | Double Branch Plain " Y" |  |  |
| 521 | 75 mm dia | Each | 514.00 |
| 522 | 100 mm dia | Each | 647.00 |
|  | Double Branch Door " Y" |  |  |
| 523 | 75 mm dia | Each | 542.00 |
| 524 | 100 mm dia | Each | 702.00 |
|  | Shoe bend |  |  |
| 525 | 75 mm dia | Each | 317.00 |
| 526 | 100 mm dia | Each | 371.00 |
| 527 | 150 mm dia | Each | 755.00 |
|  | Collar |  |  |
| 528 | 75 mm dia | Each | 217.00 |
| 529 | 100 mm dia | Each | 264.00 |
| 530 | 150 mm dia | Each | 542.00 |
|  | Cowl |  |  |
| 531 | 75 mm dia | Each | 192.00 |
| 532 | 100 mm dia | Each | 217.00 |
| 533 | 150 mm dia | Each | 488.00 |
|  | Offset 3" (76.2 mm ) projection |  |  |
| 534 | 75 mm dia | Each | 278.00 |
| 535 | 100 mm dia | Each | 337.00 |
| 536 | 150 mm dia | Each | 682.00 |
|  | Offset 6" (152.4mm ) projection |  |  |
| 537 | 75 mm dia | Each | 304.00 |
| 538 | 100 mm dia | Each | 397.00 |
| 539 | 150 mm dia | Each | 793.00 |
|  | Long Door Bend |  |  |
| 540 | 75 mm dia | Each | 639.00 |
| 541 | 100 mm dia | Each | 688.00 |
|  | Hand Hole pipe ( Inspection pipe) |  |  |
| 542 | 75 mm dia | Each | 368.00 |
| 543 | 100 mm dia | Each | 500.00 |
| 544 | 150 mm dia | Each | 666.00 |
|  | P Traps |  |  |
| 545 | 75 mm dia | Each | 397.00 |
| 546 | 100 mm dia | Each | 545.00 |
| 547 | 4" x 3" (101.6 mm x 76.2mm ) P. Traps 100 mm dia | Each | 625.00 |
| 548 | 4" x 3" (101.6 mm x 76.2mm ) Vent Traps 100 mm dia | Each | 625.00 |
| 549 | 4" x 3" (101.6 mm x 76.2mm ) Reducer 75 mm dia | Each | 388.00 |
| 550 | 6" x 4" (152.4 mm x101.6 mm ) Reducer 75 mm dia | Each | 531.00 |
| 551 | 3" x 2" ( 76.2 mm x 50.8 mm ) plain T 75 mm dia | Each | 332.00 |
| 552 | 3" x 2" ( 76.2 mm x 50.8 mm ) Door T 75 mm dia | Each | 517.00 |
| 553 | 3" x 2" ( 76.2 mm x 50.8 mm ) Plain Y 75 mm dia | Each | 517.00 |
| 554 | 3" x 2" ( 76.2 mm x 50.8 mm ) Door Y 75 mm dia | Each | 517.00 |
| 555 | 4" x 3" (101.6 mm x 76.2mm ) Plain T 75 mm dia | Each | 517.00 |
| 556 | 4" x 3" (101.6 mm x 76.2mm ) Door T 75 mm dia | Each | 549.00 |
| 557 | 4" x 3" (101.6 mm x 76.2mm ) Plain Y 75 mm dia | Each | 549.00 |
| 558 | 4" x 3" (101.6 mm x 76.2mm ) Door Y 75 mm dia | Each | 586.00 |
| 559 | 6" x 4" (152.4 mm x101.6 mm ) Plain T 75 mm dia | Each | 767.00 |
| 560 | 6" x 4" (152.4 mm x101.6 mm ) Door T 75 mm dia | Each | 830.00 |
| 561 | 6" x 4" (152.4 mm x101.6 mm ) Plain Y 75 mm dia | Each | 878.00 |
| 562 | 6" x 4" (152.4 mm x101.6 mm ) Door Y 75 mm dia | Each | 1074.00 |
|  | Special Types of Long Fittings |  |  |
|  | Plain " T" 27 " (685.8 mm ) |  |  |
| 563 | 75 mm dia | Each | 1113.00 |
| 564 | 100 mm dia | Each | 1563.00 |
|  | Door "T" 27" (685.8 mm ) |  |  |
| 565 | 75 mm dia | Each | 1243.00 |
| 566 | 100mm dia | Each | 1717.00 |
|  | Plain " Y" 27" (685.8 mm) |  |  |
| 567 | 75 mm dia | Each | 1243.00 |
| 568 | 100mm dia | Each | 1717.00 |
|  | Door " Y" 27" (685.8 mm) |  |  |
| 569 | 75 mm dia | Each | 1386.00 |
| 570 | 100mm dia | Each | 1885.00 |
|  | MISCELLANEOUS ITEMS |  |  |
| 571 | a) Supplying & Fixing solder joints up to 50.8 mm dia | Each | 45.00 |
| 572 | b) - do - labour charges only | Each | 14.00 |
| 573 | a) Supplying & Fixing solution joints up to 50.8 dia Ist quality. | Each | 9.00 |
| 574 | b) - do - labour charges only | Each | 2.00 |
| 575 | Chiselling the stone masonry wall for fixing of wooden blocks including finishing as directed by the department | Each | 21.00 |
| 576 | Chiselling the stone masonry wall and repairs as directed by the department. | 1 Rm | 98.00 |
| 577 | Chiselling the brick masonry wall and repairs as directed by the department. | 1 Rm | 21.00 |
| 578 | a) S & F NP telephonic shower spray set & valve mixer with rest and all fittings complete (ARK/ESSCO or equivalent make) | Each | 1599.00 |
| 579 | b) - do - labour charges only | Each | 286.00 |
| 580 | a) S & F NP shower spray fancy with NP Flexible pipe | Each | 425.00 |
| 581 | b) - do - labour charges only | Each | 95.00 |
| 582 | a) S & F NP cover for nahany trap | Each | 23.00 |
| 583 | b) - do - labour charges only | Each | 13.00 |
| 584 | a) Painting the flushing tank with white enamel paint | Each | 77.00 |
| 585 | b) - do - labour charges only | Each | 45.00 |
| 586 | S & F internal fittings for flush tank Parryplast wonder flush or equivalent | Each | 484.00 |
| 587 | Repairing and over hauling flushing tank | Each | 64.00 |
| 588 | a) Supplying & Fixing NP chain rubber and plugh for baisn | Each | 30.00 |
| 589 | b) - do - labour charges only | Each | 5.00 |
| 590 | a) S & F 25.4 mm dia & 609.6 mm long aluminium anodized towel rod with brackets and aluminium screws | Each | 121.00 |
| 591 | b) - do - labour charges only | Each | 31.00 |
| 592 | a) S & F 19.04 mm dia & 609.6 mm long aluminium anodized towel rod with brackets and aluminium screws | Each | 105.00 |
| 593 | b) - do - labour charges only | Each | 48.00 |
| 594 | a) S & F 609.6 mm long 127 mm wide plate glass shelf with superior quality aluminium railing complete with aluminium screws | Each | 154.00 |
| 595 | b) - do - labour charges only | Each | 40.00 |
| 596 | a) S & F 609.6 mm long 127 mm wide plate glass shelf only | Each | 50.00 |
| 597 | b) - do - labour charges only | Each | 8.00 |
| 598 | a) S & F NP soap dish heavy type with NP SCREWS | Each | 179.00 |
| 599 | b) - do - labour charges only | Each | 9.00 |
| 600 | a) S &F NP holder brackets with screws | Each | 36.00 |
| 601 | b) - do - labour charges only | Each | 10.00 |
| 602 | a) S & F liquid soap container NP metalic with NP brackets and screws. | Each | 179.00 |
| 603 | b) - do - labour charges only | Each | 16.00 |
| 604 | a) S & F 31.75 mm C.P TRAPS | Each | 231.00 |
| 605 | b) - do - labour charges only | Each | 31.00 |
| 606 | a) Supplying & fixing 88.9 mm A.C cowls | Each | 52.00 |
| 607 | b) - do - labour charges only | Each | 10.00 |
| 608 | a) Supplying & fixing A.C Cowl 63.5 mm dia | Each | 36.00 |
| 609 | b) - do - labour charges only | Each | 8.00 |
| 610 | Removing and refixing W.C s | Each | 169.00 |
| 611 | Removing and refixing wash hand basins of any size with fittings complete | Each | 151.00 |
| 612 | a) S & F 76.2 x 101.6 mm teak wood block complete | Each | 19.00 |
| 613 | b) - do - labour charges only | Each | 6.00 |
| 614 | Cutting holes in C.C; basement & repairs Labour charges only | Each | 71.00 |
| 615 | Cutting holes in stone masonry & basement and repairs Labour charges only | Each | 48.00 |
| 616 | Cutting holes in Brick masonry & repairs Labour charges only | Each | 31.00 |
| 617 | Cutting holes in RCC slab floor & repairs Labour charges only | Each | 48.00 |
| 618 | a) Supplying & Fixing 12.7 mm PVC connection with brass union nut C.P coated | Each | 74.00 |
| 619 | b) - do - labour charges only | Each | 14.00 |
| 620 | a) S & F 12.7 mm .M.P couplings | Each | 26.00 |
| 621 | b) - do - labour charges only | Each | 8.00 |
| 622 | a) S & F 19.05 mm M.P couplings | Each | 30.00 |
| 623 | b) - do - labour charges only | Each | 8.00 |
| 624 | a) S& F MP coupling 25.4 mm dia | Each | 70.00 |
| 625 | a) S & F PVC Rubber adopter for LLC and bend | Each | 14.00 |
| 626 | b) - do - labour charges only | Each | 5.00 |
| 627 | a) S& F 31.75 mm Brass union nuts | Each | 26.00 |
| 628 | b) - do - labour charges only | Each | 5.00 |
| 629 | a) S& F 38.1 mm Brass union nuts | Each | 28.00 |
| 630 | b) - do - labour charges only | Each | 6.00 |
| 631 | a) Constructing Brick masonry chamber over the gully trap or peet valve and fitted with 304.8 x 228.6 m size CI frame and hinged cover. | Each | 336.00 |
| 632 | b) - do - labour charges only | Each | 71.00 |
| 633 | a) Constructing Brick masonry support for G.I pipe with C.M 304.8 x 228.6 x228.6 mm size including plastering and finishing etc. complete. | Each | 54.00 |
| 634 | b) - do - labour charges only | Each | 16.00 |
| 635 | a) Supplying & fixing 304.8 x 228.6 mm CI frame & cover over gully taps | Each | 88.00 |
| 636 | b) - do - labour charges only | Each | 25.00 |
| 637 | a) Supplying & fixing CI Nahany trap grating | Each | 14.00 |
| 638 | b) - do - labour charges only | Each | 8.00 |
| 639 | a) Supplying & fixing CI gully trap grating | Each | 36.00 |
| 640 | b) - do - labour charges only | Each | 8.00 |
| 641 | a) Supplying & fixing valves for brass ball cocks 25.4 mm dia | Each | 14.00 |
| 642 | b) - do - labour charges only | Each | 6.00 |
| 643 | a) Supplying & fixing 12.7 m valve for bib tap or stop cocks | Each | 13.00 |
| 644 | b) - do - labour charges only | Each | 4.00 |
| 645 | a) Supplying & fixing 19.05 m valve for bib tap or stop cocks | Each | 22.00 |
| 646 | b) - do - labour charges only | Each | 5.00 |
| 647 | Removing 12.7 mm GI pipe and refixing the same | Rm | 41.00 |
| 648 | Removing 19.05 mm GI pipe and refixing the same | Rm | 48.00 |
| 649 | Removing 25.4 mm GI pipe and refixing the same | Rm | 70.00 |
| 650 | Removing 38.1 mm GI pipe and refixing the same | Rm | 88.00 |
| 651 | Removing 50.8 mm GI pipe and refixing the same | Rm | 104.00 |
| 652 | Cleaning the W.C with acid and Vim powder | Each | 24.00 |
| 653 | Cleaning the Urinal with acid and Vim power | Each | 24.00 |
| 654 | Cleaning the wash basin with acid and Vim powder | Each | 24.00 |
| 655 | Cleaning the E.W.C with acid and Vim power | Each | 24.00 |
| 656 | a) Supplying & fixing 914.4 x 457.2 mm CI man hole frame and cover (light weight) 30 Kgs | Each | 1155.00 |
| 657 | b) - do - labour charges only | Each | 64.00 |
| 658 | a) Supplying & fixing CI steps for septic tank. | Each | 36.00 |
| 659 | b) - do - labour charges only | Each | 8.00 |
| 660 | a) Supplying & fixing 50.8 cm dia C.I. Man hole frame & cover for 914.4 mm dia chamber (20 Kgs - Light weight) | Each | 562.00 |
| 661 | b) - do - labour charges only | Each | 48.00 |
| 662 | a) Supplying & fixing 50.8 cm dia C.I. Man hole frame & cover for 914.4 mm dia chamber (Medium weight) as per IS:1726 - 40 Kgs | Each | 1540.00 |
| 663 | b) - do - labour charges only | Each | 104.00 |
| 664 | Constg. 50.8 mm thick RCM baffle wall with 2.267 Kg steel & rabbit wire mesh in CM 1:3 with fine rendering in neat cement labour charges Only | 1 Sqm | 154.00 |
| 665 | a) Constg. 50.8 mm RCC partition wall | 1 Sqm | 176.00 |
| 666 | b) - do - labour charges only | 1 Sqm | 31.00 |
| 667 | a) S & F double layer of bamboo matting tarred on both sides. | 10 Sqm | 419.00 |
| 668 | b) - do - labour charges only | 10 Sqm | 80.00 |
| 669 | Removing of 101.6 m or 76.2 mm CI pipe | 1 RM | 24.00 |
| 670 | a) Supplying & fixing 76.2 mm W.I clamps | Each | 12.00 |
| 671 | b) - do - labour charges only | Each | 5.00 |
| 672 | a) Supplying & fixing NP coat hook | Each | 30.00 |
| 673 | b) - do - labour charges only | Each | 8.00 |
| 674 | Supplying & fixing small precast RCC water tanks including cover over the roof of building | 1 Lit | Deleted |
| 675 | Labour charges for laying glazed tiles | 1 Sqm | 80.00 |
| 676 | a) Supplying and fixing TV shape mirror with plastic frame size 609.6 mm x 457 .2 mm | Each | 367.00 |
| 677 | b) - do - labour charges only | Each | 83.00 |
| 678 | a) Supplying & fixing GI pipe plug of size 12.7 mm dia | Each | 3.00 |
| 679 | b) - do - labour charges only | Each | 1.00 |
| 680 | a) Supplying & fixing 19.06 mm dia GI plug | Each | 4.00 |
| 681 | b) - do - labour charges only | Each | 1.00 |
| 682 | a) Supplying and fixing 38.1 mm dia of GI plug | Each | 17.00 |
| 683 | b) - do - labour charges only | Each | 5.00 |
| 684 | a) Supplying & fixing 50.8 mm dia of GI PLUG | Each | 36.00 |
| 685 | b) - do - labour charges only | Each | 8.00 |
| 686 | a) Supplying & fixing 76.2 mm dia of GI PLUG | Each | 52.00 |
| 687 | b) - do - labour charges only | Each | 13.00 |
| 688 | a) Supplying & fixing C.P flange. | Each | 12.00 |
| 689 | b) - do - labour charges only | Each | 5.00 |
| 690 | a) Supplying & fixing stainless steel sink size 36" x 18" (914.4x457.2mm) 1 mm thick with accessories | Each | 4685.00 |
| 691 | b) - do - labour charges only | Each | 239.00 |
| 692 | a) Supplying & fixing SINGLE BOWL stainless steel sink size 24"x18"x8 " (609.6 x 457.2 x 203.2mm) 1 mm thick with accessories | Each | 3880.00 |
| 693 | b) - do - labour charges only | Each | 239.00 |
| 694 | a) Supplying & fixing stainless steel sink size 20" x18" x 8 " (508x457.2x203.2mm) 1 mm thick with accessories | Each | 3513.00 |
| 695 | b) - do - labour charges only | Each | 239.00 |
| 696 | Supplying & fixing 4 " x 24 " (101.6x609.6mm) white glazed porcelain channel fixed in brick masonry white cement pointing. | Each | 333.00 |
| 697 | Supply & Fixing of steel surgical long elbow action handle 12.7 mm dia bib cock Indian make heavy type including all materials | Each | 704.00 |
| 698 | Supply & fixing 2 1/2" (63.5mm) dia Sluice valve ISI mark with 2 Nos. flanges, 8 nos, 3/4" dia nuts & bolts and all necessary washers and packing materials etc. complete | Each | 3968.00 |
| 699 | Supply & fixing porcelain P' trap | Each | 167.00 |
| 700 | Supplying & fixing oval shape basin white(520x410mm) parry or equivalent | Each | 1728.00 |
| 701 | Supply & fixing RCC cover 22" dia with nominal reinforcement 3 " thick and hooks for lifting | Each | 585.00 |
| 702 | Supply & fixing cp spl. Grating with frame and cover of Ist quality | Each | 42.00 |
| 703 | Supply & fixing towel rod (CP) of 24" long and 3/4 " dia | Each | 273.00 |
| 704 | Supply & fixing of LLC handle set Ist quality | Each | 58.00 |
| 705 | Supply & fixing 4" dia CP thimble pipe |  | 275.00 |
| 706 | a) 150 mm long | Each | 293.00 |
| 707 | b) 220 mm long | Each | 439.00 |
| 708 | c) 300 mm long | Each | 585.00 |
| 709 | Supply & fixing CP long bend | Each | 238.00 |
| 710 | Supply & fixing CP short bend | Each | 119.00 |
| 711 | Supplying & fixing bib cock with flange C.P Jaquar make queen series. | Each | 1075.00 |
|  | Supply & fixing bib cock cum health faucet with 1 m long flexible tube and wall hook of Jaquar make queen series with 7 years warranty |  |  |
| 712 | a ) Chrome plated | Each | 3088.00 |
| 713 | b) Ivory gold | Each | 5116.00 |
|  | Supply & fixing pillar cock Jaquar make Queen series with 7 years warranty |  |  |
| 714 | a ) Chrome plated | Each | 1203.00 |
| 715 | b) Ivory gold | Each | 2015.00 |
|  | Supply & fixing Angular stop cocks with wall flange Jaquar make Queen series with 7 years warranty |  |  |
| 716 | a ) Chrome plated | Each | 1164.00 |
| 717 | b) Ivory gold | Each | 2288.00 |
|  | Supply & fixing Waste coupling Jaquar make with 7 years warranty |  |  |
| 718 | a ) Chrome plated | Each | 280.00 |
|  | Supply & fixing of Bottle trap Jaquar make with 7 years warranty |  |  |
| 719 | a ) Chrome plated | Each | 1170.00 |
| 720 | b) Ivory gold | Each | 1840.00 |
|  | Supply & Fixing copper pipe 450 mm long with nuts & washers Jaquar make with 7 years warranty |  |  |
| 721 | a ) Chrome plated | Each | 280.00 |
| 722 | b) Ivory gold | Each | 546.00 |
|  | Supply and fixing wall mirror with provision for over head shower and 115 mm long bend pipe with wall flange Jaquar make queen series with 7 years warranty |  |  |
| 723 | a) Chrome plated | Each | 3543.00 |
| 724 | b) Ivory gold | Each | 6396.00 |
|  | Supply & fixing overhead shower rose with revolving joint and 150 mm long swivel shower arm Jaquar make queen series. with 7 years warranty |  |  |
| 725 | a) Chrome plated | Each | 1625.00 |
| 726 | b) Ivory Gold | Each | 2503.00 |
| 727 | Supply and Fixing Wall Mixer with Telephonic shower arrangement with crutch and flexible pipe Jaquar series Ivory gold colour for bath tub. with 7 years warranty | Each | 9906.00 |
| 728 | Supply & fixing Bath tub overflow set complete with over flow cap, 450 mm Ball chain with rubber plug & union bend with brass nut Jaquar make, Ivory gold colour. with 7 years warranty | Each | 1138.00 |
| 729 | Supply & fixing sink cock with regular swinging spout with wall flange chromeplated Jaquar make Queen series with 7 years warranty | Each | 1710.00 |
|  | Supply & fixing soap dish Jaquar make queen series: with 7 years warranty |  |  |
| 730 | a) Chrome plated | Each | 1073.00 |
| 731 | b) Ivory Gold | Each | 1729.00 |
|  | Supply & fixing of toilet paper holder Jaquar make Queen series. with 7 years warranty |  |  |
| 732 | a) Chrome plated | Each | 904.00 |
| 733 | b) Ivory Gold | Each | 1580.00 |
|  | Supply & Fixing of Towel ring Jaquar make Queen series with 7 years warranty |  |  |
| 734 | a) Chrome plated | Each | 845.00 |
| 735 | b) Ivory Gold | Each | 1443.00 |
|  | Supply & Fixing of towel coat hook Jaquar make Queen series with 7 years warranty |  |  |
| 736 | a) Chrome plated | Each | 618.00 |
| 737 | b) Ivory Gold | Each | 1268.00 |
| 738 | Supply & Fixing of Towel rod 24 " Jaquar make Queen series Ivory gold colour with 7 years warranty | Each | 2230.00 |
| 739 | Supply & Fixing of Tumbler holder Jaquar make Queen series, Ivory gold colour with 7 years warranty | Each | 1567.00 |
| 740 | Supply & fixing of Glass shelf 22" Jaquar make Queen series, Ivory gold colour with 7 years warranty | Each | 3094.00 |
| 741 | Supply & Fixing of Bibcock cum Health Faucet with 1m long tube and wall hook of Jaquar Continental series, Chrome Finish with 7 years warranty | Each | 2197.00 |
| 742 | Supply & Fixing of Bibcock Jaquar Continental series, Chrome Finish with 7 years warranty | Each | 644.00 |
| 743 | Supply & Fixing of Pillar cock Jaquar Continental series, Chrome Finish with 7 years warranty | Each | 800.00 |
| 744 | Supply & Fixing of Angular stopcock with wall flange Jaquar Continental series, Chrome finish with 7 years warranty | Each | 631.00 |
| 745 | Supply fixing of Waste coupling Jaquar make, Chrome finish with 7 years warranty | Each | 280.00 |
| 746 | Supply & Fixing of Bottle Trap Jaquar make, Chrome finish with 7 years warranty | Each | 1235.00 |
| 747 | Supply & Fixing of copper pipe 450mm long with nuts & washers Jaquar make, Chrome Finish with 7 years warranty | Each | 280.00 |
| 748 | Supply & Fixing of Wall Mixer with provision for overhead shower and 115mm long bend pipe with wall flange Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 2828.00 |
| 749 | Supply & Fixing of Exclusive overhead shower with 190mm long light body shower arm Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 1300.00 |
| 750 | Supply & Fixing wall mixer with telephonic shower arrangement with crutch and telephone shower with flexible tube Jaquar make Continental series Chrome colour for bath tub with 7 years warranty | Each | 3991.00 |
| 751 | Supply & Fixing bath tub over flow set complete with over flow cap,450mm ball chain with rubber plug 7 Union bend with brass nuts Jaquar make, Chrome colour with 7 years warranty | Each | 819.00 |
| 752 | Supply & Fixing Sink cock with regular swinging spout with flange chrome plated Jaquar make Continental series with 7 years warranty | Each | 1138.00 |
| 753 | Supply & Fixing soap dish Jaquar make Continental series, Chrome Finish with 7 years warranty | Each | 878.00 |
| 754 | Supply & Fixing of Toilet paper holder Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 644.00 |
| 755 | Supply & Fixing of Towel Ring Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 741.00 |
| 756 | Supply & Fixing Double coat hook Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 553.00 |
| 757 | Supply & Fixing of towel rail 24'' Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 1196.00 |
| 758 | Supply & Fixing of Tumbler holder Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 806.00 |
| 759 | Supply & Fixing of Glass shelf 22'' Jaquar make Continental series, Chrome finish with 7 years warranty | Each | 1586.00 |
| 760 | Supply & Fixing of Bibcock cum Health Faucet with 1m long tube and wall hook of Jaquar Florentine(Quarter turn) series, Chrome Finish with 7 years warranty | Each | 2750.00 |
| 761 | Supply & Fixing of Bibcock Jaquar Florentine (Quarter turn)series, Chrome Finish with 7 years warranty | Each | 878.00 |
| 762 | Supply & Fixing of Pillar cock Jaquar Florentine(Quarter turn) series, Chrome Finish with 7 years warranty | Each | 878.00 |
| 763 | Supply & Fixing of Angular stopcock with wall flange Jaquar Florentine( Quarter turn)series, Chrome finish with 7 years warranty | Each | 845.00 |
| 764 | Supply & Fixing of Bottle Trap Jaquar make, Chrome finish with 7 years warranty | Each | 1235.00 |
| 765 | Supply & Fixing of Wall Mixer with provision for overhead shower and 115mm long bend pipe with wall flange Jaquar make Florentine(Quarter turn) series, Chrome finish with 7 years warranty | Each | 3283.00 |
| 766 | Supply & Fixing wall mixer with telephonic shower arrangement with crutch and telephone shower with flexible tube Jaquar make Florentine(Quarter turn) series Chrome colour for bath tub with 7 years warranty | Each | 5103.00 |
| 767 | Supply & Fixing Sink cock with regular swinging spout with flange chrome plated Jaquar make Florentine(Quarter turn) series with 7 years warranty | Each | 1359.00 |
| 768 | Supply & Fixing of Rose shower with revolving ball joint & 115mm long shower arm with 7 years warranty | Each | 1560.00 |
| 769 | Supply & Fixing of Jaquar make chrome plated Metro pole flush valve concealed type with cover plate 40mm size with 7 years warranty | Each | 2730.00 |
| 770 | Supply & Fixing of Jaquar make Sensotronic sensor faucet for wash basin, Chrome plated with 7 years warranty | Each | 10725.00 |
| 771 | Supply & fixing of Jaquar make sensotronic concealed type flushing valve for urinal complete set with installation box with control cock, Chrome plated with 7 years warranty | Each | 10725.00 |
| 772 | Supply & fixing of Jaquar make Auto closing system pillar cock for wash basin, Chrome plated with 7 years warranty | Each | 1690.00 |
| 773 | Supply& Fixing of Jaquar make Auto Closing system Bibcock, Chrome plated with 7 years warranty | Each | 1690.00 |
|  | S&F Polyethylene-Aluminium-Polyethylene (PE-AL-PE) composite pressure pipes conforming to IS-15450-2004 with UV stabilized, carbon black for hot & cold water supply, capable to withstand pressure up to 80 C including all special composite fittings, plastic & brass blend inserts for elbows, tees, reducers, couplers, connectors etc complete for external works including laying pipes, fixing in position, testing and commissioning etc |  |  |
| 774 | 16 mm OD pipe | 1 RM | 134.00 |
| 775 | 20 mm OD pipe | 1 RM | 165.00 |
| 776 | 25mm OD pipe | 1 RM | 207.00 |
| 777 | 32 mm OD pipe | 1 RM | 278.00 |
| 778 | 40 mm OD pipe | 1 RM | 365.00 |
| 779 | 50 mm OD pipe | 1 RM | 465.00 |
|  | S&F Polyethylene-Aluminium-Polyethylene (PE-AL-PE) composite pressure pipes conforming to IS-15450-2004 with UV stabilized, carbon black for hot & cold water supply, capable to withstand pressure up to 80 C including all special composite fittings, plastic & brass blend inserts for elbows, tees, reducers, couplers, connectors etc complete with MS Iron clamps at 1.0 M spacing including cutting, chasing walls, making good, laying pipes, fixing in position, testing and commissioning etc |  |  |
| 780 | 16 mm OD pipe | 1 RM | 193.00 |
| 781 | 20 mm OD pipe | 1 RM | 231.00 |
| 782 | 25mm OD pipe | 1 RM | 285.00 |
| 783 | 32 mm OD pipe | 1 RM | 376.00 |
| 784 | 40 mm OD pipe | 1 RM | 488.00 |
| 785 | 50 mm OD pipe | 1 RM | 616.00 |
|  | S&F Polyethylene-Aluminium-Polyethylene (PE-AL-PE) composite pressure pipes conforming to IS-15450-2004 with UV stabilized, carbon black for hot & cold water supply, capable to withstand pressure up to 80 C including all special composite fittings, plastic & brass blend inserts for elbows, tees, reducers, couplers, connectors etc complete including excavating trenches up to 1.0 M deep laying pipes, fixing in position, testing, refilling excavated earth and commissioning etc |  |  |
| 786 | 16 mm OD pipe | 1 RM | 157.00 |
| 787 | 20 mm OD pipe | 1 RM | 186.00 |
| 788 | 25mm OD pipe | 1 RM | 226.00 |
| 789 | 32 mm OD pipe | 1 RM | 293.00 |
| 790 | 40 mm OD pipe | 1 RM | 375.00 |
|  | Supply and laying of ASHIRVAD CPVC Pipes and Fittings or equal ant for Hot & Cold conforming to IS 15778:2007 with malleable specials such as Tees, Elbows, check nuts, unions, bushes, etc, Solvent Cement including fixing in walls ceiling floors Include for necessary excavation in all sorts of soils, chasing in the walls etc, refilling consolidation with the excavated earth & Making bores in walls ceiling etc. and making good the chased surface in cement mortar and complete. When making a transition use metal threads fittings, including all taxes for finished item of work as directed by the Engineer In charge. |  |  |
| 791 | 15.90mm OD Pipe SDR 13.5 | 1 RM | 170.00 |
| 792 | 22.20mm OD Pipe SDR 13.5 | 1 RM | 215.00 |
| 793 | 28.60mm OD Pipe SDR 13.5 | 1 RM | 298.00 |
| 794 | 34.90mm OD Pipe SDR 13.5 | 1 RM | 358.00 |
| 795 | 41.30mm OD Pipe SDR 13.5 | 1 RM | 521.00 |
| 796 | 54.00mm OD Pipe SDR 13.5 | 1 RM | 710.00 |
| 797 | 15.90mm OD Pipe SDR 11 | 1 RM | 210.00 |
| 798 | 22.20mm OD Pipe SDR 11 | 1 RM | 251.00 |
| 799 | 28.60mm OD Pipe SDR 11 | 1 RM | 282.00 |
| 800 | 34.90mm OD Pipe SDR 11 | 1 RM | 410.00 |
| 801 | 41.30mm OD Pipe SDR 11 | 1 RM | 560.00 |
| 802 | 54.00mm OD Pipe SDR 11 | 1 RM | 810.00 |
| Note: 1) All the items to be used on work are of first quality & ISI and as approved by the Engineer In charge of the work. |  |  |  |
| 2) Rates given above are: |  |  |  |
| A) Rate for finished item of work |  |  |  |
| B) Labour charges only |  |  |  |
| 3) RATES |  |  |  |
| Supply Rates for all the materials like pipes, WC's, wash hand basins etc., are based on market rates plus Sales Tax/ VAT. Composite tax on finished items may be allowed as applicable. |  |  |  |
| 4) LABOUR |  |  |  |
| Labour charges are increased based on proposals and market rates |  |  |  |
| 5) Coloured porcelain items: |  |  |  |
| 50% extra may be allowed on basic cost of white porcelain items excluding fittings |  |  |  |
| 6) SSR rates for Sanitary & water supply items are purely for guidance for the officers for preparation of estimates vide Para 113 of APPWD code. Authority who prepares the estimate is however responsible for proper rates as envisaged in Para 45 of APPWD code. |  |  |  |

ELECTRICAL ITEMS

- PREAMBLE TO ELCTRICAL SSR 2008-09.

- In case of ceiling fans and exhaust fans the prevailing price list/ catalogue rates    (as on 1.6.2008) of all brands of standard makes such as orient/Usha /Khaitan/Crompton/Bajaj/Almonard/EPC etc., will be adopted based on the requirement and necessity.

- The area allowances in case of Corporation, Municipalities, Jail Compounds, Agency & Tribal areas and Industrial areas have to be followed as per prevailing Civil SSR.

- The departmental stores procurement was dispensed with by the Government hence all the materials have to be invariably procured and got executed through the agency.

- As per the rule 45 under Indian Electricity Rules the Electrical contractor is solely responsible for both quality and quantity. Proposing specific make in the estimate from the SSR is with in the scope of concerned field officers based on the necessity and availability of materials in such areas.

- In case of stair case point wiring where two way switch control is necessary, an extra at 50 % excess over each point shall be allowed. In respect of usage of FRLS wires for important buildings, VVIP residences etc., an extra 5 % (five percent) on the cost of wires as approved in the SSR 2008-09 may be adopted.

- Where ever the light/fan point is provided at height/length of 15 feet and above, then an extra of 50% over the point rates shall be allowed.

- The rates for common civil items including labour rates have to adopt from prevailing civil SSR.

- The rates for all types of lighting fixtures proposed are inclusive of all taxes such as Sales Tax, Excise duty etc., but with out cost of tubes and bulbs and the VAT to be added as per rules.

- The data which are reviewed by the centre for good Governance and submitted to the Government are taken in to consideration for working out finished item rate for the purpose of preparing estimates.

- The Government instructions for usage of energy efficient luminaries have to invariably follow while proposing luminaries in the estimates.

- The rates for items such as Lifts, Transformers, Generators, pump sets, Refrigerators/ A.C. plants/A.C. units, Water coolers etc., have to be invariable  followed based on the prevailing market rates, since the rates and specification varies from manufacturer to manufacturer considerably. Hence, rates for these items are not incorporated in the schedule. However, relevant data with out rates is provided in the standard schedule data for electrical works.

- The rates for steel items such as M.S. poles, M.S. iron, G.I. pipes and its accessories shall be as per the rates approved by the Government/Steel India authority/ List price from reputed firms etc., from time to time.

- In case of any addition to the proposed SSR, the approval has to be obtained from the Chief Engineer (R&B) Buildings through the Superintending Engineer (R&B) Electrical circle. Temporary approvals which are under relevant specifications of the BIS/ISO/ISI can be given by the Superintending Engineer (R&B) Electrical/Executive Engineer (R&B) Electrical.

- The Superintending Engineer (R&B) Electrical will review the performance over the quality of the makes proposed in the SSR from time to time and in case any inferior quality observed the same shall be brought to the notice of the Chief Engineer (R&B) Buildings to consider for deletion from the list.

- The schedule of rates is only for guidance of officers for preparation of estimates as per the para 113 of APPWD code. The authority that sanctioned the estimate is however responsible for adopting the proper rates in the estimates as envisaged in para 45 of APPWD code.

| I N D E X |  |  |  |
| --- | --- | --- | --- |
| SL.NO. | DESCRIPTION | PAGE NO |  |
|  |  | FROM | TO |
| I | M.S.CONDUIT PIPES, PVC PIPES, CASING AND CAPING |  | 104 |
| II & III | WIRING & RUN OF MAINS (Copper wire, Switches & Acccessories) | 104 | 107 |
| IV | SWITCH GEAR & MCB DISTRIBUTION BOARDS | 107 |  |
|  | SDFU,COS,MCCB,MCB,ISOLATOR,ELCB,DB's AND VOLTAGE SURGE PROTECTORS |  | 114 |
| V | EARTHING (Covered in Miscellaneous Item i.e. at Page No.48 to 54) |  | 114 |
| VI | SERVICE MAINS & L.T.O.H. LINES |  | 114 |
| VII | FLUORESCENT, HPSV, M.H. STREET LIGHTS, POST TOP, WELL GLASS AND FLOOD LIGHT LUMANAIRES. | 115 | 120 |
| VIII | INTERNAL LUMANAIRES, MIRROR OPTIC, T-5 CFL DOWN LIGHTS | 120 | 124 |
| IX | AIR CONDITIONING, REFRIGERATION AND FANS | 125 | 127 |
| X | WATER HEATERS |  | 127 |
| XI | ACCESSORIES TO WATER PUMPS | 127 | 128 |
| XII | BUSBARS(Rates Covered in Miscellaneous Items) |  | 128 |
| XIII | SWAGED POLES, GARDEN POSTS AND ACCESSORIES |  | 128 |
| XIV | CONTROL PANEL AND METERING EQUIPMENT | 128 | 129 |
| XV | U.G. CABLES, GLANDS AND LUGS | 129 | 132 |
| XVI | TEMPORARY ILLUMINATION(As per standard Data) |  | 132 |
| XVII | GENERATORS, TRANSFORMERS, S.S. Eqpt, PROTECTIVE GEARS, CAPACITORS AND ACB's. | 132 | 135 |
| XVIII | REPAIRS TO MOTORS AND REWINDING |  | 135 |
| XIX | MISCELLANEOUS | 138 | 139 |

|  | I. M. S. Conduit Pipes (Rate per 100 Mtrs) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sl.No. |  | 1 | 2 | 3 | 4 | 5 |  |
|  | Dia |  | 20 mm | 25 mm | 32 mm | 38 mm | 50 mm |  |
|  | Thickness |  | 16 SWG | 16 SWG | 16 SWG | 16 SWG | 16 SWG |  |
|  | NAI |  | 5,670.00 | 6,990.00 | 10,800.00 | --- | --- |  |
|  | BHARAT |  | 5,400.00 | 6,360.00 | 9,600.00 | --- | --- |  |
|  | GB |  | 5,670.00 | 7,020.00 | 10,800.00 | 6,876.00 | 11,856.00 |  |
|  | I. PVC Pipes (Rate per 100 Mtrs) |  |  |  |  |  |  |  |
|  | Sl.No. | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | Dia | 20 mm | 25 mm | 20 mm | 25 mm | 32 mm | 40 mm | 50 mm |
|  | Thickness | 1.5 mm | 1.5 mm | 2 mm | 2 mm | 2 mm | 2 mm | 2 mm |
|  | Mercury | 1,348.80 | 2,094.00 | 1,773.60 | 2,342.40 | 3,726.00 | 4,969.20 | 5,068.58 |
|  | Sudhakar ISI | 1,348.80 | 2,094.00 | 1,773.60 | 2,342.40 | 3,726.00 |  | 0.00 |
|  | ATUL | 1,256.40 | 1,675.20 | 1,676.40 | 2,233.20 | 2,932.80 | 3,840.00 | 3,916.80 |
|  | Sudhakar Non ISI | 1,256.40 | 1,675.20 | 1,676.40 | 2,233.20 | 2,932.80 | 3,840.00 | 3,916.80 |
|  | Hanuman | 884.40 | 1,356.00 | --- | --- | --- | --- | --- |
|  | Rama | 1,052.40 | 1,356.00 | --- | --- | --- | --- | --- |
|  | Durga | 1,256.40 | 1,675.20 | 1,676.40 | 2,233.20 | 2,932.80 | 3,840.00 | 3,916.80 |
|  | Nelco | 1,430.40 | 2,065.20 | 2,370.00 | 2,497.20 | 3,442.80 | 4,454.40 | 4,543.49 |
|  | Avon Plast ISI | 1,920.00 | 2,520.00 |  |  | 4,680.00 | 6,600.00 | 6,732.00 |
|  | Modi ISI | 1,668.00 | 2,250.00 | 2,130.00 | 2,892.00 | 3,924.00 | 6,192.00 | 6,315.84 |
|  | I. Casing & Capping(Rate per One Mtr) |  |  |  |  |  |  |  |
|  | Sl.No. |  |  |  | 13 | 14 | 15 | 16 |
|  | Width |  |  |  | 19 / 20 mm | 25 mm | 32 mm | 38 / 40 mm |
|  | Hight |  |  |  | 12.5 mm | 12.5 mm | 12.5 mm | 2 mm |
|  | Diamond / Durga |  |  |  | 23.94 | 30.48 | 34.86 | 59.88 |
|  | MODI |  |  |  | 17.40 | 19.20 | 24.60 | 37.20 |
|  | II. & III Wiring & Run of Mains |  |  |  |  |  |  |  |
|  | FR PVC Insulated Copper Wire(Rate per 100 Mtrs) |  |  |  |  |  |  |  |
|  | Sl.No. | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
|  | Thickness | 14/0.3 mm | 22/0.3 mm | 36/0.3 mm | 56/0.3 mm | 84/0.3 mm | 140/0.3 mm | 126/0.4 mm |
|  | FINOLEX | 1,132.80 | 1,699.20 | 2,732.40 | 4,026.00 | 6,360.00 | --- | --- |
|  | Powerflex | 1,314.00 | 2,049.60 | 3,129.60 | 4,885.20 | 7,629.60 | 12,199.20 | 20,245.20 |
|  | HAVELLES | 1,219.20 | 1,826.40 | 2,719.20 | 4,262.40 | 6,660.00 | 11,041.20 | 17,373.60 |
|  | L & T | 1,132.80 | 1,699.20 | 2,732.40 | 4,026.00 | 6,360.00 | --- | --- |
|  | RPG | 1,075.20 | 1,614.00 | 2,595.60 | 3,824.40 | 6,042.00 | --- | --- |
|  | Polycab | 1,075.20 | 1,614.00 | 2,595.60 | 3,824.40 | 6,042.00 | --- | --- |
|  | Standard | 1,297.20 | 1,939.20 | 3,013.20 | 4,539.60 | 7,086.00 | 11,622.00 | 18,288.00 |
|  | Paragon | 1,075.20 | 1,614.00 | 2,595.60 | 3,824.40 | 6,042.00 | --- | --- |
|  | PAYAL | 1,214.40 | 1,851.60 | 2,384.40 | 3,540.00 | 5,349.60 | 9,189.60 | 14,810.40 |
|  | Gold Medal | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,730.00 | 9,531.60 | 20,665.20 |
|  | MILLION | 962.40 | 1,458.00 | 2,384.40 | 3,540.00 | 5,349.60 | 9,081.60 | 14,810.40 |
|  | SUNCAB | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,696.40 | 9,531.60 | 20,665.20 |
|  | FINE CAB | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,696.40 | 9,531.60 | 20,665.20 |
|  | PAWAN CAB | 962.40 | 1,458.00 | 2,384.40 | 3,540.00 | 5,349.60 | 9,081.60 | --- |
|  | SUN LIGHT | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,696.40 | 9,531.60 | 20,665.20 |
|  | Star | 962.40 | 1,458.00 | 2,384.40 | 3,540.00 | 5,349.60 | 9,081.60 | 14,810.40 |
|  | P - Flex | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,696.40 | 9,531.60 | 20,665.20 |
|  | Milltec | 1,093.20 | 1,737.60 | 2,743.20 | 4,197.60 | 6,781.20 | 11,001.60 | 17,997.60 |
|  | SMILE | 1,214.40 | 1,851.60 | 2,384.40 | 3,540.00 | 5,349.60 | 9,189.60 | 14,810.40 |
|  | Alankar | 1,032.00 | 1,572.00 | 2,498.40 | 3,817.20 | 5,696.40 | 9,531.60 | 20,665.20 |
|  | II. Wiring (Switches and Accessories) (Rate per Each) |  |  |  |  |  |  |  |
| S.No | Description |  | ANCHOR | GOLD MEDAL | MILLION | VIMAL | MARU | ICON |
| 24 | 6A Switch (One way) |  | 19.20 | 18.90 | 20.22 | 22.92 | 18.24 | 19.20 |
| 25 | 6A Switch (Two way) |  | 22.80 | 20.22 | 22.92 | 24.30 | 21.60 | 22.80 |
| 26 | 6A Two pin Socket |  | 18.60 | 18.24 | 23.58 | --- | 16.86 | 18.60 |
| 27 | 6A 3/2 pin Socket |  | 28.80 | 28.32 | 35.10 | 33.72 | 26.28 | 28.80 |
| 28 | 6A Indicator lamp Neon |  | 28.80 | 19.56 | 25.62 | 27.00 | 20.22 | 28.80 |
| 29 | 15A/10A Fuse flush |  | 33.60 | 35.10 | 39.12 | 40.50 | 36.42 | 33.60 |
| 30 | Telephone Jack with connector |  | 49.20 | 56.70 | 47.22 | 55.32 | 48.06 | 49.20 |
| 31 | T.V antenne outlet |  | 27.60 | 28.34 | 31.02 | 27.66 | 28.82 | 27.60 |
| 32 | Bed Switch /Bell push |  | 31.20 | 19.56 | 32.40 | --- | 28.32 | 31.20 |
| 33 | Ceiling Rose 3 plate |  | 20.40 | 24.30 | 27.00 | 26.28 | 21.60 | 20.40 |
| 34 | 16A switch (1 way) |  | 62.40 | 64.80 | 72.90 | 76.26 | 55.32 | 62.40 |
| 35 | 16A socket |  | 62.40 | 64.80 | 67.50 | 76.26 | 58.02 | 62.40 |
| 36 | 16A switch socket combined. |  | 94.80 | 102.60 | 124.20 | 121.50 | 97.20 | 94.80 |
| 37 | 16A (5 in one) |  | 168.00 | 145.80 | 175.50 | 152.52 | 145.80 | 168.00 |
| 38 | 16A Top Tringle |  | 54.00 | 47.22 | 62.10 | --- | 54.00 | 54.00 |
| 39 | 6A Top Tringle |  | 38.40 | 35.10 | 40.50 | --- | 33.72 | 38.40 |
| 40 | BH / SBH |  | 21.60 | 20.88 | 23.64 | 24.30 | 21.48 | 21.60 |
| 41 | 32A DP Swtich F/T |  | 96.00 | 121.50 | 135.00 | 143.10 | 117.48 | 96.00 |
| 42 | 400W Electronic Regulators (Socket type) |  | 168.00 | 170.10 | 162.00 | 190.32 | 143.10 | 168.00 |
| 43 | 600W Electronic Regulators |  | 226.80 | 276.72 | 270.00 | 297.00 | 249.72 | 226.80 |
| 44 | Call bell |  | 62.40 | --- | --- | --- | 68.82 | 62.40 |
| 45 | Buzzer |  | 54.00 | 43.20 | 48.00 | 54.00 | 49.92 | 54.00 |
| 46 | Ding Dong bell |  | 106.80 | 101.22 | 141.72 | 121.50 | 103.92 | 106.80 |
| 47 | Musical Bell Electronic |  | 265.20 | 241.62 | 324.00 | 243.00 | 202.50 | 265.20 |
| 48 | Pendent Holder / Adoptor |  | 20.40 | 18.90 | 24.30 | --- | 15.48 | 20.40 |
| 49 | Spikeguard (Deluxe / ROMA) 4 way |  | 597.60 | --- | --- | --- | 594.00 | --- |
| 50 | Anchor (EXCEL) |  | 350.40 | --- | --- | --- | --- | 350.40 |
| 51 | Bed Light Transformer. |  | --- | 110.70 | --- | --- | --- | --- |
| 52 | MC Holder |  | 12.00 | 12.00 | 12.00 | 12.00 | 12.00 | 12.00 |
|  | II. Wiring (Mini Modular Switches)(Rate per Each) |  |  |  |  |  |  |  |
|  | Description |  |  |  |  | ANCHOR | Gold Medal | MILLION |
| 53 | 6A 1 way switch |  |  |  |  | 21.6 | 24 | 19.2 |
| 54 | 6A 2 way switch |  |  |  |  | 25.2 | 28.8 | 21.6 |
| 55 | Bell push |  |  |  |  | 25.2 | 28.8 | 21.6 |
| 56 | Neon lmap indicator |  |  |  |  | 25.2 | 28.8 | 19.2 |
| 57 | Master bell push with Neon |  |  |  |  | 30 | 57.6 | 36 |
| 58 | 2 pin Socket |  |  |  |  | 21.6 | 25.2 | 21.6 |
| 59 | 2 in one ksocket |  |  |  |  | 32.4 | 34.2 | 28.8 |
| 60 | Electronic dimmer (EME) |  |  |  |  | 225.6 | 288 | 252 |
| 61 | T.V out let delux |  |  |  |  | 37.2 | 39.6 | 32.4 |
| 62 | Kit KAT fuse 16A |  |  |  |  | 34.8 | 38.4 | 33.6 |
| 63 | Telephone out let with connector |  |  |  |  | 45.6 | 57.6 | 42 |
| 64 | 16A switch / Socket |  |  |  |  | 64.8 | 68.4 | 64.8 |
| 65 | 16A switch / Socket 2 in one |  |  |  |  | 132 | 140.4 | 108 |
| 66 | 16A 5 in one |  |  |  |  | 162 | 177.6 | 144 |
| 67 | BH /SBH |  |  |  |  | 31.2 | 31.2 | 18.6 |
| 68 | Ceiling Rose 3 Plate |  |  |  |  | 21.6 | 26.4 | 24 |
| 69 | Plug top 6A 3 pin |  |  |  |  | 31.2 | 33.6 | 30 |
| 70 | Plug top 16A 3 pin |  |  |  |  | 44.4 | 45.6 | 42 |
| 71 | Cover frames for 6A switches |  |  |  |  | 6.6 | 6.6 | 7.2 |
| 72 | Cover frames for 6A Socket / Master bell push |  |  |  |  | 9.6 | 8.1 | 9.6 |
| 73 | Cover frames 16 A switch / Socket |  |  |  |  | 26.4 | 27 | 26.4 |
| 74 | Cover frames for 16A 2 in 1 |  |  |  |  | 42 | 40.2 | 36 |
| 75 | Cover frames for 16A 5-in-one. |  |  |  |  | 54 | 60 | 49.2 |
|  | II. Wiring (Modular Switches)(Rate per Each) |  |  |  |  |  |  |  |
|  | Sl.No. | 76 | 77 | 78 | 79 | 80 | 81 | 82 |
|  | Description | 6 / 10 A - 1 Way Switch | 6 / 10 A - 2 Way Switch | Neon Indicator with switch | Bell Push with indicator | 6 / 10 A 2 Pin socket | 6 / A 2 in one | Electronic dimmer |
|  | Anchor Woods | 128.4 | 136.8 | 140.4 | 152.4 | 100.8 | 0 | 476.4 |
|  | Mossaic | 130.8 | 146.4 | 165.6 | 165.6 | 110.4 | 168 | 522 |
|  | Cabtree | 110.4 | 121.2 | 132 | 138 | 91.2 | 201.6 | 468 |
|  | ABB | 106.8 | 117.6 | 126 | 154.8 | 117.6 | 162 | 540 |
|  | Gold Medal | 81.6 | 93.6 | 96 | 105.6 | 72 | 120 | 384 |
|  | Vimal | 80.4 | 84 | 86.4 | 86.4 | 72 | 108 | 360 |
|  | North West | 72.9 | 85.08 | 97.2 | 113.4 | 68.88 | --- | --- |
|  | Million | 80.4 | 93.6 | 96 | 86.4 | 72 | 108 | 360 |
|  | Siemen Delta vega | 122.4 | 130.8 | 129.6 | 165.6 | 110.4 | 174 | 582 |
|  | Spectra Metallic | 129.6 | 145.2 | 92.4 | 162 |  | 166.8 | 558 |
|  | Wave | 80.4 | 93.6 | 96 | 86.4 | 72 | 108 | 360 |
|  | Sl.No. | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
|  | Description | TV out let delux | Kit KAT fuse 16A | Telephone out let with jack | Plug top 6A 3 pin | Plug top 16A 3pin | Cover frames 1 - Module (metallic) | Cover frames 2 - Module |
|  | Anchor Woods | 104.4 | 118.8 | 130.8 | --- | --- | 114 | 114 |
|  | Mossaic | 112.8 | --- | 99.6 | 110.4 | 193.2 | 132 | 138 |
|  | Cabtree | 96 | 122.4 | 139.2 | 100.8 | 128.4 | 128.4 | 128.4 |
|  | ABB | 108 | --- | 102 | --- | --- | 84 | 86.4 |
|  | Gold Medal | 79.2 | 105.6 | 129.6 | 44.4 | 56.4 | 52.8 | 52.8 |
|  | Vimal | 102 | 102 | 102 | --- | --- | 54 | 54 |
|  | North West | 98.58 | --- | 130.92 | --- | --- | 52.68 | 52.68 |
|  | Million | 102 | 102 | 102 | 44.4 | 56.4 | 52.8 | 52.8 |
|  | Siemen Delta vega | 122.4 | --- | 114 | --- | --- | 117.6 | 118.8 |
|  | Spectra Metallic | 145.2 | 162 | 138 | 99.6 | 132 | 64.8 | 64.8 |
|  | Wave / Pointer | 102 | 102 | 102 | 44.4 | 56.4 | 52.8 | 52.8 |
|  | Sl.No. | 90 | 91 | 92 | 93 | 94 | 95 | 96 |
|  | Description | Cover frames 3 - Module | Cover frames 4 - Module | Cover frame 6 - Module | Cover frames 8-Horizontal | Cover frames 8 - Vertical | Cover frames 12 - Module | Metal box concealed 1/2 module (78x78mm x 50mm deep) |
|  | Anchor Woods | 144 | 165.6 | 237.6 | 319.2 | 288 | 388.8 | 31.2 |
|  | Mossaic | 166.8 | 205.2 | 243.6 | 262.8 | --- | 604.8 | 46.8 |
|  | Cabtree | 141.6 | 162 | 202.8 | 273.6 | 242.4 | 328.8 | 33.6 |
|  | ABB | 96 | 108 | --- | --- | 180 | 252 | 50.4 |
|  | Gold Medal | 62.4 | 72 | 100.8 | 138 | 168 | 192 | 28.8 |
|  | Vimal | 66 | 78 | 108 | 158.4 | 138 | 168 | 28.8 |
|  | North West | 59.4 | 74.28 | 102.6 | --- | 148.5 | 206.52 | 24.3 |
|  | Million | 62.4 | 72 | 100.8 | 138 | 138 | 168 | 28.8 |
|  | Siemen Delta vega | 150 | 174 | 246 | 334.8 | 298.8 | 402 | 45.6 |
|  | Spectra Metallic | 78 | 93.6 | 128.4 | 168 | 177.6 | 216 | 33.6 |
|  | Wave | 24.00 | 135.00 |  | 137.00 | 135.00 | 110.00 | --- |
|  | 12 | 16A Socket. | 169.00 | 52.00 | 186.00 | 150.00 | --- | --- |
|  | Wave | 62.4 | 72 | 100.8 | 138 | 138 | 168 | 28.8 |
|  | Sl.No. |  | 97 | 98 | 99 | 100 | 101 | 102 |
|  | Description |  | Metal box concealed 3 module (78x100 mm) | Metal box concealed 4 module (78x140 mm) | Metal box concealed 6 module (78x205 mm) | Metal box concealed 8 module (78x 230x60mm deep) | Metal box concealed 8 module vertical (135x120 mm) | Metal box concealed 12 module vertical (135x205 mm) |
|  | Anchor Woods |  | 46.8 | 57.6 | 82.8 | 134.4 | 127.2 | 139.2 |
|  | Mossaic |  | 46.8 | 66 | 103.2 | 103.2 | --- | 175.2 |
|  | Cabtree |  | 42.12 | 54 | 81.6 | 126 | 117.6 | 132 |
|  | ABB |  | 50.4 | 62.4 | --- | --- | 118.8 | 174 |
|  | Gold Medal |  | 43.2 | 50.4 | 87.6 | 122.4 | --- | 144 |
|  | Vimal |  | 44.4 | 52.8 | 81.6 | 129.6 | 117.6 | 132 |
|  | North West |  | 25.68 | 32.4 | 64.8 | --- | 64.8 | 91.8 |
|  | Million |  | 44.4 | 52.8 | 81.6 | 129.6 | 117.6 | 144 |
|  | Siemen Delta vega |  | 50.4 | 64.8 | 103.2 | 142.8 | 139.2 | 168 |
|  | Spectra Metallic |  | 40.8 | 56.4 | 84 | 116.4 | 120 | 148.8 |
|  | Wave |  | 44.4 | 52.8 | 81.6 | 129.6 | 117.6 | 144 |
|  | IV. Switchgear & MCB Distribution Boards |  |  |  |  |  |  |  |
|  | CUBICLE TYPE SDFU (Rate per Each) |  |  |  |  |  |  |  |
|  | Sl.No. | 103 | 104 | 105 | 106 | 107 | 108 | 109 |
|  | Description | 32A TPN | 63A TPN | 125A TPN | 200A TPN | 250A TPN | 400A TPN | 630A TPN |
|  | L&T | 1,407.15 | 2,124.00 | 5,097.60 | --- | 8,137.58 | 12,345.75 | 18,850.50 |
|  | GE | 1,274.40 | 1,844.34 | 4,779.00 | 5,741.41 | 7,301.25 | 11,615.63 | 18,585.00 |
|  | C & S | 1,367.33 | 1,884.46 | 4,579.88 | 6,106.50 | --- | 12,345.75 | 20,576.25 |
|  | Havells | 1,250.80 | 1,787.70 | 4,071.00 | 5,740.70 | 6,330.70 | 11,074.30 | --- |
|  | HPL/MOELLER | 1,274.40 | 1,762.92 | 4,290.48 | 5,915.34 | 6,860.52 | 10,949.22 | 17,438.04 |
|  | Standard | 1,185.90 | 1,719.26 | 3,902.26 | 5,393.78 | 6,569.06 | 10,336.80 | 16,731.22 |
|  | Crystal | --- | 2,933.48 | 4,248.00 | 5,920.06 | 7,832.84 | 12,013.58 | 14,164.72 |
| (b) | CHANGE OVER SWITCHES ON LOAD(Rate per Each) |  |  |  |  |  |  |  |
| S.No | Description |  | HPL/MOELLER | GE | Havells | L & T | Standard | Crystal |
| 110 | 63 A - 4 Pole |  | 3,823.20 | 3,836.18 | 3,233.20 | 3,318.16 | 3,160.04 | 5,143.62 |
| 111 | 125 A - 4 Pole |  | 7,180.30 | 6,623.34 | 5,546.00 | 5,973.16 | 5,302.92 | --- |
| 112 | 200 A - 4 Pole |  | 8,947.94 | 8,256.46 | 7,552.00 | 10,288.13 | 7,416.30 | 12,754.62 |
| 113 | 250 A - 4 Pole |  | 12,878.52 | 12,266.10 | 10,785.20 | 12,279.38 | 9,900.20 | --- |
| 114 | 400 A - 4 Pole |  | 19,676.50 | 18,464.64 | 16,236.80 | 18,253.13 | 15,104.00 | 20,974.50 |
| 115 | 630 A - 4 Pole |  | 22,281.94 | 21,531.46 | 18,472.90 | 21,240.00 | 18,260.50 | 32,683.64 |
| 116 | 800 A - 4 Pole |  | 31,194.48 | 30,664.66 | 26,514.60 | 29,205.00 | 24,142.80 | 41,749.58 |
| 117 | 1000 A - 4 Pole |  | 50,740.00 | 51,705.24 | 42,828.10 | 46,462.50 | 39,471.00 | 54,187.96 |
| (c) | PANEL MOUNTED MCCB'S (RATE PER EACH) |  |  |  |  |  |  |  |
| Sl.No. | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 |
| Description | 63/60A 3 pole 10/18 KA | 63/60A 4 pole 10/18/25 KA | 125A 3 Pole 25/35/36 KA | 125A 4 Pole 25 KA | 250A 3 Pole 36/35/50KA | 250A 4 Pole 36/35/50KA | 400A 3 Pole 35/36/50 KA | 400A 4 POLE 35/36/50 KA |
| C & S | 2,883.00 | 5,625.00 | 6,750.00 | 8,788.00 | 16,087.00 | 19,775.00 | 19,406.00 | 24,581.00 |
| MDS | 5,460.00 | 7,285.00 | 6,900.00 | 9,300.00 | 13,666.00 | 17,437.00 | 23,300.00 | 37,000.00 |
| MOELLER | 5,460.00 | 7,285.00 | 6,900.00 | 9,300.00 | 13,666.00 | 17,437.00 | 23,300.00 | 37,000.00 |
| L & T | 5,460.00 | 7,285.00 | 6,900.00 | 9,300.00 | 13,666.00 | 17,437.00 | 23,300.00 | 37,000.00 |
| Havells | 4,800.00 | 5,700.00 | 5,800.00 | 7,100.00 | 13,900.00 | 17,500.00 | 19,406.00 | 24,581.00 |
| HPL | 2,287.00 | 4,781.00 | 5,777.00 | 6,386.00 | 12,830.00 | 15,260.00 | 16,340.00 | 20,570.00 |
| Standard | 4,800.00 | 5,700.00 | 5,800.00 | 7,100.00 | 13,900.00 | 17,500.00 | --- | --- |
| Siemens | 4,800.00 | 5,700.00 | 5,800.00 | 7,100.00 | 13,900.00 | 17,500.00 | 19,406.00 | 24,581.00 |
| Indo Asian | 2,287.00 | 4,781.00 | 6,200.00 | 7,470.00 | 14,906.00 | 17,381.00 | 19,417.00 | --- |
| Crystal | 3,560.00 | --- | 5,777.00 | 6,386.00 | 12,830.00 | 15,260.00 | 16,340.00 | 20,570.00 |

| Sl.No. | 126 | 127 | 128 | 129 | 130 | 131 | 132 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | 6-32A 10KA SPMCB C- Curve | 40A 10 KA SPMCB to 63A - do - | 6-32A 10 KA SPN MCB - do - | 40A 10KA SPN MCB to 63A - do - | 6-32A DP MCB - do - | 40A DP MCB to 63A D.P MCB-do - | 6-32A TP MCB - do - |  |
| MDS Legrend | 207.00 | 427.00 | 600.00 | 973.00 | 600.00 | 973.00 | 966.00 |  |
| G. E | 191.00 | 393.00 | 540.00 | 883.00 | 540.00 | 883.00 | 877.00 |  |
| L & T Hager | 191.00 | 393.00 | 540.00 | 883.00 | 540.00 | 883.00 | 877.00 |  |
| MOELLER | 191.00 | 393.00 | 540.00 | 883.00 | 540.00 | 883.00 | 877.00 |  |
| Havells | 171.00 | 342.00 | 468.00 | 756.00 | 477.00 | 756.00 | 774.00 |  |
| HPL | 171.00 | 342.00 | 468.00 | 756.00 | 477.00 | 756.00 | 774.00 |  |
| Standard | 171.00 | 342.00 | 468.00 | 756.00 | 477.00 | 756.00 | 774.00 |  |
| Siemens | 191.00 | 393.00 | 540.00 | 883.00 | 540.00 | 883.00 | 877.00 |  |
| Indo Asian | 171.00 | 342.00 | 468.00 | 756.00 | 477.00 | 756.00 | 774.00 |  |
| Sputnik | 156.00 | 342.00 | 402.00 | 512.00 | 360.00 | 550.00 | 536.00 |  |
| Crystal | 156.00 | 342.00 | 402.00 | 512.00 | 360.00 | 550.00 | 536.00 |  |
| Milltec | 156.00 | 342.00 | 402.00 | 512.00 | 360.00 | 550.00 | 536.00 |  |
| GECO | 156.00 | 342.00 | 402.00 | 512.00 | 360.00 | 550.00 | 536.00 |  |

| Sl.No. | 133 | 134 | 135 | 136 | 137 | 138 | 139 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | 40A TP MCB to 63A T.P MCB -do - | 6-32A TPN MCB - do - | 40A TPN MCB TO 63A TPN MCB - do - | 6-32A FP MCB - do - | 40A FP MCB to 63 AFP MCB - do - | 63 FP MCB - do - | 32A DP Isolator |  |
| MDS Legrend | 1,593.90 | 1,368.50 | 2,049.30 | 1,368.50 | 2,049.30 | --- | 440.45 |  |
| G. E | 1,593.90 | 1,368.50 | 2,049.30 | 1,368.50 | 2,049.30 | --- | 335.8 |  |
| L & T Hager | 1,593.90 | 1,368.50 | 2,049.30 | 1,368.50 | 2,049.30 | --- | --- |  |
| MOELLER | 1,593.90 | 1,368.50 | 2,049.30 | 1,368.50 | 2,049.30 | --- | --- |  |
| Havells | 1,196.00 | 983.25 | 1,500.75 | 1,104.00 | 1,397.25 | 1,598.50 | --- |  |
| HPL | 1,198.30 | 1,047.65 | 1,397.25 | 1,152.30 | 1,490.40 | 1,722.70 | --- |  |
| Standard | 1,161.50 | 948.75 | 1,466.25 | 1,052.25 | 1,339.75 | 1,546.75 | --- |  |
| Siemens | 1,328.25 | 1,161.50 | 1,696.25 | 1,765.25 | --- | --- | --- |  |
| Indo Asian | 1,150.00 | 943 | 1,461.65 | 1,060.30 | 1,347.80 | 1,536.40 | --- |  |
| Sputnik | 1,052.25 | 708.4 | 1,186.80 | 749.8 | 931.5 | 1,254.65 | --- |  |
| Crystal | 1,182.20 | 984.4 | 1,495.00 | 1,077.55 | 1,345.50 | 1,565.15 | --- |  |
| Milltec | 1,052.25 | 708.4 | 1,186.80 | 749.8 | 931.5 | 1,254.65 | --- |  |
| GECO | 1,052.25 | 708.4 | 1,186.80 | 749.8 | 931.5 | 1,254.65 | --- |  |
| Sl.No. | 140 | 141 | 142 | 143 | 144 | 145 | 146 |  |
| Description | 40A DP Isolator | 63A DP Isolator | 40A TP Isolator | 63A TP Isolator | 40A FP Isolator | 63A FP Isolator | 80 A FP Isolator |  |
| MDS Legrend | 440.45 | 592.25 | --- | 885.5 | 987.85 | 1,078.70 | 1,135.05 |  |
| G. E | 348.45 | 439.3 | 607.2 | 698.05 | 776.25 | 853.3 | 917.7 |  |
| L & T Hager | 348.45 | 439.3 | 607.2 | 698.05 | 776.25 | 853.3 | --- |  |
| MOELLER | 348.45 | 439.3 | 607.2 | 698.05 | 776.25 | --- | --- |  |
| Havells | 348.45 | 439.3 | 607.2 | 698.05 | 776.25 | --- | --- |  |
| HPL | 313.95 | 395.6 | 570.4 | 639.4 | 709.55 | 745.2 | 918.85 |  |
| Standard | 287.5 | 362.25 | 523.25 | 569.25 | 638.25 | 661.25 | --- |  |
| Siemens | 322 | 391 | 529 | 586.5 | 673.9 | 736 | --- |  |
| Indo Asian | 293.25 | 358.8 | 525.55 | 578.45 | 632.5 | 676.2 | 885.5 |  |
| Sputnik | 255.3 | 281.75 | 414 | 439.3 | 491.05 | 530.15 | --- |  |
| Crystal | 343.85 | 414 | 600.3 | 646.3 | 716.45 | 763.6 | --- |  |
| Milltec | 255.3 | 281.75 | 414 | 439.3 | 491.05 | 530.15 | --- |  |
| GECO | 255.3 | 281.75 | 414 | 439.3 | 491.05 | 530.15 | --- |  |
| Sl.No. | 147 | 148 | 149 | 150 | 151 | 152 | 153 |  |
| Description | 100 A FP Isolator | 40A 2 POLE 30 mA RCCB | 63A 2 Pole 30 mA RCCB | 40A 2 Pole 100m A RCCB | 63A 2 Pole 100 mA RCCB | 40A 4 Pole 30 mA RCCB | 63A 4 Pole 30 mA RCCB |  |
| MDS Legrend | --- | 2,796.80 | 3,218.85 | 2,908.35 | 3,344.20 | 3,200.45 | 3,697.25 |  |
| G. E | 1,035.00 | 2,593.25 | 2,981.95 | 2,701.35 | 3,105.00 | 2,910.65 | 3,428.15 |  |
| L & T Hager | 1,035.00 | 2,593.25 | 2,981.95 | 2,696.75 | 3,105.00 | 2,910.65 | 3,428.15 |  |
| MOELLER | 1,035.00 | 2,593.25 | 2,981.95 | 2,701.35 | 3,105.00 | --- | --- |  |
| Havells | --- | 1,932.00 | 2,593.25 | 1,897.50 | 2,484.00 | 2,369.00 | 2,765.75 |  |
| HPL | 967.15 | 2,456.40 | 2,829.00 | 2,561.05 | 2,922.15 | 2,781.85 | 3,247.60 |  |
| Standard | 862.5 | 1,932.00 | 2,593.25 | 1,897.50 | 2,484.00 | 2,294.25 | 2,616.25 |  |
| Siemens | --- | 2,357.50 | 2,788.75 | 2,461.00 | 2,857.75 | 2,719.75 | 3,174.00 |  |
| Indo Asian | 885.5 | 1,871.05 | 2,478.25 | 1,835.40 | 2,377.05 | 2,317.25 | 2,702.50 |  |
| Sputnik | 737.15 | --- | --- | --- | --- | --- | --- |  |
| Crystal | 961.4 | 2,160.85 | 2,917.55 | 2,395.45 | 2,777.25 | 2,509.30 | 3,045.20 |  |
| GECO | 737.15 | --- | --- | --- | --- | --- | --- |  |
| Sl.No. | 154 | 155 | 156 | 157 | 158 | 159 | 160 |  |
| Description | 40A 4 Pole 100 mA RCCB | 63A 4 Pole 100 mA RCCB | SPN DB 4way with IP-20 Protection. | SPN DB 8way with IP-20 Protection. | SPN DB 12 way with IP-20 Protection. | SPN DB 4 way with IP-42 Protection. | SPN DB 8 way with IP-42 Protection. |  |
| MDS Legrend | 3,322.35 | 3,846.75 | 907.35 | 1,083.30 | 1,337.45 | 1,324.80 | 1,604.25 |  |
| G. E | 3,085.45 | 3,556.95 | 776.25 | 905.05 | 1,099.40 | 1,035.00 | 1,293.75 |  |
| L & T Hager | 3,085.45 | 3,556.95 | 776.25 | 905.05 | 1,099.40 | 1,035.00 | 1,293.75 |  |
| MOELLER | 3,085.45 | 3,556.95 | 776.25 | 905.05 | 1,099.40 | --- | --- |  |
| Havells | 2,288.50 | 2,633.50 | 506 | 632.5 | 822.25 | 897 | 1,207.50 |  |
| HPL | 2,922.15 | 3,364.90 | 477.25 | 639.4 | 838.35 | 791.2 | 1,070.65 |  |
| Standard | 2,202.25 | 2,576.00 | 396.75 | 523.25 | 672.75 | 672.75 | 914.25 |  |
| Siemens | 2,823.25 | 3,266.00 | --- | --- | --- | --- | --- |  |
| Indo Asian | 2,235.60 | 2,580.60 | 426.65 | 557.75 | 726.8 | 691.15 | 948.75 |  |
| Sputnik | --- | --- | 303.6 | 400.2 | 549.7 | 510.6 | 633.65 |  |
| Crystal | 2,474.80 | 2,916.40 | 414 | 577.3 | 763.6 | 731.4 | 1,020.05 |  |
| Milltec | --- | --- | 303.6 | 400.2 | 549.7 | 510.6 | 633.65 |  |
| GECO | --- | --- | 303.6 | 400.2 | 549.7 | 510.6 | 633.65 |  |
| Sl.No. | 161 | 162 | 163 | 164 | 165 | 166 | 167 |  |
| Description | SPN DB 12 way with IP-42 Protection. | TPN DB 4 way with IP-20 Protection. | TPN DB 6 way with IP-20 Protection. | TPN DB 8 way with IP-20 Protection. | TPN DB 12 way with IP-20 Protection. | TPN DB 4 way with IP-42 Protection.Metal doors | TPN DB 6 way with IP-42 Protection. |  |
| MDS Legrend | 1,983.75 | 1,981.45 | 2,455.25 | 2,918.70 | 4,341.25 | 2,691.00 | 3,322.35 |  |
| G. E | 1,552.50 | 1,681.30 | --- | 2,457.55 | 3,686.90 | 2,166.60 | --- |  |
| L & T Hager | 1,552.50 | 1,681.30 | --- | 2,457.55 | 3,686.90 | 2,166.60 | --- |  |
| MOELLER | 1,552.50 | 1,681.30 | --- | 2,457.55 | 3,686.90 | 2,166.60 | --- |  |
| Havells | 1,650.25 | 1,322.50 | 1,581.25 | 1,742.25 | 2,265.50 | 2,737.00 | 3,461.50 |  |
| HPL | 1,420.25 | 1,583.55 | 1,886.00 | 2,211.45 | 3,085.45 | 2,176.95 | 2,735.85 |  |
| Standard | 1,178.75 | 1,299.50 | 1,558.25 | 1,736.50 | 2,484.00 | 1,891.75 | 2,455.25 |  |
| Siemens | --- | --- | --- | --- | --- | --- | --- |  |
| Indo Asian | 1,266.15 | 1,397.25 | 1,650.25 | 1,956.15 | 2,760.00 | 1,949.25 | 2,420.75 |  |
| Sputnik | 788.9 | 969.45 | 1,138.50 | 1,370.80 | 1,552.50 | 1,313.30 | 1,397.25 |  |
| Crystal | 1,345.50 | 1,461.65 | 1,811.25 | 2,160.85 | 2,975.05 | 2,043.55 | 2,626.60 |  |
| Milltec | 788.9 | 969.45 | 1,023.50 | 1,370.80 | 1,552.50 | --- | --- |  |
| GECO | 788.9 | 969.45 | 1,138.50 | 1,370.80 | 1,552.50 | 1,313.30 | 1,397.25 |  |
| Sl.No. | 168 | 169 | 170 | 171 | 172 | 173 | 174 |  |
| Description | TPN DB 8 way with IP-42 Protection. | TPN DB 12 way with IP-42 Protection. | TPN Vertical type DB 4 way with IP-20 Protection. | TPN Vertical type DB 6 way with IP-20 Protection. | TPN Vertical type DB 8 way with IP-20 Protection. | TPN Vertical type DB 12 way with IP-20 Protection. | TPN Vertical type DB 4 way with IP-42 Protection. |  |
| MDS Legrend | 3,979.00 | 5,868.45 | 5,330.25 | --- | 6,574.55 | 9,570.30 | 6,417.00 |  |
| G. E | 3,169.40 | 4,721.90 | --- | --- | --- | --- | --- |  |
| L & T Hager | 3,169.40 | 4,721.90 | --- | --- | --- | --- | --- |  |
| MOELLER | 3,169.40 | 4,721.90 | --- | --- | --- | --- | --- |  |
| Havells | 3,950.25 | --- | 5,951.25 | --- | 7,101.25 | 8,159.25 | 7,342.75 |  |
| HPL | 3,376.40 | 4,726.50 | 4,016.95 | --- | 5,309.55 | 6,484.85 | 5,589.00 |  |
| Standard | 2,754.25 | 4,122.75 | 4,105.50 | --- | 5,249.75 | 6,388.25 | 5,290.00 |  |
| Siemens | --- | --- | --- | --- | --- | --- | 5,543.00 |  |
| Indo Asian | 3,018.75 | 4,260.75 | 3,231.50 | --- | 4,358.50 | 5,545.30 | 46,010.35 |  |
| Sputnik | 1,500.75 | 1,875.65 | --- | --- | --- | --- | --- |  |
| Crystal | 3,324.65 | 4,605.75 | --- | --- | --- | --- | --- |  |
| GECO | 1,500.75 | 1,875.65 | --- | --- | --- | --- | --- |  |
| Sl.No. | 175 | 176 | 177 | 178 | 179 | 180 | 181 | 182 |
| Description | TPN Vertical type DB 6 way with IP-42 Protection. | TPN Vertical type DB 8 way with IP-42 Protection. | TPN Vertical type DB 12 way with IP-42 Protection. | TPN DB with IP-42 Protection 4way 7 segment PSS 4way | TPN DB with IP-42 Protection 8way 7 segment PSS | SPPlug and Socket in SS enclosure 20A | 3 Phase Plug and Socket in SS enclosure 20A | 3 Phase Plug and Socket in SS enclosure 32A |
| MDS Legrend | --- | 8,127.00 | 10,880.00 | 6,336.00 | 7,974.00 | 1,216.00 | 2,230.00 | 2,639.00 |
| G. E | --- | --- | --- | 5,304.00 | 6,533.00 | 905.00 | --- | 2,070.00 |
| L & T Hager | --- | --- | --- | 5,304.00 | 6,533.00 | 905.00 | --- | 2,070.00 |
| MOELLER | --- | --- | --- | 5,304.00 | 6,533.00 | 905.00 | --- | 2,070.00 |
| Havells | --- | 8,021.00 | 9,217.00 | --- | --- | 626.00 | 1,196.00 | 1,667.00 |
| HPL | --- | 6,403.00 | 7,684.00 | --- | --- | 687.00 | 1,304.00 | 1,793.00 |
| Standard | --- | 6,601.00 | 8,136.00 | 4,916.00 | 6,723.00 | 586.00 | 1,109.00 | 1,500.00 |
| Siemens | 6,417.00 | 7,011.00 | 9,372.00 | --- | --- | 821.00 | --- | 1,674.00 |
| Indo Asian | --- | 6,601.00 | 8,136.00 | 4,916.00 | 6,723.00 | 586.00 | 1,109.00 | 1,500.00 |
| Sputnik | --- | --- | --- | --- | --- | 517.00 | 808.00 | 1,099.00 |
| Crystal | --- | --- | --- | --- | --- | 687.00 | --- | 1,229.00 |
| GECO | --- | --- | --- | --- | --- | 517.00 | 808.00 | 1,099.00 |
|  | Accessories to Switch Gears Rate per Each |  |  |  |  |  |  |  |
| Sl.No. | 183 | 184 | 185 | 186 | 187 | 188 | 189 |  |
| Description | 240V D.P 32A Sheet Steel Changeover switch | 415V, F.P 16A S.S.Changeover switch | 415V, F.P 32A S.S.Changeover switch | 415V, F.P 100A S.S.Changeover switch | 240V, SPN, 32A Rewirable fuse switch | 415V, TPN, 16A Rewirable fuse switch | 415V, TPN, 32A Rewirable fuse switch in SS enclouser |  |
| L & T | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| G. E | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| C & S | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| MOELLER | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Havells | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| H. P. L | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Standard | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Crystal | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Milltec | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Sputnik | 891.00 | 1,297.00 | 1,560.00 | 6,420.00 | 684.00 | 775.00 | 1,085.00 |  |
| Sl.No. | 190 | 191 | 192 | 193 | 194 | 195 | 196 |  |
| Description | 415V, TPN, 63A Rewirable fuse switch in SS enclouser | 415V, TPN, 100A Rewirable fuse switch in SS enclouser | 415V, TPN, 100A Double break switch in SS enclouser | 415V, TPN, 200A Double break switch in SS enclouser | HRC Fuse links upto 32A | HRC Fuse links 32A to 63A | HRC Fuse links 125A to 200A |  |
| L & T | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| G. E | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| C & S | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| MOELLER | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| Havells | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| HPL | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| Standard | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| Crystal | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| Milltec | 2,324.60 | 4,466.30 | --- | --- | --- | --- | --- |  |
| Sputnik | 2,324.60 | 4,466.30 | 4,637.40 | 6,543.10 | 29.5 | 67.85 | 213.93 |  |
| Sl.No. | 197 | 198 | 199 | 200 | 201 | 202 | 203 |  |
| Description | HRC Fuse links 250A to 315A | HRC Fuse links 400A | HRC Fuse links 630A | 240V, 32A porcelain rewirable fuse units. | 415V, 32A porcelain rewirable fuse units. | 415V, 63A porcelain rewirable fuse units. | 415V, 100A porcelain rewirable fuse units. |  |
| L & T | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| G. E | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| C & S | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| MOELLER | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| Havells | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| H. P. L | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| Standard | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| Crystal | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| Sputnik | 299.72 | 613.60 | 690.30 | 88.50 | 109.74 | 184.08 | 339.84 |  |
| Sl.No. | 204 | 205 |  |  |  |  |  |  |
| Description | 415V, 200A porcelain rewirable fuse units. | 415V, 320A porcelain rewirable fuse units. |  |  |  |  |  |  |
| L & T | 692.66 | 1,082.06 |  |  |  |  |  |  |
| G. E | 692.66 | 1,082.06 |  |  |  |  |  |  |
| C & S | 692.66 | 1,082.06 |  |  |  |  |  |  |
| MOELLER | 692.66 | 1,082.06 |  |  |  |  |  |  |
| Havells | 692.66 | 1,082.06 |  |  |  |  |  |  |
| H. P. L | 692.66 | 1,082.06 |  |  |  |  |  |  |
| Standard | 692.66 | 1,082.06 |  |  |  |  |  |  |
| Crystal | 692.66 | 1,082.06 |  |  |  |  |  |  |
| Sputnik | 692.66 | 1,082.06 |  |  |  |  |  |  |
| Voltage Surge Protector(RATE PER EACH) |  |  |  |  |  |  |  |  |
| Sl.No. | 206 | 207 | 208 | 209 | 210 | 211 | 212 |  |
| Description | 1 pole ( 1 Module ) 40 KA, 1.4 KV | 2 pole ( 2 Module ) 40 KA, 1.4 KV | 4 pole ( 4 Module ) 40 KA, 1.4 KV | 1 pole ( 1 Module ) 15 KA, 1.2 KV | 2 pole ( 2 Module ) 15 KA, 1.2 KV | 4 pole ( 4 Module ) 15 KA, 1.2 KV | Lexic changeover switch 20A, double pole 2 way, 400V A.C. (Suitable to fix on din channel ) |  |
| L & T | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| G. E | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| C & S | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| MOELLER | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| Havells | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| H. P. L | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| Standard | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| Crystal | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| Sputnik | 4,422.64 | 8,047.60 | 15,131.14 | 3,537.64 | 6,438.08 | 12,106.80 | 1,690.94 |  |
| Sl.No. | 213 | 214 | 215 | 216 | 217 | 218 | 219 |  |
| Description | 63A Neutral link | 100A Neutral link | 200A Neutral link | 20A, Bakelite fuse units | 32A, Bakelite fuse units | 63A Bakelite fuse units | 100A Bakelite fuse units |  |
| L & T | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| G. E | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| C & S | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| MOELLER | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| Havells | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| H. P. L | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| Standard | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| Crystal | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| Sputnik | 59.00 | 118.00 | 236.00 | 82.60 | 141.60 | 177.00 | 442.50 |  |
| V. Earthing |  |  |  |  |  |  |  |  |
| The rates for earthing are coverd in Miscellaneous item no. XIX and balance items as per Civil SSR |  |  |  |  |  |  |  |  |
| VI. Service Mains & L. T. O. H Lines(RATE PER 100 MTRS) |  |  |  |  |  |  |  |  |
| S. No | Description |  | SUN LIGHT | Gold Medal | Million | Payal | Power Flex | Miltec |
| 220 | 2.5 Sq.mm WPSC (3/20) |  | 606 | 606 | 606 | 606 | 606 | 606 |
| 221 | 6 Sq.mm WPSC (7/18) |  | 1,332 | 1,332 | 1,152 | 1,332 | --- | --- |
| 222 | 10 Sq.mm WPSC |  | --- | --- | --- | 1,488 | --- | --- |
| 223 | 16 Sq.mm WPSC |  | 2,473 | 2,473 | 2,270 | --- | --- | --- |
| 224 | 6 Sq.mm WPTC |  | 2,626 | 2,626 | 2,406 | 2,626 | --- | --- |
| 225 | 10 Sq.mm WPTC |  | --- | --- | --- | 3,620 | --- | --- |
| 226 | 2 Pair Telephone wire |  | 1,110 | 1,110 | 1,032 | 1,110 | --- | --- |
| 227 | 6 Pair Telephone wire |  | 3,391 | 3,391 | 3,163 | 3,391 | --- | --- |
| 228 | Co-axial cable RG-06 |  | 1,852 | 1,852 | 880 | 1,852 | --- | --- |
| 229 | 23/0060 Twin core flat wire |  | 812 | 812 | 726 | --- | --- | --- |
| 230 | 3 core 1.5 Sqmm submercible cable |  | 5,454 | 5,454 | 5,166 | 5,534 | 8,000 | 5,995 |
| 231 | 3 core 2.5 Sqmm submercible cable |  | 8,392 | 8,392 | 8,017 | 8,504 | 12,456 | 9,060 |
| 232 | 3 core 4 Sq.mm sub mersible cable. |  | 12,635 | 12,635 | 12,276 | 12,824 | 19,368 | 14,068 |
| 233 | 3 core 6.0 Sqmm submercible cable |  | 17,712 | 17,712 | 18,414 | 19,844 | 29,263 | 19,590 |
| 234 | 3 core 10 Sqmm submercible cable |  | 29,132 | 29,132 | --- | 31,860 | --- | --- |
| 235 | Lan cable (Cat-5) |  | 2,284 | 2,284 | --- | --- | --- | --- |
| VII. Flourescent, HPSV, M. H Street lights, Post Top, Well glass & Flood Light Luminaries |  |  |  |  |  |  |  |  |
| STREET LIGHTS, M.V., S.V. AND MH LUMINARIES Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. | 236 | 237 | 238 | 239 | 240 | 241 | 242 |  |
| Description | 1X40W STl | 2 x 11w CFL (STL) | 1 x 18w CFL (STL) | 80W MV luminarie | 125W MV Luminaire | 70W SV Luminaire | 150W SV Luminaire |  |
| G. E | 1,423.40 | --- | --- | --- | 2,722.50 | 3,062.40 | 4,516.60 |  |
| Wipro | 1,375.00 | --- | --- | 2,640.00 | 2,722.50 | 3,062.40 | 4,516.60 |  |
| Photon | 1,447.60 | 1,373.90 | 1,237.50 | 3,031.60 | 3,055.80 | 3,873.10 | 4,999.50 |  |
| Havells | 1,546.60 | 2,505.80 | 1,237.50 | --- | 3,830.20 | 4,324.10 | 4,596.90 |  |
| Phillips | 1,620.30 | 2,487.10 | --- | --- | 3,168.00 | 3,576.10 | 7,201.70 |  |
| C. G | 1,617.00 | 2,481.60 | --- | --- | 3,162.50 | 3,564.00 | 7,183.00 |  |
| Bajaj | 1,447.60 | 1,373.90 | 1,237.50 | 3,031.60 | 3,055.80 | 3,873.10 | 4,999.50 |  |
| R. K | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Krishlite | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Fixolite | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Ballast Makers | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Kumar | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Kushima Luminars | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Indo Asian | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Flouracraft | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Kushima Luminars | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Indo Asian | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Flouracraft | 1,134.10 | 1,261.70 | 1,261.70 | 2,116.40 | 2,227.50 | 2,524.50 | 3,712.50 |  |
| Sl.No. | 243 | 244 | 245 | 246 | 247 | 248 | 249 |  |
| Description | 250W SV Luminaire | 250W SV IP 65/55 | 70W MH Luminarie | 150W MH Luminarie | 250W MH Luminarie | 250W MH IP 65/66 Luminarie | 2x400W MH Flood light reflector with control gear (High mast) |  |
| G. E | 4,900.50 | 5,902.60 | --- | --- | 6,619.80 | --- | Please refer item No. 306 for rates |  |
| Wipro | 4,900.50 | 5,902.60 | --- | --- | 6,619.80 | --- |  |  |
| Photon | 6,273.30 | 6,892.60 | 3,545.30 | 4,999.50 | 6,273.30 | 6,892.60 |  |  |
| Havells | 5,197.50 | 6,744.10 | 4,386.80 | 6,237.00 | 7,115.90 | 7,920.00 |  |  |
| Phillips | 7,635.10 | 8,637.20 | --- | --- | --- | 8,340.20 |  |  |
| C. G | 7,612.00 | 8,625.10 | --- | --- | --- | 8,274.20 |  |  |
| Bajaj | 6,273.30 | 6,892.60 | 3,545.30 | 4,999.50 | 6,273.30 | 6,892.60 |  |  |
| R. K | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Krishlite | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Ballast Makers | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Fixolite | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Kumar | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Kushima Luminars | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Indo Asian | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Flouracraft | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Kushima Luminars | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Indo Asian | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Flouracraft | 4,187.70 | --- | 3,267.00 | 5,049.00 | 5,791.50 | --- |  |  |
| Well Glass and Low Bay Luminaries Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. | 250 | 251 |  |  |  |  |  |  |
| Description | 80W MV well glass luminaire | 70W SV well glass luminaire |  |  |  |  |  |  |
| G. E | --- | 2,772.00 |  |  |  |  |  |  |
| Wipro | 2,623.50 | 2,673.00 |  |  |  |  |  |  |
| Havells | --- | 2,963.40 |  |  |  |  |  |  |
| Phillips | 2,623.50 | 2,673.00 |  |  |  |  |  |  |
| C. G | 2,376.00 | 2,598.20 |  |  |  |  |  |  |
| Bajaj | 2,376.00 | 2,598.20 |  |  |  |  |  |  |
| R. K | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Ballast Makers | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Krishlite | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Fixolite | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Kumar | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Kushima Luminars | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Indo Asian | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Flouracraft | 2,227.50 | 2,709.30 |  |  |  |  |  |  |
| Post top Lantern Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. | 252 | 253 | 254 | 255 | 256 | 257 | 258 |  |
| Description | 1 X 11W post Top CFL Luminaire | 80w MV standard post top lantern luminaire | 125w MV standard post top lantern luminaire | 70w SV standard post top lantern luminaire | 80w MV Sphear / Diamond post Top luminaire | 125w MV Sphear / Diamond post Top luminaire | 70w SV Sphear / Diamond post Top luminaire |  |
| G. E | --- | --- | --- | --- | --- | --- | --- |  |
| Wipro | --- | --- | --- | --- | 3,241.70 | 3,241.70 | 3,489.20 |  |
| Havells | --- | --- | --- | --- | --- | --- | --- |  |
| Phillips | --- | --- | --- | --- | 3,241.70 | 3,550.80 | 3,650.90 |  |
| C. G | 3,898.40 | 2,140.60 | 2,215.40 | 2,425.50 | 3,241.70 | 3,241.70 | 3,489.20 |  |
| Bajaj | 3,898.40 | 2,140.60 | 2,215.40 | 2,425.50 | 3,241.70 | 3,241.70 | 3,489.20 |  |
| Photon | 3,898.40 | 2,140.60 | 2,215.40 | 2,425.50 | 3,241.70 | 3,241.70 | 3,489.20 |  |
| R. K | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Krishlite | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Ballast Makers | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Fixolite | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Kumar | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Kushima Luminars | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Indo Asian | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Flouracraft | --- | 2,041.60 | 2,152.70 | 249.7 | 2,376.00 | 2,487.10 | 2,673.00 |  |
| Sl.No. |  | 259 | 260 | 261 | 262 | 263 | 264 |  |
| Description |  | Sonora Post top 70 W Mh | Bollards N. G. C. 100 type GLS 60 W | Bollards F. G. C. 100/109 1 x 9 W | Bollards F. G. C. 100/109 1 x 18 W | Bollards Wall Mount 1 x 18 W | Bollards 1 x 70 W Sodium |  |
| G. E |  | --- | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| Wipro |  | --- | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| Havells |  | --- | --- | --- | --- | --- | --- |  |
| Phillips |  | 6,793.60 | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| C. G |  | --- | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| Bajaj |  | --- | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| Photon |  | --- | 3,353.90 | 3,551.90 | 4,727.80 | 2,734.60 | 8,118.00 |  |
| Hallogen / M. H Flood Light Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. |  |  |  | 265 | 266 | 267 | 268 |  |
| Description |  |  |  | 500W Hallozen Luminaire IP-54 | 1000w Hallozen Luminaire IP-54 | 250w MH Luminaire | 400w MH Luminaire |  |
| G. E |  |  |  | 877.80 | 1,794.10 | 7,090.60 | 7,672.50 |  |
| Wipro |  |  |  | 877.80 | 1,794.10 | 7,090.60 | 7,672.50 |  |
| Havells |  |  |  | 618.20 | 1,485.00 | 6,435.00 | 7,114.80 |  |
| Phillips |  |  |  | 1,039.50 | 2,091.10 | 8,019.00 | 8,302.80 |  |
| C. G |  |  |  | 1,039.50 | 1,855.70 | 6,718.80 | 7,375.50 |  |
| Bajaj |  |  |  | 928.40 | 1,855.70 | 6,718.80 | 7,375.50 |  |
| Photon |  |  |  | 928.40 | 1,855.70 | 6,718.80 | 7,375.50 |  |
| R. K |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Ballast Makers |  |  |  | 816.20 | 1,039.50 | 6,718.80 | 7,128.00 |  |
| Krishlite |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Fixolite |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Kumar |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Kushima Luminars |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Indo Asian |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Flouracraft |  |  |  | 816.20 | 1,039.50 | 6,534.00 | 7,128.00 |  |
| Energy Saving Led Light Fitting Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. |  |  | 269 | 270 | 271 | 272 | 273 |  |
| Description |  |  | 15 Watts Led light fitting (equavalent to 54 w) | 22 Watts led street light fittings (equavalent to 90 W) | 30 Watts led street light fittings (equavalent to 180 W) | 42 Watts led street light fittings (equavalent to 280 W) | 60 Watts led street light fittings (equavalent to 440 W) |  |
| Kushima Luminars |  |  | 3,203.20 | 9,152.00 | 11,440.00 | 16,016.00 | 22,880.00 |  |
| Accessories to Street lights Rate per Each |  |  |  |  |  |  |  |  |
| Sl.No. | 274 | 275 | 276 | 277 | 278 | 279 | 280 |  |
| Description | 80W M.V. Copper Ballast | 125W M.V. Copper Ballast | 250W M.V. Copper Ballast | 70W S.V. / M.H Copper Ballast | 150W S.V. / M.H Copper Ballast | 250W S.V. / M.H Copper Ballast | 400W S.V. / M.H Copper Ballast |  |
| Phillips | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| Bajaj | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| Crompton | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| Havells | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| G. E | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| Wipro | 627.00 | 693.00 | 1,023.00 | 792.00 | 1,199.00 | 1,507.00 | 2,090.00 |  |
| R. K. | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Krishlite | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Kushima luminaries | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Ballast Makers | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Fixolite | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Kumar | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Indo Asian | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Florocraft | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Photon | 605.00 | 627.00 | 935.00 | 638.00 | 936.10 | 1,460.80 | 2,090.00 |  |
| Sl.No. | 281 | 282 | 283 | 284 | 285 | 286 | 287 |  |
| Description | Ignitor for S.V / M.H Lamp | HPF Capacitor 4 mfd | HPF Capacitor 10 mfd | HPF Capacitor 15 mfd | HPF Capacitor 20 mfd | HPF Capacitor 33 mfd | HPF Capacitor 42 mfd |  |
| Phillips | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| Bajaj | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| Crompton | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| Havells | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| G. E | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| Wipro | 321.20 | 117.70 | 198.00 | 247.50 | 327.80 | 556.60 | 618.20 |  |
| R. K. | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Krishlite | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Ballast Makers | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Kushima luminaries | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Fixolite | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Kumar | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Indo Asian | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Florocraft | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Photon | 165.00 | 88.00 | 132.00 | 181.50 | 247.50 | 440.00 | 462.00 |  |
| Sl.No. | 288 | 289 | 290 | 291 | 292 | 293 | 294 |  |
| Description | 500W Halogen Lamp | 1000W Halogen Lamp | 70W S.V Lamp | 150W S.V Lamp | 250W S.V Lamp | 400W S.V Lamp | 70W double end M.H Lamp |  |
| Phillips | 108.90 | 132.00 | 495.00 | 715.00 | 770.00 | 847.00 | 931.70 |  |
| Bajaj | 108.90 | 132.00 | 495.00 | 715.00 | 770.00 | 847.00 | 931.70 |  |
| Crompton | 108.90 | 132.00 | 495.00 | 715.00 | 770.00 | 847.00 | 931.70 |  |
| Sl.No. | 295 | 296 | 297 | 298 | 299 | 300 | 301 |  |
| Description | 150W double end M.H Lamp | 70W M.H Lamp | 150W M.H Lamp | 250W M.H Lamp | 400W M.H Lamp | 80W M.V Lamp | 125W M.V Lamp |  |
| Phillips | 605 | 913 | 990 | 1,353.00 | 1,540.00 | 179.3 | 187 |  |
| Bajaj | 605 | 913 | 990 | 1,353.00 | 1,540.00 | 179.3 | 187 |  |
| Crompton | 605 | 913 | 990 | 1,353.00 | 1,540.00 | 179.3 | 187 |  |
| Sl.No. | 302 | 303 | 304 | 305(a) | 305 | 306(a) | 306 |  |
| Description | 250W M.V Lamp | 400W M.V Lamp | MLL 160W Lamp | 1X250 MH Non-integral flood light luminaire with control gear, without lamp | 2X250 MH Non-integral flood light luminaire with control gear, without lamp (for high mast) | 1X400 MH Non-integral flood light luminaire with control gear without lamps | 2X400 MH Non-integral flood light luminaire with control gear without lamps (For High mast). |  |
| Phillips | 456.5 | 643.5 | 239.8 | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| Bajaj | 456.5 | 643.5 | 239.8 | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| Crompton | 456.5 | 643.5 | 239.8 | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| Havells | --- | --- | --- | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| G. E | --- | --- | --- | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| Wipro | --- | --- | --- | 14,419.00 | 16,545.10 | 14,578.00 | 17,395.40 |  |
| R. K. | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Krishlite | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Kushima luminaries | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Fixolite | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Kumar | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Indo Asian | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Florocraft | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
| Photon | --- | --- | --- | 9,270.00 | 12,177.00 | 9,727.00 | 13,068.00 |  |
|  | Supply of Swadeep Nature switchs.(Rate per Each) |  |  |  |  |  | Kakatiya Energy Saving System |  |
| 307 | Swadeep - 01 Independent Model |  |  |  |  |  | 2,517.00 |  |
| 308 | Swadeep - 02 Independent Model |  |  |  |  |  | 2,974.00 |  |
| 309 | Sign switch |  |  |  |  |  | 3,318.00 |  |
| 310 | Swadeep - 02 EC 5KW 1 Ph. |  |  |  |  |  | 9,724.00 |  |
| 311 | Swadeep - 02 EC 8KW 1 Ph. |  |  |  |  |  | 10,868.00 |  |
| 312 | Swadeep - 02 EC 5KW 3 Ph. |  |  |  |  |  | 10,754.00 |  |
| 313 | Swadeep - 02 EC 8KW 3 Ph. |  |  |  |  |  | 12,470.00 |  |
| 314 | Swadeep - 02 EC 12KW 3 Ph. |  |  |  |  |  | 13,842.00 |  |
| 315 | Supply of single fancy bracket with glass shade best Indian make |  |  |  |  |  | 215.00 |  |
| 316 | Supply of double fancy bracket with glass shade best Indian make |  |  |  |  |  | 385.00 |  |
| 317 | Supply of 10" ceiling fittings with glass doom best Indian make |  |  |  |  |  | 200.00 |  |
| 318 | Supply of Bulk head fitting with cast aluminium enamel painted body with water tight, clear glass with wire guard hinged with lock and key arrangements etc., complete best Indian make |  |  |  |  |  | 410.00 |  |
| VIII. Internal Luminaries, Mirror Optic, T - 5 CFL Down Lights etc., Rate per each |  |  |  |  |  |  |  |  |
| Sl.No. | 319 | 320 | 321 | 322 | 323 | 324 | 325 |  |
| Description | Supply of 1x11W mirror light with all standard accessories diffuser | Supply of 1x36/40W box type tube light luminaire with copper / VPIT Choke, condensor, starters. | Supply of 2x36/40W box type tube light luminaire with 2Nos copper / VPIT chokes, starters, condensor. | Supply of 1x40W patty type tube light luminaire with copper choke, starter etc. | Supply of 1x20W patty type tube light luminaire with copper choke, starter etc. | Supply of 1x36 / 40W industrial box type luminaire with Copper / VPIT Choke, Condensor, Starters. | Supply of 2x36 / 40W industrial box type luminaire with 2Nos Copper / VPIT Choke, Condensor, Starters with reflector |  |
| Phillips | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| Bajaj | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| Crompton | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| Havells | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| G. E | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| Wipro | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| Phuton ISI | 643.5 | 660 | 1,075.80 | 352 | --- | 1,441.00 | 1,793.00 |  |
| R. K. | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Krishlite | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Kushima luminaries | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Ballast Makers | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Fixolite | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Kumar | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Indo Asian | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Florocraft | 517 | 544.5 | 808.5 | 321.2 | 321.2 | 1,020.80 | 1,424.50 |  |
| Morson | 0 | 544.5 | 808.5 | 321.2 | 321.2 | --- | --- |  |
| Sl.No. | 326 | 327 | 328 | 329 | 330 | 331 | 332 |  |
| Description | 2x36 / 40W surface mounting mirror optic luminaire with powder coated CRCA sheet steel housing with aluminium mirror reflectors with VPIT chokes, Starters, Condensor etc. | 2x36/40W recessed mounting mirror optic (darklite) luminaire with powder coated CRCA sheet steel housing with aluminium mirror reflector with VPIT Chokes, Starter, Condensor etc. | 2x11W surface mounting mirror optic luminaire with powder coated CRCA sheet steel housing with aluminium mirror refector with VPIT Chokes | 2x11W Recessed type luminaire with powder coated CRCA sheet steel housing with aluminium mirror refector with VPIT Chokes. | 2x36W CFL paralite louvre (P5) surface low glare mirror optic luminaire comprising bright anodised aluminium reflector with VPIT Chokes. | 2x36W CFL paralite louvre (P5) recessed low glare mirror optic luminaire comprising bright anodised aluminium reflector with VPIT Chokes | 1x28W surface pendent mounting T-5 extruded allu. Reflector luminaire with all standard accessories etc. |  |
| Phillips | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| Bajaj | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| Crompton | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| Havells | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| G. E | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| Wipro | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| Phuton ISI | 2,783.00 | 3,366.00 | 1,485.00 | 1,485.00 | 4,950.00 | 4,950.00 | 1,898.60 |  |
| R. K. | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Ballast Makers | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Krishlite | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Kushima luminaries | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Fixolite | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Kumar | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Indo Asian | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Florocraft | 1,881.00 | 1,881.00 | 1,083.50 | 1,083.50 | --- | --- | --- |  |
| Sl.No. | 333 | 334 | 335 | 336 | 337 | 338 | 339 |  |
| Description | 2X28W recessed pendant mounting low glare T- 5 mirror optic luminaire with electronic Ballasts etc., | Recessed mounting 1x13W CFL down light with all standard accessories. | Recessed mounting 2x13W CFL down light with all standard accessories. | Recessed mounting 1x18W CFL down light with all standard accessories. | Recessed mounting 2x18W CFL down light with all standard accessories. | Recess mounting 70W metal hallide down light with control gear. | Ceiling / Wall mounted Circular 1x13W luminaire with decorative acrylic diffuser and 1 No. 13W CFL lamp (circular type) etc., complete. |  |
| Phillips | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| Bajaj | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| Crompton | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| Havells | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| G. E | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| Wipro | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | 2,146.10 |  |
| Photon | 6,001.60 | 928.4 | 1,175.90 | 1,175.90 | 1,608.20 | 2,722.50 | --- |  |
| Sl.No. | 340 | 341 | 342 | 343 | 344 | 345 | 346 |  |
| Description | Ceiling / Wall mounted Circular 2x13W luminaire with decorative acrylic diffuser and 2 No. 13W CFL lamp (circular type) etc., complete. | Recess mounting 12v, 50w adjustable halogen down light with white / colour / Gold Shade including transformer | Recess mounting 12v, 50w Zoom type halogen down light with white / colour Shade including transformer. | VPIT 9/13w Ballast for CFL | VPIT 18w Ballast for CFL | 20w copper wound Ballast | 40w copper wound Ballast |  |
| Phillips | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| Bajaj | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| Crompton | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| Havells | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| G. E | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| Wipro | 2,530.00 | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| Photon | --- | 742.50 | 979.00 | 174.90 | 179.30 | 220 | 220 |  |
| R. K. | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Ballast Makers | --- | --- | --- | 174.90 | 179.30 | 220 | 220 |  |
| Krishlite | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Kushima luminaries | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Fixolite | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Kumar | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Indo Asian | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Florocraft | --- | --- | --- | 136.40 | 136.40 | 220 | 220 |  |
| Morson | --- | --- | --- | --- | --- | 220 | 220 |  |
| Sl.No. | 347 | 348 | 349 | 350 | 351 | 352 | 353 |  |
| Description | VPIT 18/20w Ballast | VPIT 36/40w Ballast | 40w Starter. | 20w allum. Wound ballast | 40w allum. Wound ballast | Rotor holder (FTL) | HID Lamp holder |  |
| Phillips | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| Bajaj | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| Crompton | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| Havells | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| G. E | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| Wipro | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | 22.00 | 66 |  |
| Photon | 215.60 | 215.60 | 12.10 | 148.50 | 148.50 | --- | --- |  |
| R. K. | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Ballast Makers | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Krishlite | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Kushima luminaries | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Fixolite | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Kumar | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Indo Asian | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Florocraft | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Morson | 199.10 | 199.10 | 7.70 | 116.60 | 116.60 | 6.60 | 44 |  |
| Sl.No. | 354 | 355 | 356 |  |  |  |  |  |
| Description | CFL Holder | FTL Starter Holder | Electronic ballast for 36w FTL |  |  |  |  |  |
| Phillips | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Bajaj | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Crompton | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Havells | 33 | 13.20 | 606.10 |  |  |  |  |  |
| G. E | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Wipro | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Photon | --- | --- | 606.10 |  |  |  |  |  |
| R. K. | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Ballast Makers | 33 | 13.20 | 606.10 |  |  |  |  |  |
| Krishlite | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Kushima luminaries | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Fixolite | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Kumar | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Indo Asian | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Florocraft | 23.10 | 8.80 | 467.50 |  |  |  |  |  |
| Morson | 23.10 | 6.60 | --- |  |  |  |  |  |
| Sl.No. | 357 | 358 | 359 | 360 | 361 | 362 | 363 |  |
| Description | 40-60w Lamp | 100w lamp | 40-60w Milky lamp | Night Lamp | 18 / 20w Flourescent Lamp | 36 / 40w Flourescent Lamp | 9w CFL |  |
| Phillips | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| Bajaj | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| Crompton | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| Havells | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| G. E | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| Wipro | 12.10 | 13.20 | 14.30 | 12.10 | 47.30 | 50.60 | 82.50 |  |
| HPL | --- | --- | --- | --- | --- | --- | 82.50 |  |
| Sl.No. | 364 | 365 | 366 | 367 | 368 | 369 | 370 |  |
| Description | 11w CFL | 13w CFL | 18w CFL | 26w CFL | 36w CFL | 5w-7w CFL with B-22 Caps best Indian make | 9w CFL with B-22 Caps best Indian make |  |
| Phillips | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | 143 |  |
| Bajaj | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| Crompton | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| Havells | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| G. E | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| HPL | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| Wipro | 88 | 118.80 | 181.50 | 165 | 198 | 137.50 | --- |  |
| Sl.No. | 371 | 372 | 373 | 374 | 375 | 376 | 377 |  |
| Description | 11w CFL with B-22 Caps best Indian make | Under water luminaire | 4 x 18 w Recess mounting C5 luminair | 2 x 18 w Recess mounting P5 luminair | 2 x 36 w Recess mounting P5 E. B. W luminair | 3 x 36 w Recess mounting P5 E. B. W luminair | 4 x 36 w Recess mounting D6 luminair |  |
| Phillips | 154 | 5,110.60 | 4,789.40 | 3,712.50 | 5,742.00 | 6,744.10 | 8,500.80 |  |
| Sl.No. | 378 | 379 | 380 |  |  |  |  |  |
| Description | 2 x 36 w Recess mounting P5 luminair | 4 x 14 w Recess mounting FNS D6 MMHF luminair | 2 x 55w Recess mounting FNS D6 MMHF luminair |  |  |  |  |  |
| Phillips | 4,630.00 | 7,080.00 | 7,720.00 |  |  |  |  |  |
| S.No. | The follwong rates are for Trinik Make(Rate per Each) and For the makes Phillips /Crompton/Havells/Bajaj / Ballast Makers etc., Prevailing market rates may be adopted as on 01-06-2008. |  |  |  |  |  |  |  |
| 381 | 1x28W energy conserving Electronic ordinary patty type powder coated. Flourescent fitting with 28W electronic ballast of P+ 0.90. |  |  |  |  |  | 1,017.50 |  |
| 382 | --do-- Box type --do-- |  |  |  |  |  | 1,155.00 |  |
| 383 | --do-- Industrial Box type --do-- |  |  |  |  |  | 2,200.00 |  |
| 384 | --do- surface mounted Mirror optic --do-- (suitable for 1x36 / 40w) |  |  |  |  |  | 2,310.00 |  |
| 385 | --do- surface mounted Mirror optic --do-- (suitable for 2x36 / 40w) |  |  |  |  |  | 3,135.00 |  |
| 386 | --do-- recess mounted Mirror optic (suitable for 1x36 / 40W) |  |  |  |  |  | 3,300.00 |  |
| 387 | --do-- recess mounted mirror optic (Suitable for 2 x36 / 40w) |  |  |  |  |  | 3,850.00 |  |
|  | C. F. L Rate per Each |  |  |  |  |  |  |  |
| Sl.No. | Description |  | G. E | Standard | HPL | Phillips | Bajaj |  |
| 388 | Retrofit Tiny CFL upto 5 W |  | 121 | 123.20 | 150 | 110 | 121 |  |
| 389 | Retrofit Tiny CFL upto 8 W |  | 137.5 | 137.50 | 140 | 110 | 121 |  |
| 390 | Retrofit Tiny CFL upto 10/11 W |  | 143 | 148.50 | 160 | 110 | 121 |  |
| 391 | Retrofit Tiny CFL upto 13/14 W |  | --- | 159.50 | 145 | 110 | 121 |  |
| 392 | Retrofit CFL upto 15 W |  | 159.50 | 231 | 254.10 | 187 | 121 |  |
| 393 | Retrofit CFL upto 18/20 W |  | 187 | 247.50 | 272.25 | 170.50 | 121 |  |
| 394 | Retrofit Tiny CFL upto 23 W |  | 220 | 264 | 290.4 | 220 | 121 |  |
| 395 | Retrofit Tiny CFL upto 25/26 W |  | 253 | 438.90 | 482.79 | 220 | 121 |  |
| 396 | Non Retrofit CFL upto 5 W |  | 71.50 | 71.50 | 78.65 | 71.50 | 121 |  |
| 397 | Non Retrofit CFL upto 7 W |  | 77 | 95 | 104.50 | 104.50 | 121 |  |
| 398 | Non Retrofit CFL upto 9 W |  | 82.50 | 82.50 | 90.75 | 104.50 | 110 |  |
| 399 | Non Retrofit CFL upto 11 W |  | 88 | 88 | 96.8 | 110 | 116 |  |
| 400 | Non Retrofit CFL upto 10/13 W |  | 126.50 | 118.80 | 130.68 | 143 | 143 |  |
| 401 | Non Retrofit CFL upto 18 W |  | 137.50 | 123.20 | 135.52 | 176 | 176 |  |
|  | IX. Air Conditioning, Refrigeration, Stablizers, Fans and Exhaust Fans Rate per each |  |  |  |  |  |  |  |
|  | Repairs to Air Conditioners Rate per Each |  |  |  |  |  |  |  |
| 402 | Supply, Fabrication of well seasoned T.W frame of size 27" x 19" (with 3"x4" size TW) for 1.0 / 1.5 / 2.0 Ton Window A.C.Unit. |  |  |  |  |  | 1065.00 |  |
| 403 | Copper tubing between indoor and outdoor unit for split Air-Conditioner including foaming etc., complete. |  |  |  |  |  | 195.00 |  |
| 404 | Supply of 1.0 / 1.5 Ton brand new sealed compressor. |  |  |  |  |  | 9370.00 |  |
| 405 | Supply of 2.0 Ton brand new sealed compressor. |  |  |  |  |  | 10630.00 |  |
| 406 | 1/6th H.P multi speed Fan Motor suitable for 1.0 / 1.5 / 2.0 Ton Window A.C.Unit. |  |  |  |  |  | 690.00 |  |
| 407 | Fan motor capacitor 6 mfd by new capacitor for A.C machine of 1.0 1.5 / Ton Capacity. |  |  |  |  |  | 180.00 |  |
| 408 | Thermostat switch suitable for 1.0 / 1.5 / 2.0 Ton A.C.Unit. |  |  |  |  |  | 470.00 |  |
| 409 | Starting capacitor of 80 to 120 mfd. by new approved make capacitor suitable for compressor motor of 1.0 / 1.5 Ton A.C. Unit. |  |  |  |  |  | 207.00 |  |
| 410 | Running capacitor of 36 mfd by new approved make capacitor suitable for compressor motor of 1.0 / 1.5 Ton A.C.Unit. |  |  |  |  |  | 255.00 |  |
| 411 | Relay by approved make having pickup voltage 220 Volts and dropout voltage 120 Volts suitable for 1.0 / 1.50 Ton A.C. Unit. |  |  |  |  |  | 260.00 |  |
| 412 | Filter and capillary tube including welding etc., complete for 1.0 / 1.5 / 2.0 Ton A.C.Unit. |  |  |  |  |  | 555.00 |  |
| 413 | Air filter suitable for 1.50 Ton Window A.C.Unit P.V.C. / NYLON. |  |  |  |  |  | 260.00 |  |
| 414 | Air filter suitable for 1.50 Ton Split A.C.Unit P.V.C. / NYLON. |  |  |  |  |  | 545.00 |  |
| 415 | Rewiring the entire Window type 1.50 Ton A.C.unit with 2.5 Sq.mm Flexible copper cable and putting the same in working conditions. |  |  |  |  |  | 375.00 |  |
| 416 | Overhauling and servicing the Window type 1.0 / 1.5 Ton A.C. Machine by Cleaning the condensor cooling Coil etc., by blower and oiling the Fan Motor and Cleaning all parts and putting the machine in working condition. |  |  |  |  |  | 400.00 |  |
| 417 | Fan blade for 1.0 / 1.5 Ton A.C.Machine. |  |  |  |  |  | 185.00 |  |
| 418 | Grill of 1.0 / 1.5 ton A.C. Machine (for assembled Units). |  |  |  |  |  | 490.00 |  |
| 419 | Grill of 1.0 / 1.5 ton A.C. Machine of branded units. |  |  |  |  |  | 1955.00 |  |
| 420 | Rotary / Selector switch by new one. |  |  |  |  |  | 135.00 |  |
| 421 | Flushing the line, leak testing, leak stopping, vaccumming, dehydrating and charging the gas in the refrigeration system for 1.0/ 1.5 /2.0 Ton Window A.C.Unit. |  |  |  |  |  | 1725.00 |  |
| 422 | Flushing the line, leak testing, leak stopping, vaccumming, dehydrating and charging the gas in the refrigeration system for 1.5 /2.0 Ton Split A.C.Unit. |  |  |  |  |  | 2415.00 |  |
| 423 | Spray painting of Refrigerator using good quality paint after scraping the surface etc. |  |  |  |  |  | 850.00 |  |
| 424 | New sealed compressor of 1/8 HP suitable for 165 to 185 Ltrs. Capacity Refrigerator. |  |  |  |  |  | 3570.00 |  |
| 425 | Sealed compressor of 1/6 HP suitable for 285 / 310 Ltrs. Capacity Refrigerator. |  |  |  |  |  | 4310.00 |  |
| 426 | Supply of fan motor by new 1/10 t0 1/12 H.P 1250 RPM fan motor for Water Cooler of 80-150 Ltrs. |  |  |  |  |  | 1830.00 |  |
| 427 | Bush for fan motor of 80-150 Lts. |  |  |  |  |  | 145.00 |  |
| 428 | Fan hook of 12/15mm dia M.S. rod / bar |  |  |  |  |  | 50.00 |  |
| 429 | 19/20mm steel tube down rod of 1 meter length. |  |  |  |  |  | 75.00 |  |
| 430 | Resistance type regulator for ceiling fans 900/1200/1400mm sweep complete erected on existing board. |  |  |  |  |  | 45.00 |  |
| Note: Technology is fast changing which is reflecting on the specifications and price structure. Hence the AC Units etc. rates are not incorporated in the rate Stablizers Rate per Eachanalysis. However the rate analysis as per the prevailing market rate may be considered in the approved data while workingout finished item of rate. |  |  |  |  |  |  |  |  |
| Sl. No | Description | V-Gaurd | KEELINE | Capri | Real Guard | UNI- STAB | ITL |  |
| 431 | 0.5 KVA Stabilizer | 1,090.00 | 1,240.00 | 1,090.00 | 1,090.00 | 1345 | 1,545.00 |  |
| 432 | 1 KVA Stabilizer | 2,040.00 | 2,040.00 | 2,040.00 | 2,040.00 | 2,040.00 | 2,040.00 |  |
| 433 | 3 KVA Stabilizer | 2,650.00 | 2,650.00 | 2,650.00 | 2,650.00 | 2,650.00 | 2,650.00 |  |
| 434 | 4 KVA Stabilizer | 3,565.00 | 3,565.00 | 3,565.00 | 3,565.00 | 3,565.00 | 3,565.00 |  |
| 435 | 5 KVA Stabilizer | 4,700.00 | 4,700.00 | 4,700.00 | 4,700.00 | 4,700.00 | 4,700.00 |  |
| Fans and Exhaust Fans Rate per Each in adition to following makes and models For the makes USHA/ Orient/Almonard/Khaitan and other reputed firms make etc., Prevailing market rates may be adopted as on 01-06-2008. |  |  |  |  |  |  |  |  |
| 436 | 36" (900mm) Sweep Ceiling Fan |  |  |  |  |  |  |  |
|  | Crompton | Bajaj | Havells |  |  |  |  |  |
|  | High Speed | Ultima | Velocity |  |  |  |  |  |
|  | Decora | G Gold | Fusion Mt. |  |  |  |  |  |
|  | Reviera | R Gold | Candy |  |  |  |  |  |
| 437 | 48" (1200mm) Sweep Ceiling Fan |  |  |  |  |  |  |  |
|  | Crompton | Bajaj | Havells |  |  |  |  |  |
|  | High Speed | Ultima | Velocity |  |  |  |  |  |
|  | Decora | G Gold | Fusion Mt. |  |  |  |  |  |
|  | Reviera | R Gold | Candy |  |  |  |  |  |
| 438 | 56" (1200mm) Sweep Ceiling Fan |  |  |  |  |  |  |  |
|  | Crompton | Bajaj | Havells |  |  |  |  |  |
|  | High Speed | Ultima | Velocity |  |  |  |  |  |
|  | Decora | G Gold | Fusion Mt. |  |  |  |  |  |
|  | Reviera | R Gold | Candy |  |  |  |  |  |
| 439 | 16" (400mm ) (Wall mounted) |  |  |  |  |  |  |  |
|  | Crompton | Bajaj | Havells |  |  |  |  |  |
|  | Gold | Media BW05 | Swing |  |  |  |  |  |
|  |  | Elegance | Premia Gold |  |  |  |  |  |
|  |  | Grace |  |  |  |  |  |  |
| 440 | 16" (400mm) Pedestal Fan |  |  |  |  |  |  |  |
|  | Crompton | Bajaj | Havells |  |  |  |  |  |
|  | SDX | Media BP 5 | Mobile |  |  |  |  |  |
|  | MPF-G | Media IOA 11 | Mobile With Remote |  |  |  |  |  |
|  | Classical |  | Swift |  |  |  |  |  |
|  | Description |  |  |  |  |  |  |  |
| 441 | 18" (450mm) Air circulators |  | Crompton | Bajaj | Havells |  |  |  |
| 442 | 24" (600mm) Air Circulators |  | Crompton | Bajaj | Havells |  |  |  |
| 443 | 12" (300mm ) (Cabin Fan) |  | Crompton | Bajaj | Havells |  |  |  |
| 444 | 12" (300mm) (Fresh air Fans) |  | Crompton | Bajaj | Havells |  |  |  |
| 445 | 9" (225mm) Fresh air Fans |  | Crompton | Bajaj | Havells |  |  |  |
| 446 | 12" (300mm) H.D.Exhaust Fan 900 RPM |  | Crompton | Bajaj | Havells |  |  |  |
| 447 | 15" (375mm) 900 RPM HD Exhaust Fan |  | Crompton | Bajaj | Havells |  |  |  |
| 448 | 18" (450mm) HD Exhaust Fan |  | Crompton | Bajaj | Havells |  |  |  |
|  | X. Water Heaters |  |  |  |  |  |  |  |
|  | Geysers & Instant Water Heaters Rate per Each from the reputed firms in adition to the following makes etc., Prevailing market rates may be adopted as on 01-06-2008 |  |  |  |  |  |  |  |
|  | Description |  |  |  |  |  |  |  |
| 449 | 25 Lt Geyser (copper tank) Vertical |  | Venus | Bajaj | V- Guard | Racold | Usha |  |
| 450 | 35 Lt Geyser (copper tank) Vertical |  | Venus | Bajaj | V- Guard | Racold | Usha |  |
| 451 | 50 Lt Geyser (copper tank) Vertical |  | Venus | Bajaj | V- Guard | Racold | Usha |  |
| 452 | Instant water heater 3 KW |  | Venus | Bajaj | V- Guard | Racold | Usha |  |
| 453 | 25 Lt Geyser (copper tank) Vertical Plastic body |  | Venus | Bajaj | V- Guard | Racold | Usha |  |
|  | Repairs to Geysers Rate per Each |  |  |  |  |  |  |  |
| 454 | Heating element of 1.5 / 2.0 / 3.0 KW. |  |  |  |  |  | 350.00 |  |
| 455 | Supply of thermostat with range 300 to 900C.Adjustable and suitable for 25 to 50 Ltrs. Capacity. |  |  |  |  |  | 300.00 |  |
| 456 | Supply of pressure release valve for water heater of 15 to 50 Ltrs.capacity. |  |  |  |  |  | 350.00 |  |
| 456 a | Flexible Nylon / PVC Metallic hume pipe |  |  |  |  |  | 135.00 |  |
|  | XI. Water Pumps and Accessories |  |  |  |  |  |  |  |
|  | Note:Techonology is fast changing which is reflecting on the specifications and price structure. Hence not incorporated in the rate analysis. However, the rate analysis as per the prevailing market rate may be considered in the approved data while working |  |  |  |  |  |  |  |
|  | Accessories Rate per Each |  |  |  |  |  |  |  |
| 457 | D.O.L Starter 250V, Single phase, 50Hz. With SS enclouser comprising of contactor with bi-metalic relay with necessary push buttons, etc., complete . |  |  |  |  |  | 960.00 |  |
| 458 | D.O.L Starter 415V, 3 phase, 50Hz.. enclosed with no volt coil and overload protection with necessary materials, etc., complete suitable upto 7.5.HP. |  |  |  |  |  | 1750.00 |  |
| 459 | Semi automatic star / delta starter suitable for 10 HP to 12.5 HP Induction Motor totally enclosed with over load protection and no volt relay etc., complete. |  |  |  |  |  | 4400.00 |  |
| 460 | Semi automatic star / delta starter suitable for 15 HP to 25 HP Induction Motor totally enclosed with over load protection and no volt relay etc., complete. |  |  |  |  |  | 5290.00 |  |
| 461 | Automatic star / delta starter suitable for 10 HP to 12.5 HP Induction Motor totally enclosed with over load protection and no volt relay etc., complete. |  |  |  |  |  | 6620.00 |  |
| 462 | Erecting automatic star / delta starter suitable up to 15 HP Induction Motor totally enclosed with over load protection and no volt relay etc., complete. |  |  |  |  |  | 7015.00 |  |
| 463 | Single phasing preventer connected to A.C. supply upto 45A range. |  |  |  |  |  | 2520.00 |  |
| 464 | Motor made from GI sheet of 18SWG for protection from rain complete. |  |  |  |  |  | 200.00 |  |
|  | XII. Bus Bars |  |  |  |  |  |  |  |
|  | Note: In view of fast changing of market rates for steel items under copper / Alluminum materials the rate is to be arived based on the observed data as per the civil SSR / Local market. |  |  |  |  |  |  |  |
|  | XIII. Swaged Poles, Garden Posts & Accessories |  |  |  |  |  |  |  |
|  | Note: The rate for the steel items such as MS poles, MS iron, MS Channels, GI Pipes and its accessories shall be as per the rates approved by the Govt./as per rate of Steel India authrity/Price list rates from reputed firms etc from time to time. |  |  |  |  |  |  |  |
| 465 | Fabrication, supply and transportation of all sizes swaged poles up to site confirming to IS 2713 / 1980 specifications. The pole should be painted with primary coat of red oxide and black bituminous paint up to ground level with base plate, including co |  |  |  |  |  |  |  |
| 466 | Fabrication and Supply of M.S. Poles for garden posts with 'B' class 60.25mm outer dia with base plate and suitable pole cap including cost and conveyance of all material including fixing of luminaire and labour charges etc., complete. (Rate per K. G) |  |  |  |  |  |  |  |
| 467 | Supply of different sizes of M. S. Pipe brackets (Rate per K.G) |  |  |  |  |  |  |  |
|  | XIV. Control Panel & Metering Equipment |  |  |  |  |  |  |  |
|  | Fabrication and erection of following guage CRCA sheet steel enclosure for power control cubicle panel / boxes etc., including cutting, bending drilling, welding and reveting etc., complete with cleaning with 7 tank process and painting with powder coat. (Rate per Sq.inch.) |  |  |  |  |  |  |  |
| 468 | 14 SWG CRCA sheet steel enclosure |  |  |  |  |  | 2.50 |  |
| 469 | 16 SWG CRCA sheet steel enclosure |  |  |  |  |  | 2.25 |  |
| 470 | 18 SWG CRCA sheet steel enclosure |  |  |  |  |  | 1.90 |  |
|  | Note: In view of fast changing of market rates for steel items under M.S. accessories the rate is to be arived based on the observed data as per the civil SSR / Local market. |  |  |  |  |  |  |  |
|  | Metering Equipment Rate per Each |  |  |  |  |  |  |  |
|  | Description |  |  |  |  | Analog | Digital |  |
| 471 | Supply and erection of 0-415V volt meter of size 96x96mm on exisiting box / panel board including connections etc., complete.(Conzen/AE/MECO/ HPL make) |  |  |  |  | 540.00 | 3000.00 |  |
| 472 | Supply and erection of 0-100 / 200 / 400 / 600 /1000 - 5A CT Ammeter of size 96x96mm on existing box / panel board including connections etc., complete. (Conzen/AE/MECO/ HPL make) |  |  |  |  | 540.00 | 3000.00 |  |
| 473 | Supply and erection of frequency meter of size 96x96mm on existing box / panel board including connections etc., complete. (Conzen/AE/MECO/ HPL make) |  |  |  |  | --- | 5,470.00 |  |
| 474 | Supply and erection of power factor meter, 3 phase, 4 wire balanced or unabalanced load 5A, 415V of size 96x96mm on existing box / panel board including connections etc., complete. (Conzen/AE/MECO/ HPL make) |  |  |  |  | --- | 3,240.00 |  |
| 475 | Supply and erection of L.T. 3phase multifunction energy meter with 3 lines LCD display of size 96x96mm for measuring all the parameters (voltage / Current / Power / frequency / power factor / kw / kwh / KVAR/ KVARH/) on existing box panel board including |  |  |  |  | --- | 12,030.00 |  |
| 476 | Supply and erection of selector switch for voltmeter / Ammeter on the existing box / panel including connections etc., complete. (HPL/Conzerv/AE makes) |  |  |  |  | 210.00 | --- |  |
| 477 | Supply and erection of LED indicator lamps on the existing box / panel board.(HPL/Conzerv/AE makes) |  |  |  |  | 165.00 | --- |  |
| 478 | Supply and erection of LT current transformer with bar primary 100/5A to 1000 / 5A ratio in the existing box / panel including connections etc., complete. (HPL/Conzerv/AE makes) |  |  |  |  | 550.00 | --- |  |
| 479 | Supply and fixing of single phase energy Meter,240V AC,50 HZ as per IS13010 with 2.5-5 /5-10/10-20 Amps range with 200% over load capacity complete with connections etc., complete on the existing board. (HPL/Conzerv/AE makes) |  |  |  |  | 1,215.00 | --- |  |
| 480 | Supply and fixing of three phase energy Meter, 415V AC,50 HZ as per IS13010 with 2.5-5 /5-10/10-20 Amps range with 200% over load capacity complete with connections etc., complete on the existing board. (HPL/Conzerv/AE makes) |  |  |  |  | 3,640.00 | --- |  |
|  | XV. U. G. Cables, Glands, Lugs Etc., |  |  |  |  |  |  |  |
|  | XLPE Armoured 1100 V U.G. Cables Rate per Metre |  |  |  |  |  |  |  |
|  | Sl.No. | 481 | 482 | 483 | 484 | 485 | 486 | 487 |
|  | Description | XLPE PVC 4 Sq. mm 2 core Armoured Allu. Cable | XLPE PVC 6 Sq. mm 2 core Armoured Allu. Cable | XLPE PVC 10 Sq. mm 2 core Armoured. Allu. Cable | XLPE PVC 4 Sq. mm 4 core Armoured Allu. Cable | XLPE PVC 6 Sq. mm 4 core Armoured Allu. Cable | XLPE PVC 10 Sq. mm 4 core Armoured Allu. Cable | XLPE PVC 16 Sq. mm 4 core Armoured Allu. Cable |
|  | Torrent | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Polycab | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Universal | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Power Plast | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Paragon | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Payal | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Finolex | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Surana | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Unicab | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Nicco | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Asian | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | FINE CAB | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Suncab | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Alankar | 121.55 | 137.72 | 168.52 | 185.37 | 203.91 | 224.30 | 246.73 |
|  | Sl.No. | 488 | 489 | 490 | 491 | 492 | 493 | 494 |
|  | Description | XLPE PVC 25 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 35 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 50 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 70 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 95 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 120 Sq. mm 3.5 core Armoured Allu. Cable | XLPE PVC 150 Sq. mm 3.5 core Armoured Allu. Cable |
|  | Torrent | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Polycab | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Universal | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Power Plast | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Paragon | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Payal | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Finolex | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Surana | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Unicab | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Nicco | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Asian | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | FINE CAB | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Suncab | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Alankar | 325.71 | 383.46 | 506.99 | 557.69 | 613.46 | 674.80 | 742.28 |
|  | Sl.No. | 495 | 496 | 497 | 498 |  |  |  |
|  | Description | XLPE PVC 185 Sq. mm 3.5 core Ar. Allu. Cable | XLPE PVC 240 Sq. mm 3.5 core Ar. Allu. Cable | XLPE PVC 300 Sq. mm 3.5 core Ar. Allu. Cable | XLPE PVC 400 Sq. mm 3.5 core Ar. Allu. Cable |  |  |  |
|  | Torrent | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Polycab | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Universal | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Power Plast | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Paragon | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Payal | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Finolex | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Surana | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Unicab | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Nicco | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | FINE CAB | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Asian | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Suncab | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Alankar | 1,585.54 | 1,744.09 | 1,918.50 | 2,110.35 |  |  |  |
|  | Cable Glands, Lugs & Cable Joints Etc., Rate per Each |  |  |  |  |  |  |  |
|  | Description |  |  |  |  |  |  |  |
| 499 | Supply and Fixing of heavy duty flange type brass cable glands for 2 to 4 core, 2.5 Sq.mm to 6 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 45.00 |  |
| 500 | Supply and Fixing of heavy duty flange type brass cable glands for 2 to 4 core, 10 Sq.mm to 6 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 79.00 |  |
| 501 | Supply and Fixing of heavy duty flange type brass cable glands for 4 core, 16 Sq.mm / 3.5 core 25Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 95.00 |  |
| 502 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 35 Sq.mm / 3.0 core 50Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 100.00 |  |
| 503 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 50 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 110.00 |  |
| 504 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 70 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 138.00 |  |
| 505 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 95 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 170.00 |  |
| 506 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 120 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 170.00 |  |
| 507 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 150 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 191.00 |  |
| 508 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 185 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 220.00 |  |
| 509 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 240 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 290.00 |  |
| 510 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 300 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 297.00 |  |
| 511 | Supply and Fixing of heavy duty flange type brass cable glands for 3.5 core, 400 Sq.mm PVC armoured cable with brass washers complete. |  |  |  |  |  | 434.00 |  |
| 512 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable upto 16 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 9.00 |  |
| 513 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 25 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 12.00 |  |
|  | Description |  |  |  |  |  |  |  |
| 514 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 35 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 23.00 |  |
| 515 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 50 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 30.00 |  |
| 516 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 70 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 37.00 |  |
| 517 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 95 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 44.00 |  |
| 518 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 120 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 71.00 |  |
| 519 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 150 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 115.00 |  |
| 520 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 185 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 127.00 |  |
| 521 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 240 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 184.00 |  |
| 522 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 300 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 256.00 |  |
| 523 | Supply and fixing of crimping type copper lugs confirming to I.S specifications for cable of 400 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 370.00 |  |
| 524 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable upto 16 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 6.00 |  |
| 525 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 25 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 7.00 |  |
| 526 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 35 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 8.00 |  |
| 527 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 50 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 9.00 |  |
| 528 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 70` Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 11.00 |  |
| 529 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 95 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 14.00 |  |
| 530 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 120 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 18.00 |  |
| 531 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 150 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 26.00 |  |
| 532 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 185 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 29.00 |  |
| 533 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 240 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 44.00 |  |
| 534 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 300 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 81.00 |  |
| 535 | Supply and fixing of crimping type aluminium lugs confirming to I.S specifications for cable of 400 Sq.mm including nuts and bolts with connections. |  |  |  |  |  | 85.00 |  |
| 536 | Providing and erecting epoxy staight through joint out door / indoor for L.T, PVC Armoured cable 1100V grade complete with ferrule, bushings, accessories and suitable clamps duly erected and connected to the supply for the following multi core cables including cost and conveyance of materials and all labour charges etc., complete. |  |  |  |  |  |  |  |
| 537 | --do--2.5 Sq.mm to 16 Sq.mm --do-- |  |  |  |  |  | 620.00 |  |
| 538 | --do-- from 25 Sq.mm to 50 Sq.mm cable --do-- |  |  |  |  |  | 880.00 |  |
| 539 | --do-- from 70 Sq.mm to 95 Sq.mm cable --do-- |  |  |  |  |  | 1020.00 |  |
| 540 | --do-- from 120 Sq.mm to 185 Sq.mm cable --do-- |  |  |  |  |  | 1700.00 |  |
| 541 | --do-- 240Sq.mm cable --do-- |  |  |  |  |  | 2400.00 |  |
| 542 | --do-- 300Sq.mm cable --do-- |  |  |  |  |  | 2850.00 |  |
| 543 | --do-- 400Sq.mm cable --do-- |  |  |  |  |  | 3250.00 |  |
|  | XVI. Temporary Illumination |  |  |  |  |  |  |  |
| 544 | Note: | The rates for different materials are incorporated under different categories and the same are to be incorporated from the observed data while working out specification of item of work with rate for arriving data for Temporary Illumination works. |  |  |  |  |  |  |
|  | XVII. Generators Transformers & Substation Equipment & Protective Gears. |  |  |  |  |  |  |  |
| 545 | Note: | Techonology is fast changing which is reflecting on the specifications and price structure of Generators and Transformers, hence not incorporated in the rate analysis. However, the rate analysis as per the prevailing market rate may be considered in the approved data while working out finished item of rate. |  |  |  |  |  |  |
|  | Sub Station Equipment Rate per Each |  |  |  |  |  |  |  |
|  | Description |  |  |  |  |  |  |  |
| 546 | Carryout routine maintenance, servicing and testing of 630A to 1000A L.T.ACB by cleaning fixed contacts, moving contacts and arcchutes including servicing and putting the ACB in perfect working condition after detting all relay and ON, OFF mechanism etc. |  |  |  |  |  | 2160.00 |  |
| 547 | Carryout routine maintenance and servicing of LT.Cubical pannel board having approximately 15 to 22 Nos of switches cleaning with power brower and servicing the contacts, tightning the connection of incoming and outgoing cables, and clicking the operating machanism for smooth functioning of switches and putting them in smooth working condition. |  |  |  |  |  | 3000.00 |  |
| 548 | Routine maintenance of transformer in operation covering insulation resistance oil, break down strength. Reconditioning / replacement of silicazel, cleaning of transformer and topping upto oil level if required (extra cost will be paid for oil). |  |  |  |  |  | 5220.00 |  |
| 549 | Testing of Transformer oil for dielectric breakdown strength. |  |  |  |  |  | 600.00 |  |
|  | Removing the existing transformer from the plinth / Pole and refixing the standby transformer of following capacity including connections etc., complete. |  |  |  |  |  |  |  |
| 550 | Upto 315 KVA |  |  |  |  |  | 3000.00 |  |
| 551 | 500 / 1000 KVA |  |  |  |  |  | 4200.00 |  |
|  | Replacing old and unserviceable gasket of top cover plate of transformer having following capacity. |  |  |  |  |  |  |  |
| 552 | Upto 315 KVA |  |  |  |  |  | 900.00 |  |
| 553 | 500 / 1000 KVA |  |  |  |  |  | 1260.00 |  |
|  | Replacing old and unserviceable gasket of H. T bushing for the following Transformer capacity. |  |  |  |  |  |  |  |
| 554 | Upto 315 KVA |  |  |  |  |  | 360.00 |  |
| 555 | 500 / 1000 KVA |  |  |  |  |  | 660.00 |  |
|  | Replacing old and unserviceable gasket of L.T bushing for the following transformer capacity. |  |  |  |  |  |  |  |
| 556 | Upto 315 KVA |  |  |  |  |  | 840.00 |  |
| 557 | 500 / 1000 KVA |  |  |  |  |  | 1740.00 |  |
|  | Replacement the H/T / L. T burntout winding coils of distribution transformer including rewinding, varnishing and vaccuming etc.,duly retaining the removed burnt winding coils by the Contractor / Firm |  |  |  |  |  |  |  |
| 558 | For copper coils |  |  |  |  |  | 710.00 |  |
| 559 | For aluminium coils |  |  |  |  |  | 480.00 |  |
| 560 | Insulating HT / LT copper / aluminium coils for Transformers. |  |  |  |  |  | 400.00 |  |
|  | Painting with 2 coats Zinchromate red oxide duly scraping existing paint followed by 2 coats of hammerton grey paint etc., complete. |  |  |  |  |  |  |  |
| 561 | Upto 500 KVA |  |  |  |  |  | 3600.00 |  |
| 562 | 630 / 1000 KVA |  |  |  |  |  | 4800.00 |  |
| 563 | Replacing H.T bushing for distribution Transformer by new one. |  |  |  |  |  | 450.00 |  |
| 564 | Replacing H.T bushing metal parts for distribution Transformer by new one. |  |  |  |  |  | 480.00 |  |
|  | Replacing L.T bushing for distribution Transformer by new one for the following capacities. |  |  |  |  |  |  |  |
| 565 | Upto 315 KVA |  |  |  |  |  | 210.00 |  |
| 566 | 500 / 1000 KVA |  |  |  |  |  | 240.00 |  |
|  | Replacing L.T bushing metal parts for distribution Transformer by new one for the following capacities. |  |  |  |  |  |  |  |
| 567 | 100 KVA |  |  |  |  |  | 370.00 |  |
| 568 | 250 / 315 KVA |  |  |  |  |  | 650.00 |  |
| 569 | 500 KVA |  |  |  |  |  | 1650.00 |  |
| 570 | 1000 KVA |  |  |  |  |  | 2500.00 |  |
|  | Drying the repaired transformer with appropriate heat / air of following capacity of transformers. |  |  |  |  |  |  |  |
| 571 | Upto 315 KVA |  |  |  |  |  | 3700.00 |  |
| 572 | 500 / 1000KVA |  |  |  |  |  | 5200.00 |  |
|  | Loading and Unloaidng of transformer from site to Workshop or vice verse. |  |  |  |  |  |  |  |
| 573 | Upto 500 KVA |  |  |  |  |  | 5500.00 |  |
| 574 | 630 / 1000 KVA |  |  |  |  |  | 6460.00 |  |
|  | PROTECTIVE GEAR (Rate per Each) |  |  |  |  |  |  |  |
| 575 | Supply and erecting approved make 11KV, 5KA metal oxide lightning arrestors suitable for 11 KV supply with necessary materials on existing cross arm. |  |  |  |  |  | 1800.00 |  |
| 576 | Supply and erecting porcelain disc type insulator suitable for 11KV line with suitable hardware on existing cross arms. |  |  |  |  |  | 720.00 |  |
| 577 | Supply and erecting porcelain pin type insulator suitable for 11KV line with suitable hardware on existing cross arms. |  |  |  |  |  | 360.00 |  |
| 578 | Supply and erecting porcelain post type insulator suitable for 11KV line with suitable hardware on existing cross arms. |  |  |  |  |  | 450.00 |  |
| 579 | Supply and erecting porcelain pin/shackle type insulator suitable for L. T. |  |  |  |  |  | 110.00 |  |
| 580 | Supply and fixing of lightning protective system conforming to IS : 2309 with all standard accessories such as metal globe finials on the top of the builiding with 25.4 mm dia 3.0mtrs. Length of 'B' class G.I pipe etc., complete without down conductors and earthing. |  |  |  |  |  | 720.00 |  |
| 581 | Approved made 11KV, 400A A.B switch with all standard accessories etc., complete with cost and conveyance of all materials. |  |  |  |  |  | 8900.00 |  |
| 582 | Approved made 11KV, 200A A.B switch with all standard accessories etc., complete with cost and conveyance of all materials. |  |  |  |  |  | 7500.00 |  |
| 583 | Approved made 11KV, 200A A.B switch (tilting) with all standard accessories etc., complete with cost and conveyance of all materials. |  |  |  |  |  | 7500.00 |  |
| 584 | 11KV, H.G fuse set including all standard accessories etc., complete with cost and conveyance of all materials. |  |  |  |  |  | 5200.00 |  |
| Capacitors (Rate per KVAR) |  |  |  |  |  |  |  |  |
| Sl.No. | 585 | 586 | 587 | 588 | 589 | 590 | 591 |  |
| Description | Metalised poly propylene condensor | Mixdieelectric condensor | Electronic NKP Type oil filled allu. Cylinder H. D L. V Capacitor | DTCO - 2 - 100 Detuned capacitor bank (Operating system 3.3 KV, 3Ph. 50 Hz. | DTCO - 2 - 100 Detuned capacitor bank (Operating system 6.6 KV, 3Ph. 50 Hz. | DTCO - 2 - 100 Detuned capacitor bank (Operating system 11 KV, 3Ph. 50 Hz. | APFC/MPFC/AMPFC panel type capacitor Bank V4N - 100 - 150 KVAR |  |
| Visa Power | 566.40 | 679.68 | 815.62 | 978.74 | 1,174.49 | 1,409.38 | 1,691.26 |  |
| Alto | 396.00 | 475.20 | --- | --- | --- | --- | --- |  |
| Ikon | 396.00 | 475.20 | --- | --- | --- | --- | --- |  |
| Asian | 396.00 | 475.20 | --- | --- | --- | --- | --- |  |
| L & T | 566.40 | 679.68 | --- | --- | --- | --- | --- |  |
| AEE | 566.40 | 679.68 | --- | --- | --- | --- | --- |  |
| Epcos | 396.00 | 475.20 | --- | --- | --- | --- | --- |  |
|  | Air Circuit Breaker (Rate per each) Note: For other Reputed makes the price list rate as on 1.06.2008 may be adopted. |  |  |  |  |  |  |  |
|  |  | L & T | C & S |  |  |  |  |  |
| 592 | 630 Amps Fixed type 3 pole ACB | 63,600.00 | 58,700.00 |  |  |  |  |  |
| 593 | 800 Amps Fixed type 3 pole ACB | 75,100.00 | 62,700.00 |  |  |  |  |  |
| 594 | 1000 Amps Fixed type 3 pole ACB | 76,900.00 | 64,100.00 |  |  |  |  |  |
|  | XVIII. Repairs to Motors and Rewinding |  |  |  |  |  |  |  |
| 595 | Bearing of approved make for electric motor single phase / 3 phase AC upto 1 HP of all types. |  |  |  |  |  | 215.00 |  |
| 596 | Bearing of approved make for electric motor single phase / 3 phase AC upto 2 to 3 HP of all types. |  |  |  |  |  | 277.00 |  |
| 597 | Bearing of approved make for electric motor single phase / 3 phase AC upto 5 to 7.5 HP of all types. |  |  |  |  |  | 367.00 |  |
| 598 | Bearing of approved make for electric motor single phase / 3 phase AC upto 10 to 12.5 HP of all types. |  |  |  |  |  | 383.00 |  |
| 599 | Bearing of approved make for electic motor single phase / 3 phase AC upto 15 to 20 HP of all types. |  |  |  |  |  | 587.00 |  |
| 600 | Bearing of table / cabin / pedestal / ceiling / exhaust fan complete with alignment of shaft and tested. |  |  |  |  |  | 101.00 |  |
| 601 | Bearing of bush / rubber / ring washers complete with alignment of shaft and tested. |  |  |  |  |  | 48.00 |  |
| 602 | Capacitor of table / cabin / pedestal / ceiling / exhaust fan complete. |  |  |  |  |  | 46.00 |  |
|  | XIX. Miscellaneous |  |  |  |  |  |  |  |
|  | Description |  |  |  |  |  |  |  |
| 603 | M. S. Junction box big size long neck 19 / 20 mm |  |  |  |  | Each | 19.50 |  |
| 604 | M. S. Junction box big size long neck 25 mm |  |  |  |  | Each | 19.90 |  |
| 605 | M. S. Junction box big size long neck 32 mm |  |  |  |  | Each | 52.45 |  |
| 606 | M. S. Junction box big size long neck 40 / 38 mm |  |  |  |  | Each | 62.40 |  |
| 607 | M. S. Junction box big size long neck 50 mm |  |  |  |  | Each | 112.30 |  |
| 608 | M. S. Junction box deep 63 mm 25 / 20 mm |  |  |  |  | Each | 32.40 |  |
| 609 | M. S. Junction box deep 63 mm 32 mm |  |  |  |  | Each | 74.90 |  |
|  | Description |  |  |  |  |  |  |  |
| 610 | M. S. Junction box deep 63 mm 38/40 mm |  |  |  |  | Each | 77.40 |  |
| 611 | M. S. Inspection Bends 19/20 mm |  |  |  |  | Each | 17.90 |  |
| 612 | M. S. Inspection Bends 25 mm |  |  |  |  | Each | 18.40 |  |
| 613 | M. S. Inspection Bends 32 mm |  |  |  |  | Each | 46.20 |  |
| 614 | M. S. Inspection Bends 38/40 mm |  |  |  |  | Each | 56.20 |  |
| 615 | M. S. Inspection Bends 50 mm |  |  |  |  | Each | 96.10 |  |
| 616 | M. S Inspection Tee 19/20 mm |  |  |  |  | Each | 19.30 |  |
| 617 | M. S Inspection Tee 25 mm |  |  |  |  | Each | 19.90 |  |
| 618 | M. S Inspection Tee 32 mm |  |  |  |  | Each | 52.40 |  |
| 619 | M. S Inspection Tee 38/40 mm |  |  |  |  | Each | 62.40 |  |
| 620 | M. S Inspection Tee 50 mm |  |  |  |  | Each | 112.30 |  |
| 621 | M. S Inspection Elbows 19/20 mm |  |  |  |  | Each | 16.80 |  |
| 622 | M. S Inspection Elbows 25 mm |  |  |  |  | Each | 17.10 |  |
| 623 | M. S Inspection Elbows 52 mm |  |  |  |  | Each | 43.70 |  |
| 624 | M. S Inspection Elbows 38/40 mm |  |  |  |  | Each | 53.60 |  |
| 625 | M. S Inspection Elbows 50 mm |  |  |  |  | Each | 89.90 |  |
| 626 | PVC Flexible pipe 19/20 mm |  |  |  |  | 50 Mtrs | 150.00 |  |
| 627 | PVC Flexible pipe 25 mm |  |  |  |  | 50 Mtrs | 199.00 |  |
| 628 | PVC Flexible pipe 32 mm |  |  |  |  | 50 Mtrs | 249.00 |  |
| 629 | PVC Flexible pipe 38 mm |  |  |  |  | 50 Mtrs | 500.00 |  |
| 630 | PVC Flexible pipe 50 mm |  |  |  |  | 50 Mtrs | 811.00 |  |
| 631 | Flexible ccoupling special 19mm |  |  |  |  | Each | 9.00 |  |
| 632 | Flexible ccoupling special 25mm |  |  |  |  | Each | 11.00 |  |
| 633 | Flexible ccoupling special 32mm |  |  |  |  | Each | 23.00 |  |
| 634 | Flexible ccoupling special 38mm |  |  |  |  | Each | 25.00 |  |
| 635 | Flexible ccoupling special 50mm |  |  |  |  | Each | 50.00 |  |
| 636 | 10 Guage G. I checknut 19/20 mm |  |  |  |  | 100 Nos | 173.00 |  |
| 637 | 10 Guage G. I checknut 25 mm |  |  |  |  | 100 Nos | 215.00 |  |
| 638 | 10 Guage G. I checknut 32 mm |  |  |  |  | 100 Nos | 400.00 |  |
| 639 | 10 Guage G. I checknut 38/40 mm |  |  |  |  | 100 Nos | 540.00 |  |
|  | Description |  |  |  |  | Unit |  |  |
| 640 | 10 Guage G. I checknut 50 mm |  |  |  |  | 100 Nos | 1,070.00 |  |
| 641 | Saddle with base 19/20 mm |  |  |  |  | 100 Nos | 177.00 |  |
| 642 | Saddle with base 25 mm |  |  |  |  | 100 Nos | 185.00 |  |
| 643 | Saddle with base 32 mm |  |  |  |  | 100 Nos | 305.00 |  |
| 644 | Saddle with base 38/40 mm |  |  |  |  | 100 Nos | 325.00 |  |
| 645 | Saddle with base 50 mm |  |  |  |  | 100 Nos | 375.00 |  |
| 646 | 20 guage Saddles 19/20 mm |  |  |  |  | 100 Nos | 95.00 |  |
| 647 | 20 guage Saddles 25 mm |  |  |  |  | 100 Nos | 125.00 |  |
| 648 | 20 guage Saddles 32 mm |  |  |  |  | 100 Nos | 205.00 |  |
| 649 | 20 guage Saddles 38/40 mm |  |  |  |  | 100 Nos | 259.00 |  |
| 650 | 20 guage Saddles 50 mm |  |  |  |  | 100 Nos | 315.00 |  |
| 651 | 16 guage Socket coupler 19/20 mm |  |  |  |  | 1 Dozen | 48.00 |  |
| 652 | 16 guage Socket coupler 25 mm |  |  |  |  | 1 Dozen | 60.00 |  |
| 653 | 16 guage Socket coupler 32 mm |  |  |  |  | 1 Dozen | 90.00 |  |
| 654 | 16 guage Socket coupler 38 mm |  |  |  |  | 1 Dozen | 108.00 |  |
| 655 | 16 guage Socket coupler 40 mm |  |  |  |  | 1 Dozen | 144.00 |  |
| 656 | 16 guage Socket coupler 50 mm |  |  |  |  | 1 Dozen | 178.00 |  |
| 657 | Ball socket 19 mm (A) |  |  |  |  | 1 Dozen | 178.00 |  |
| 658 | Ball socket 25 mm (B) |  |  |  |  | 1 Dozen | 174.00 |  |
| 659 | PVC Junction Boxes 19/20 mm |  |  |  |  | 100 Nos | 1,300.00 |  |
| 660 | PVC Junction Boxes 25 mm |  |  |  |  | 100 Nos | 1,450.00 |  |
| 661 | PVC Junction Boxes 32 mm |  |  |  |  | 100 Nos | 2,550.00 |  |
| 662 | PVC Junction Boxes 40 mm |  |  |  |  | 100 Nos | 2,900.00 |  |
| 663 | PVC deep Junction Boxes 19/20 mm |  |  |  |  | 100 Nos | 1,700.00 |  |
| 664 | PVC deep Junction Boxes 25 mm |  |  |  |  | 100 Nos | 2,000.00 |  |
| 665 | PVC deep Junction Boxes 32 mm |  |  |  |  | 100 Nos | 3,100.00 |  |
| 666 | PVC deep Junction Boxes 40 mm |  |  |  |  | 100 Nos | 3,400.00 |  |
| 667 | PVC Bends 19/20 mm |  |  |  |  | 100 Nos | 350.00 |  |
| 668 | PVC Bends 25 mm |  |  |  |  | 100 Nos | 450.00 |  |
| 669 | PVC Bends 32 mm |  |  |  |  | 100 Nos | 1,200.00 |  |
|  | Description |  |  |  |  | Unit |  |  |
| 670 | PVC Bends 40 mm |  |  |  |  | 100 Nos | 1,450.00 |  |
| 671 | PVC Bends 50 mm With Cover |  |  |  |  | 100 Nos | 2,200.00 |  |
| 672 | 50 x 25 x 2 mm G. I perforated tray Without Cover |  |  |  |  | 1 RM | 160.00 |  |
| 673 | 50 x 25 x 2 mm G. I perforated tray With Cover |  |  |  |  | 1 RM | 215.00 |  |
| 674 | 100 x 25 x 2 mm G. I perforated tray without cover |  |  |  |  | 1 RM | 235.00 |  |
| 675 | 100 x 25 x 2 mm G. I perforated tray with cover |  |  |  |  | 1 RM | 340.00 |  |
| 676 | 100 x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 450.00 |  |
| 677 | 150 x 25 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 450.00 |  |
| 678 | 150 x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 530.00 |  |
| 679 | 300 x 25 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 560.00 |  |
| 680 | 250 x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 760.00 |  |
| 681 | 300 x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 900.00 |  |
| 682 | 300 x 25 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 800.00 |  |
| 683 | 450 x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 1,200.00 |  |
| 684 | 500x 50 x 2 mm G. I perforated tray |  |  |  |  | 1 RM | 1,500.00 |  |
| 685 | 600 x 50 x 2mm G. I perforated tray |  |  |  |  | 1 RM | 1,550.00 |  |
| 686 | OD 75 mm single socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 210.00 |  |
| 687 | OD 75 mm double socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 230.00 |  |
| 688 | OD 90 mm single socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 340.00 |  |
| 689 | OD 90 mm double socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 380.00 |  |
| 690 | OD 110 mm single socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 410.00 |  |
| 691 | OD 110 mm double socket UV Resistant (SWR) pipe |  |  |  |  | 3 Mtr | 450.00 |  |
| 692 | Plastic enclosure I.P. 20 1/2 way |  |  |  |  | Each | 90.00 |  |
| 693 | Plastic enclosure I.P. 20 3/4 way |  |  |  |  | Each | 160.00 |  |
| 694 | Plastic enclosure I.P. 20 6 way |  |  |  |  | Each | 250.00 |  |
| 695 | 3 mm Hylam sheet |  |  |  |  | Sft | 65.00 |  |
| 696 | 6mm x 1 m x 2 m Rubber mat 11 KV ISI |  |  |  |  | SqMttr | 1,350.00 |  |
| 697 | Fire buckets |  |  |  |  | Each | 250.00 |  |
| 698 | STC laminated chart |  |  |  |  | Each | 280.00 |  |
|  | Description |  |  |  |  | Unit |  |  |
| 699 | First Aid box 18" x 12" x 8" |  |  |  |  | Each | 1,250.00 |  |
| 700 | 11 KV/22 KV Hand gloves |  |  |  |  | Each | 200.00 |  |
| 701 | Copper plate/ Copper strip (Per K. G) |  |  |  |  | 1 Kg | 590.00 |  |
| 702 | No. 8/10 SWG copper wire (per K. G) |  |  |  |  | 1 Kg | 600.00 |  |
| 703 | PVC Insulated copper winding wire of fine quality (per K. G) |  |  |  |  | 1 Kg | 660.00 |  |
| 704 | No. 14 SWG copper wire (per K. G) |  |  |  |  | 1 Kg | 600.00 |  |
| 705 | Copper winding wire (per K. G) |  |  |  |  | 1 Kg | 710.00 |  |
| 706 | G. I Strip/ Plate/Wire/ M. S (per K. G) |  |  |  |  | 1 Kg | 62.00 |  |
| 707 | Alluminum Bus bars/Strips/accessories |  |  |  |  | 1 Kg | 230.00 |  |
| 708 | Alluminum winding wire |  |  |  |  | 1 Kg | 468.00 |  |
| 709 | 4" x 4" T. W Board |  |  |  |  | Each | 8.00 |  |
| 710 | 6" x 6" T. W Board |  |  |  |  | Each | 9.00 |  |
| 711 | 6" x 8" T. W Board |  |  |  |  | Each | 16.00 |  |
| 712 | 10" x 8" T. W Board |  |  |  |  | Each | 28.00 |  |
| 713 | 12" x 10" T. W Board |  |  |  |  | Each | 40.00 |  |
| 714 | 12" x 15" T. W Board |  |  |  |  | Each | 60.00 |  |
| 715 | 4" x 4" SDC Board |  |  |  |  | Each | 13.00 |  |
| 716 | 6" x 8" SDC Board |  |  |  |  | Each | 29.00 |  |
| 717 | 10" x 8" SDC Board |  |  |  |  | Each | 40.00 |  |
| 718 | 12" x 10" SDC Board |  |  |  |  | Each | 60.00 |  |
| 719 | 12" x 15" SDC Board |  |  |  |  | Each | 110.00 |  |
| 720 | 6" x 4" Deep box |  |  |  |  | Each | 9.00 |  |
| 721 | 10" x 8" Deep box |  |  |  |  | Each | 30.00 |  |
| 722 | 12" x 10" Deep box |  |  |  |  | Each | 40.00 |  |
| 723 | 12" x 15" Deep box |  |  |  |  | Each | 60.00 |  |
| 724 | 9" x 14" Peta Board |  |  |  |  | Each | 35.00 |  |
| 725 | 12" x 10" Peta Board |  |  |  |  | Each | 40.00 |  |
| 726 | 15" x 20" Peta Board |  |  |  |  | Each | 130.00 |  |
| 727 | 18" x 24" Peta Board |  |  |  |  | Each | 180.00 |  |
| 728 | 18" x 30" Peta Board |  |  |  |  | Each | 220.00 |  |
| 729 | 6.2 Volt series bulb (25 Nos.) |  |  |  |  | 25 Nos | 110.00 |  |
| 730 | 6.2 Volt series bulb with navar plate(25 Nos.) |  |  |  |  | 25 Nos | 125.00 |  |
| 731 | 10 K. W Rotating machine |  |  |  |  | Each | 18,000.00 |  |
|  | Description |  |  |  |  | Unit |  |  |
| 732 | EICHER model 11:10 with 100 H. P direct injestion 4 stroke engine with one spare wheel 3800 mm cabin high deck with jangla (full body) (4 wheeler) for generator |  |  |  |  | Each | 8.50,000.00 |  |
| 733 | Transformer oild tested as per IS-335 - 1972 (per Ltr) |  |  |  |  | 1 Ltr | 115.00 |  |
| 734 | ABC powder (store pressure type) Fire extinguisher 5 kg confirm to IS- Standards |  |  |  |  | Each | 6,300.00 |  |
| 735 | ABC powder (store pressure type) Fire extinguisher 10 kg confirm to IS- Standards |  |  |  |  | Each | 8,750.00 |  |
| 736 | Filteration of transformer oil as per ISS specifications filters to die electric strength above 40 K. V at site. (per Ltr) |  |  |  |  | 1 Ltr | 9.50 |  |
| 737 | Filteration of transformer oil as per ISS specifications filters to die electric strength above 40 K. V at workshop (per Ltr) |  |  |  |  | 1 Ltr | 6.50 |  |
| 738 | Cover blades to PVC junction boxes |  |  |  |  | Each | 5.00 |  |
| 739 | 12 mm Screws No8 |  |  |  |  | 100 Nos. | 50.00 |  |
| 740 | 25 mm Screws No8 |  |  |  |  | 100 Nos. | 60.00 |  |
| 741 | 30 mm Screws No8 |  |  |  |  | 100 Nos. | 75.00 |  |
| 742 | 35 / 38 mm Screws No8 |  |  |  |  | 100 Nos. | 80.00 |  |
| 743 | 50 mm Screws No8 |  |  |  |  | 100 Nos. | 90.00 |  |
| 744 | 63 / 75 mm Screws No8 |  |  |  |  | 100 Nos. | 110.00 |  |
| 745 | TW Seperators / Gutties |  |  |  |  | 100 Nos. | 200.00 |  |
| 746 | Rawl Plugs |  |  |  |  | 100 Nos. | 30.00 |  |
| 747 | U Nails |  |  |  |  | 100 Nos. | 55.00 |  |
| 748 | Alluminium Link Clips |  |  |  |  | 144 Nos. | 70.00 |  |
| 749 | 50 mm G.I.Nut and Bolts. |  |  |  |  | 1 Set | 18.00 |  |
| 750 | 63 to 100 mm G.I.Nut and Bolts. |  |  |  |  | 1 Set | 50.00 |  |
| 751 | 50 mm 0 Gauge Nails |  |  |  |  | 1 Dozen | 60.00 |  |
| 752 | 63 to 100 mm 0 Gauge Nails |  |  |  |  | 1 Dozen | 144.00 |  |
| 753 | Hard Coke |  |  |  |  | 1 K. G | 12.00 |  |
| 754 | Salt |  |  |  |  | 1 K. G | 8.00 |  |
|  | Description |  |  |  |  | Unit |  |  |
| 755 | PVC Cleats 2 Way |  |  |  |  | 100 Nos. | 420.00 |  |
| 756 | PVC Cleats 4 Way |  |  |  |  | 100 Nos. | 520.00 |  |
| 757 | Binding Wire |  |  |  |  | 1 K. G | 56.00 |  |
| 758 | PVC / Cotton Insulation tape Role ( 10 Mtrs) |  |  |  |  | Each | 20.00 |  |
| 759 | C.I.Knob |  |  |  |  | Each | 15.00 |  |
| 760 | Stay tight set (Jum bracket 300 mm) |  |  |  |  | Pair | 310.00 |  |
| 761 | Stay clamp with bolts and Nuts) |  |  |  |  | 1 Set | 230.00 |  |
| 762 | L.T.Guy insulator |  |  |  |  | 1 No. | 45.00 |  |
| 763 | 12 / 15 MS plain rod fan hook |  |  |  |  | 1 No. | 50.00 |  |
| 764 | 19 / 20 mm Steel tube fan down rod |  |  |  |  | 1 Rm. | 72.00 |  |
| 765 | Consealed box PVC / MS with hook for fan |  |  |  |  | Each | 150.00 |  |
| 766 | Coil Earthing |  |  |  |  | Each | 75.00 |  |
| 767 | Cable root Indicators |  |  |  |  | Each | 90.00 |  |
| 768 | 40 mm x 40 mm x 6 mm angle iron with I bolt. |  |  |  |  | Each | 250.00 |  |
| 769 | Acrylic Cover of size 4" x 7" for night lamps. |  |  |  |  | Each | 20.00 |  |
| 770 | TW / PVC Round Blocks |  |  |  |  | Each | 7.00 |  |
| 771 | G.I.Funnel with Mesh 1/2" x 4" |  |  |  |  | Each | 100.00 |  |
| 772 | Asbestos Sheet |  |  |  |  | Sqft | 24.00 |  |

Conveyance Rates of

Materials

RATES FOR CONVEYANCE OF BRICKS, TILES & AC SHEETS INCLUDING

LOADING, UNLOADING &STACKING ON METALLED ROAD

| S.No | Description | S S RATE FOR 2008-09 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Bricks Mangalore Tiles 2500 Kgs per 1000 Nos | Terrace Bricks, Flat Tiles pan tiles 2500 Kgs per 1000 Nos | Lime coke & wood 1000 Kgs/ Cum | A.C sheets per tonne |
| 1 | 2 |  |  |  |  |
| 1 | 0 - 500 Mts | 234.00 | 65.00 | 86.00 | 86.00 |
| 2 | 1 km | 240.00 | 66.00 | 88.00 | 88.00 |
| 3 | 2 Km | 248.00 | 69.00 | 93.00 | 93.00 |
| 4 | 3 Km | 256.00 | 71.00 | 97.00 | 97.00 |
| 5 | 4 Km | 264.00 | 75.00 | 99.00 | 99.00 |
| 6 | 5 Km | 272.00 | 77.00 | 102.00 | 102.00 |
| 7 | 6 Km | 293.00 | 85.00 | 110.00 | 110.00 |
| 8 | 7 Km | 303.00 | 87.00 | 113.00 | 113.00 |
| 9 | 8 Km | 309.00 | 90.00 | 118.00 | 118.00 |
| 10 | 9 Km | 317.00 | 92.00 | 122.00 | 122.00 |
| 11 | 10 Km | 326.00 | 97.00 | 125.00 | 125.00 |
| 12 | 11 Km | 336.00 | 99.00 | 127.00 | 127.00 |
| 13 | 12 Km | 344.00 | 103.00 | 132.00 | 132.00 |
| 14 | 13 Km | 350.00 | 105.00 | 136.00 | 136.00 |
| 15 | 14 Km | 359.00 | 107.00 | 140.00 | 140.00 |
| 16 | 15 Km | 368.00 | 110.00 | 142.00 | 142.00 |
| 17 | 16 Km | 377.00 | 113.00 | 146.00 | 146.00 |
| 18 | 17 Km | 385.00 | 118.00 | 151.00 | 151.00 |
| 19 | 18 Km | 392.00 | 120.00 | 155.00 | 155.00 |
| 20 | 19 Km | 401.00 | 123.00 | 157.00 | 157.00 |
| 21 | 20 Km | 410.00 | 125.00 | 160.00 | 160.00 |
| 22 | BEYOND 20 to 50 Km (per Km) | 11.00 | 4.00 | 5.00 | 5.00 |
| 23 | Above 50 Km (per Km) | 10.00 | 3.00 | 4.00 | 4.00 |
| 24 | loading | 25.00 | 12.00 | 12.00 | 12.00 |
| 25 | Unloadng | 20.00 | 10.00 | 9.00 | 9.00 |
| 26 | Stacking | 10.00 | 7.00 | 6.00 | 6.00 |
|  | Total | 54.00 | 29.00 | 27.00 | 27.00 |

NOTE:

- The above rates are governed by the definition of carriage given under the clause 34 of PS to A.P.S.S.

- The above rates are inclusive of Loading, Unloading and Stacking Charges (single operation). As the rates for the various items under materials include stacking charges also, the same should be deducted from the conveyance rates obtained from the table where not applicable.

- Where sandy track and cart track are met with, the equivalent length of these tracks may be arrived by multiplying with factors 1.40 and 1.10 for the length of sandy or cart track respectively and lead for the equivalent length so arrived at may be worked out for the cost of conveyance.

- When the material like Surkhi, Flyash and Bitumen are conveyed on emergency basis to avoid Wharfage, Demurrage from Railway station/Factories to Departmental Godown which are available within a lead of  8 km., 40% extra rate shall be allowed over above rates (Subject to the condition that the Departmental Godown is not available within 8 Kms).

5. The conveyance rates of Surkhi and Flyash are allowed for conveyance of food grains under food for work program.

6.  Lead with fraction should not be rounded off twice.  Illustratively, if actual

lead on sandy track is 10.20 Km. say, then the lead for purpose of data

should be only 10.20 x 1.40 = 14.280 or 15 km and not 10.20 rounded to

11.00 multiplied by 1.40 and again rounded to 16 kms.

Copy of

GOVERNMENT OF ANDHRA PRADESH

# ABSTRACT

R&B – Constitution of Committee of Chief Engineers for finalization of SSR for both electrical and civil items specific to the functioning of buildings works – Accorded – Orders - Issued

TRANSPORT, ROADS & BUILDINGS (R.1) DEPARTMENT

G.O.Rt.No.471,							Dated: 20-04-2007

Read the following:

1. Govt Memo. No.48425/ B.I (1)/ 2006, dt: 06-1-2007.

- From the Chief Engineer (R&B) Buildings Letter No. TA8/ JE3/ SSR-2006-07 dt:17-11-2006 and dt. 8-1-2007.

*******

ORDER:

In the circumstances reported by the Chief Engineer (R&B) Buildings in his letters 2nd read above, Government after careful examination of the matter hereby constitute a Committee with the following Engineer-in-Chief and Chief Engineers of departments in-charge of execution of building works, for finlisation of SSR for both electrical and civil items specific to the functioning of building works: -

- Engineer-in-Chief, Public Health Engg. 			: 	Member

- Chief Engineer(R&B)Buildings,				: 	Member Convener

- Chief Engineer, Panchayat Raj 				: 	Member

- Chief Engineer,AP Housing Board			: 	Member

- Chief Engineer, Greater Hyderabad (MCH)		:	Member

- Chief Engineer, AP Housing Corporation			:	Member

- Chief Engineer, APSRTC				: 	Member

- Chief Engineer, APHMHIDC 				: 	Member

- Superintending Engineer (R&B), Electrical Circle, Hyd	: 	Member

2. The Chief Engineer (R&B) Buildings shall take necessary further action accordingly.

3. This order issues with the concurrence of Finance Department vide their U.O.No.3326/ 342/ A2/ Expr.PW/ 07. dt. :3-4-2007.

(BY ORDER AND IN THE NAME OF THE GOVERNOR OF ANDHRA PRADESH)

T.CHATTERJEE

PRINCIPAL SECRETARY TO GOVERNMENT

To,

The Chief Engineer (R&B) Buildings, Hyderabad

All the Member concerned | through the CE(R&B) Buildings |

Copy to:

The PS to Minister R&B and Ports

The I & CAD (Reforms) Department

The PS to thePrl. Secretray to Government (TC), TR&B,

SF/ SC

FORWARDED BY ORDER